3GPP TSG-RAN5 Meeting #92-e	R5-214809r1

# Electronic Meeting, 16th Aug– 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1988 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE Policy Delivery msg |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Some UE policy Delivery msg is not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Change msg of MANAGE UE POLICY COMMAND, MANAGE UE POLICY COMMAND REJECT, UE POLICY PROVISIONING REQUEST, UE POLICY PROVISIONING REJECT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X test case will have no default configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: 1, Change UPSC value of Table 4.7.6-1/3 to be 2 octets value. 2, Comment in Table 4.7.6-5 changed to "UE policies for V2X communication over PC5 requested". |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.6	Contents of UE Policy Delivery messages

–	MANAGE UE POLICY COMMAND

Table 4.7.6-1: MANAGE UE POLICY COMMAND

| Derivation Path: TS 24.501 Table D.5.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| MANAGE UE POLICY COMMAND message identity |  | ’0000 0001’B | MANAGE UE POLICY COMMAND message |  |
| UE policy section management list |  |  |  |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  | 1 entry |  |  |
| UE policy section management sublist (PLMN-1) |  |  |  |  |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management sublist contents |  |  |  |  |
| Instruction 1 |  |  |  |  |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UPSC |  | '00 01'H | 2 bytes, value set by PCF |  |
| UE policy section contents |  |  |  |  |
| UE policy part 1 |  |  |  |  |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B |  | URSP |
|  |  | ‘0010’B |  | ANDSP |
|  |  | ‘0011’B |  | V2XP |
| UE policy part contents |  | FFS |  | URSP |
|  |  | FFS |  | ANDSP |
|  |  | See Table 4.7.5.4-1 |  | V2XP |

| Condition | Explanation |
| --- | --- |
| URSP | UE Route Selection Policy (URSP) |
| ANDSP | UE Access Network Discovery and Selection Policy (ANDSP) |
| V2XP | UE policies for V2X (V2XP) |

–	MANAGE UE POLICY COMPLETE

Table 4.7.6-2: MANAGE UE POLICY COMPLETE

| Derivation Path: TS 24.501 Table D.5.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in MANAGE UE POLICY COMMAND message. |  |  |
| MANAGE UE POLICY COMPLETE message identity |  | ’0000 0010’B | MANAGE UE POLICY COMPLETE message |  |

–	MANAGE UE POLICY COMMAND REJECT

Table 4.7.6-3: MANAGE UE POLICY COMMAND REJECT

| Derivation Path: TS 24.501 Table D.5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in MANAGE UE POLICY COMMAND message. |  |  |
| MANAGE UE POLICY COMMAND REJECT message identity |  | ’0000 0011’B | MANAGE UE POLICY COMMAND REJECT message |  |
| UE policy section management result |  |  |  |  |
| Length of UE policy section management result contents |  | Set to the actual length of 'UE policy section management result contents' in bytes |  |  |
| UE policy section management list contents |  | 1 entry |  |  |
| UE policy section management subresult (PLMN 1) |  |  |  |  |
| Number of results |  | 1 |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management subresult contents |  | 1 entry |  |  |
| Result 1 |  |  |  |  |
| UPSC |  | '00 01'H |  |  |
| Failed instruction order |  | Set according to specific message content. |  |  |
| Cause |  | Set according to specific message content. |  |  |

–	UE STATE INDICATION

Table 4.7.6-4: UE STATE INDICATION

| Derivation Path: TS 24.501 Table D.5.4.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| UE STATE INDICATION message identity |  | ’0000 0100’B |  |  |
| UPSI list |  | 1 entry |  |  |
| Length of UPSI list contents |  | Set to the actual length of 'UPSI list contents' in bytes |  |  |
| UPSI sublist (PLMN 1) |  | 1 entry |  |  |
| Length of UPSI sublist |  | Set to the actual length of 'UPSI sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UPSC 1 |  | Set according to specific message content. |  |  |
| UE policy classmark |  | Not checked |  |  |
| Length of Policy information contents |  | Set to the actual length of 'Policy information contents' in bytes |  |  |
| Spare |  | ‘0000 000’B |  |  |
| SupportANDSP |  | ‘1’B |  | ANDSP |
|  |  | Not checked |  |  |
| UE OS Id |  |  |  |  |
| Length of OS Id information contents |  | Set to the actual length of 'OS Id information contents' in bytes |  |  |
| OS Id_1 |  | Set according to parameter given in test case |  |  |

| Condition | Explanation |
| --- | --- |
| ANDSP | UE Access Network Discovery and Selection Policy (ANDSP) |

–	UE POLICY PROVISIONING REQUEST

Table 4.7.6-5: UE POLICY PROVISIONING REQUEST

| Derivation Path: TS 24.587 Table 7.2.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| UE POLICY PROVISIONING REQUEST message identity |  | ’0000 0101’B |  |  |
| Requested UE policies |  |  |  |  |
| Length of Requested UE policies contents |  | 2 |  |  |
| Requested UE policies contents |  | '0000 0001 0000 0000'B | UE policies for V2X communication over PC5 requested |  |

–	UE POLICY PROVISIONING REJECT

Table 4.7.6-6: UE POLICY PROVISIONING REJECT

| Derivation Path: TS 24.587 Table 7.2.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in UE POLICY PROVISIONING REQUEST message. |  |  |
| UE POLICY PROVISIONING REJECT message identity |  | ’0000 0110’B |  |  |
| UPDS cause |  | '0001 1111'B | Request rejected, unspecified |  |

<End of modified section 1>