3GPP TSG-RAN WG5 Meeting #92-e	R5-215051r1

Online, , 16th August 2021 – 27th August 2021

Source:	Qualcomm Incorporated, Ericsson, AT&T, FirstNet, ZTE, Orange

Title:	Discussion paper on early implementation of Release 14 EIEI feature

# Document for:	Agreement

Agenda Item:	6.6.6.5

## 1.	Introduction

Conformance test aspects of “Evolution to and Interworking with eCall in IMS” is a Rel-14 workitem (“EIEI-UEConTest”) in RAN5. All the test cases defined in [1], [2] were made applicable for Release 14 and onwards UEs. This document is to discuss about early implementation of EIEI feature.

## 2.	Discussion

For this workitem, as specified in [3], when RRC receives SystemInformationBlockType1 with eCallOverIMS-Support IE set to true, it shall be forwarded to upper layers. This IE is to indicate to the UE whether the cell supports eCall over IMS services via EPC or not.

Other than the eCallOverIMS-Support IE that UE has to comprehend, support and testing of this 3GPP Rel-14 SA2, SA4, CT1 feature does not require any modification of signalling procedures on Access Stratum level which means that UEs, declaring in their UE Capability Information to be compliant with earlier releases e.g. Rel-9 and onwards on Access Stratum level (= ASN.1), and having additionally implemented the Release 14 EIEI functionality, should not be excluded from execution of EIEI-UEConTest test cases.

## 3.   Summary

Based on the discussion in section 2, this feature does not have a dependency on access stratum release of the UE and hence execution of EIEI test cases on such pre-release 14 UEs shall be allowed.

## 4.   Proposal/Way Forward

Proposal – Extend the applicability of EIEI-UEConTest test cases defined in [4], [5] to Release 9 and onwards UEs.

## 5.	References

[1]	TS 36.523-1 - 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1 specification

[2]	TS 34.229-1 - Technical Specification Group Radio Access Network; Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1

[3]	TS 36.331 – 	Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC);

[4]	TS 36.523-2 – Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification

[5]	TS 34.229-2 - Technical Specification Group Radio Access Network; Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) specification