3GPP TSG-RAN5 Meeting #92-e	R5-215174r2

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1373 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test frequencies for LTE band 24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Regulatory updates limits operations on a limited frequency range within the specified LTE band 24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to reference test frequencies and consideration of dual Tx-Rx spacings |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies will not reflect the regulatory updates for LTE band 24 and will result in test cases with frequencies that will not be deployed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 4.3.1.1.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rel-17 to trigger a spec. upgrade Band 24 is unique in that after regulatory update, only 10 MHz of DL is usable while 20 MHz (2 10 MHz non-contiguous blocks) of UL is usable. Two Tx-Rx spacing have been specified so that 10 MHz of DL can be paired with either of the 2 10 MHz UL blocks. To ensure that both the 10 MHz UL blocks are fully tested, the test frequencies have specified such that the Low and Mid test frequencies are based on Tx-Rx spacing of -101.5 MHz while High test frequency is based on Tx-Rx spacing of -120.5 MHz. This will eliminate the need to put Band 24 specific test point information in individual test cases. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Checkmark against ME on the coversheet is removed. Rev2: Merged test frequencies for different Tx-Rx spacing into Table 4.3.1.1.24-1 and Deleted Table 4.3.1.1.24-2; Also deleted the sentence in the clause 4.3.1 |  |  |  |  |  |  |  |  |

### 4.3.1	Test frequencies

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 KHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 4.3.1.1.

E-UTRA/TDD is designed to operate in unpaired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 4.3.1.2.

For non-CA Signalling testing, E-UTRA frequency to be tested is mid range and E-UTRA channel bandwidth to be tested is 5MHz for all operating bands for all test cases as the default configuration unless specific channel bandwidth is specified for the operating band below:

For Band 11, 13, 18 and 20, channel bandwidth to be tested is 10 MHz as the default configuration.

For Band 38, 39, 40, 41 and 42, channel bandwidth to be tested is 20 MHz as the default configuration.

For operating band 46, the test frequencies defined in Clause 4.3.1.2.14 and 4.3.1.2.14A are derived based on the rules defined in TS 36.104 [30], table 5.7.3-1 Note 3.

For CA Signalling testing with two component carriers, E-UTRA frequencies to be tested are according to sub clause 6.2.3.2 and the E-UTRA channel bandwidth combination is according to Table 4.3.1-1.

<<Unchanged Text Skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 4.3.1.1.24	FDD reference test frequencies for operating band 24

Table 4.3.1.1.24-1: Test frequencies for E-UTRA channel bandwidth for operating band 24

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 25735 | 1630 | 7735 | 1528.5 |
|  | 10 | 25760 | 1632.5 | 7760 | 1531 |
| Mid Range | 5 | 25785 | 1635 | 7785 | 1533.5 |
|  | 10 | 25760 | 1632.5 | 7760 | 1531 |
| High Range | 5 | 25975 | 1654 | 7785 | 1533.5 |
|  |  | 10 |  | 25950 |  | 1651.5 |  | 7760 |  | 1531 |  |
| Note: Low and Mid test frequencies are specified using Tx-Rx spacing of -101.5 MHz and High test frequency is specified using Tx-Rx spacing of -120.5 MHz |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------