3GPP TSG-RAN5 Meeting #92-e                      	R5-215432r2

# Electronic Meeting, 16th Aug – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0251 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Test Tolerance analysis for FR2 event triggered reporting in non-DRX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Existing TT contains invalid a3-offset value which has been corrected in both 38.133 and 38.533 and needs to be corrected in TT analysis as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ## A3-offset changed to correct value, offsetMO introduced. No changes to applied offsets. ## Remove the following .zip file from TR 38.903: ## “38.533 5.6.2.1+5.6.2.3+7.6.2.1+7.6.2.3 TT” ## Add the following attached .zip file to TR 38.903: ## “38.533 5.6.2.1+5.6.2.3+7.6.2.1+7.6.2.3 TT v2” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TT analysis will remain incorrect in 38.903. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Replace zip file in TR 38.903 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependant R1: Corrected cover page + some minor corrections in .doc R2: Further correction of cover page |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ///Start of change

## Remove the following .zip file from TR 38.903:

## “38.533 5.6.2.1+5.6.2.3+7.6.2.1+7.6.2.3 TT”

## Add the following attached .zip file to TR 38.903:

## “38.533 5.6.2.1+5.6.2.3+7.6.2.1+7.6.2.3 TT v2”

### ///End of changes