3GPP TSG-RAN5 Meeting #92-e	R5-215483r2

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5349 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MPR, ACLR, SEM CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test points are not aligned for MPR, SEM, and ACLR test cases for CA (intra-band contiguous DL CA and UL CA) in clauses 6.2.3A.1, 6.6.2.1A.1, and 6.6.2.3A.1. Test configuration of 15MHz/10MHz is missing in Table 6.6.2.3A.1.4.1-1. Minimum requirement of MPR CA test case 6.2.3A.1 is not aligned to TS 36.101. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Testpoints have been aligned in Tables 6.2.3A.1.4.1-1, 6.6.2.1A.1.4.1 and 6.6.2.3A.1.4.1-1. Test configuration for 15MHz/10MHz has been specified in Tables 6.2.3A.1.4.1-1, 6.6.2.1A.1.4.1 and 6.6.2.3A.1.4.1-1. For the MPR CA test case the minimum conformance requirement clause has been aligned to TS 36.101. Test requirements have been specified for the added test points in Table 6.2.3A.1.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA configurations with bandwidth sets including 15MHz/10MHz, e.g., CA_7C, cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3A.1.3, 6.2.3A.1.4.1, 6.2.3A.1.5, 6.6.2.1A.1.4.1, 6.6.2.3A.1.4.1, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Corrected specification number on cover sheet. r2: Duplicated test IDs in MPR TC have been removed from spec. Reordered and aligned all test IDs between MPR, SEM, ACLR TCs Corrected test requirement Updated Annex F.3.2 for MPR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.2.3A.1	Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA)

##### 6.2.3A.1.1	Test purpose

The number of RB identified in Table 6.2.3A.1.3-1 is based on meeting the requirements for Adjacent Channel Leakage power Ratio (ACLR) for CA and the maximum power reduction (MPR) for intra-band contiguous CA Bandwidth Class C due to Cubic Metric (CM).

##### 6.2.3A.1.2	Test applicability

The requirements of this test apply in test case 6.6.2.3A.1 Adjacent Channel Leakage power Ratio (ACLR) for CA to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

NOTE:	As a result TC 6.2.3A.1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

##### 6.2.3A.1.3	Minimum conformance requirements

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A.0-2 due to higher order modulation and contiguously aggregated transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3A.1.3-1 for UE power class 3 CA bandwidth classes B and C. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3A.1.3-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | CA bandwidth Class B and C / Smallest Component Carrier Transmission Bandwidth Configuration |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 25 RB | 50 RB | 75 RB | 100 RB |  |
| QPSK | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 25 | > 50 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 25 | > 50 | > 75 | > 100 | ≤ 3 |

For PUCCH and SRS transmission, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For UE power class 3 intra-band contiguous CA Bandwidth Class C with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A.0-2 is specified as follows:

MPR = CEIL { min(MA, MIM5), 0.5}

Where MA is defined as follows QPSK, 16 QAM and 64 QAM:

MA = 	8.2; 				0 ≤ A < 0.025

9.2 - 40A; 		0.025 ≤ A < 0.05

8 - 16A; 			0.05 ≤ A < 0.25

4.83 - 3.33A; 		0.25 ≤ A ≤ 0.4

3.83 - 0.83A; 		0.4 ≤ A ≤ 1

and MIM5 is defined as follows

MIM5 =		4.5		; IM5 < 1.5 * BWChannel_CA

6.0		; 1.5 * BWChannel_CA ≤ IM5 < BWChannel_CA/2 + FOOB

MA		; IM5 ≥ BWChannel_CA/2 + FOOB

For UE power class 3 intra-band contiguous carrier aggregation bandwidth class B with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A.0-2 is specified as follows

MPR = CEIL { MA, 0.5}

Where MA is defined as follows:

MA =	10.5 – 17.5A			; 0 ≤ A < 0.2

8.5 – 7.5A 			; 0.2 ≤ A < 0.6

5.5 – 2.5A 			; 0.6 ≤ A ≤ 1

Where

A = NRB_alloc / NRB_agg.

IM5 = max( | FC_agg – (3*Fagg_alloc_low – 2*Fagg_alloc_high) |,  | FC_agg  – (3*Fagg_alloc_high – 2*Fagg_alloc_low) | )

FC_agg = (Fedge_high + Fedge_low)/2

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB, i.e. MPR ∈[3.0, 3.5 4.0 4.5 5.0 5.5 6.0 6.5 7.0 7.5, 8.0, 8.5]

For intra-band carrier aggregation, the MPR is evaluated per slot and given by the maximum value taken over the transmission(s) on all component carriers within the slot, the maximum MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by MPR, the power limits specified in sub-clause 6.2.5A.1 apply.

The normative reference for this requirement is in TS 36.101 [2] clause 6.2.3A.

##### 6.2.3A.1.4	Test description

6.2.3A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_25@0 | S_50@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 | N/A for this test | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 |  | 75 |  | 75 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@74 |  | - |  | - |  |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 |  | 100 |  | 50 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@49 |  | - |  | - |  |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 |  | 100 |  | 100 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@99 |  | - |  | - |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS and interfering sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.3A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.3A.1.4.3.

6.2.3A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.2.3A.1.4.3.

3.	SS activates SCC by sending the MAC-CE according to TS 36.321 [13] clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133 [4] clause 8.3.3.2.

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.2.3A.1.4.1-1on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2.3A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.2.3A.1.5	Test Requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3A.1.5-1 or Table.6.2.3A.1.5-2, as applicable.

.

Table 6.2.3A.1.5-1: CA UE Power Class test requirements, E UTRA bands ≤ 3GHz

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7/-2.7 |
| 2 |  |  |  |  | 23 | +2.7/-3.7 |
| 3 |  |  |  |  | 23 | +2.7/-4.7 |
| 4 |  |  |  |  | 23 | +2.7/-3.7 |
| 5 |  |  |  |  | 23 | +2.7/-4.7 |
| 6 |  |  |  |  | 23 | +2.7/-6.2 |
| 7 |  |  |  |  | 23 | +2.7/-15.7 |
| 8 |  |  |  |  | 23 | +2.7/-2.7 |
| 9 |  |  |  |  | 23 | +2.7/-3.7 |
| 10 |  |  |  |  | 23 | +2.7/-4.7 |
| 11 |  |  |  |  | 23 | +2.7/-3.7 |
| 12 |  |  |  |  | 23 | +2.7/-4.7 |
| 13 |  |  |  |  | 23 | +2.7/-6.2 |
| 14 |  |  |  |  | 23 | +2.7/-17.2 |
| 15 |  |  |  |  | 23 | +2.7/-2.7 |
| 16 |  |  |  |  | 23 | +2.7/-3.7 |
| 17 |  |  |  |  | 23 | +2.7/-4.7 |
| 18 |  |  |  |  | 23 | +2.7/-3.7 |
| 19 |  |  |  |  | 23 | +2.7/-4.7 |
| 20 |  |  |  |  | 23 | +2.7/-6.2 |
| 21 |  |  |  |  | 23 | +2.7/-17.2 |
| 22 |  |  |  |  | 23 | +2.7/-2.7 |
| 23 |  |  |  |  | 23 | +2.7/-3.7 |
| 24 |  |  |  |  | 23 | +2.7/-4.7 |
| 25 |  |  |  |  | 23 | +2.7/-3.7 |
| 26 |  |  |  |  | 23 | +2.7/-4.7 |
| 27 |  |  |  |  | 23 | +2.7/-6.2 |
| 28 |  |  |  |  | 23 | +2.7/-17.2 |
| 29 |  |  |  |  | 23 | +2.7/-2.7 |
| 30 |  |  |  |  | 23 | +2.7/-3.7 |
| 31 |  |  |  |  | 23 | +2.7/-4.7 |
| 32 |  |  |  |  | 23 | +2.7/-3.7 |
| 33 |  |  |  |  | 23 | +2.7/-4.7 |
| 34 |  |  |  |  | 23 | +2.7/-6.2 |
| 35 |  |  |  |  | 23 | +2.7/-14.2 |
| 36 |  |  |  |  | 23 | +2.7/-2.7 |
| 37 |  |  |  |  | 23 | +2.7/-3.7 |
| 38 |  |  |  |  | 23 | +2.7/-4.7 |
| 39 |  |  |  |  | 23 | +2.7/-3.7 |
| 40 |  |  |  |  | 23 | +2.7/-4.7 |
| 41 |  |  |  |  | 23 | +2.7/-6.2 |
| 42 |  |  |  |  | 23 | +2.7/-14.2 |
| 43 |  |  |  |  | 23 | +2.7/-2.7 |
| 44 |  |  |  |  | 23 | +2.7/-3.7 |
| 45 |  |  |  |  | 23 | +2.7/-4.7 |
| 46 |  |  |  |  | 23 | +2.7/-3.7 |
| 47 |  |  |  |  | 23 | +2.7/-4.7 |
| 48 |  |  |  |  | 23 | +2.7/-6.2 |
| 49 |  |  |  |  | 23 | +2.7/-14.2 |
| 50 |  |  |  |  | 23 | +2.7/-2.7 |
| 51 |  |  |  |  | 23 | +2.7/-3.7 |
| 52 |  |  |  |  | 23 | +2.7/-4.7 |
| 53 |  |  |  |  | 23 | +2.7/-3.7 |
| 54 |  |  |  |  | 23 | +2.7/-4.7 |
| 55 |  |  |  |  | 23 | +2.7/-6.2 |
| 56 |  |  |  |  | 23 | +2.7/-14.2 |
| 57 |  |  |  |  | 23 | +2.7/-2.7 |
| 58 |  |  |  |  | 23 | +2.7/-3.7 |
| 59 |  |  |  |  | 23 | +2.7/-4.7 |
| 60 |  |  |  |  | 23 | +2.7/-3.7 |
| 61 |  |  |  |  | 23 | +2.7/-4.7 |
| 62 |  |  |  |  | 23 | +2.7/-6.2 |
| 63 |  |  |  |  | 23 | +2.7/-14.2 |
| 64 |  |  |  |  | 23 | +2.7/-2.7 |
| 65 |  |  |  |  | 23 | +2.7/-3.7 |
| 66 |  |  |  |  | 23 | +2.7/-4.7 |
| 67 |  |  |  |  | 23 | +2.7/-3.7 |
| 68 |  |  |  |  | 23 | +2.7/-4.7 |
| 69 |  |  |  |  | 23 | +2.7/-6.2 |
| 70 |  |  |  |  | 23 | +2.7/-14.2 |
| NOTE 1: Lower limit is assuming ΔTC,c and ΔTIB,c are zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

Table 6.2.3A.1.5-2: CA UE Power Class test requirements, 3GHz < E UTRA bands ≤ 4.2GHz

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +3.0/-4.0 |
| 2 |  |  |  |  | 23 | +3.0/-5.0 |
| 3 |  |  |  |  | 23 | +3.0/-6.0 |
| 4 |  |  |  |  | 23 | +3.0/-5.0 |
| 5 |  |  |  |  | 23 | +3.0/-6.0 |
| 6 |  |  |  |  | 23 | +3.0/-7.0 |
| 7 |  |  |  |  | 23 | +3.0/-16.0 |
| 8 |  |  |  |  | 23 | +3.0/-4.0 |
| 9 |  |  |  |  | 23 | +3.0/-5.0 |
| 10 |  |  |  |  | 23 | +3.0/-6.0 |
| 11 |  |  |  |  | 23 | +3.0/-5.0 |
| 12 |  |  |  |  | 23 | +3.0/-6.0 |
| 13 |  |  |  |  | 23 | +3.0/-7.0 |
| 14 |  |  |  |  | 23 | +3.0/-17.5 |
| 15 |  |  |  |  | 23 | +3.0/-4.0 |
| 16 |  |  |  |  | 23 | +3.0/-5.0 |
| 17 |  |  |  |  | 23 | +3.0/-6.0 |
| 18 |  |  |  |  | 23 | +3.0/-5.0 |
| 19 |  |  |  |  | 23 | +3.0/-6.0 |
| 20 |  |  |  |  | 23 | +3.0/-7.0 |
| 21 |  |  |  |  | 23 | +3.0/-17.5 |
| 22 |  |  |  |  | 23 | +3.0/-4.0 |
| 23 |  |  |  |  | 23 | +3.0/-5.0 |
| 24 |  |  |  |  | 23 | +3.0/-6.0 |
| 25 |  |  |  |  | 23 | +3.0/-5.0 |
| 26 |  |  |  |  | 23 | +3.0/-6.0 |
| 27 |  |  |  |  | 23 | +3.0/-7.0 |
| 28 |  |  |  |  | 23 | +3.0/-17.5 |
| 29 |  |  |  |  | 23 | +3.0/-4.0 |
| 30 |  |  |  |  | 23 | +3.0/-5.0 |
| 31 |  |  |  |  | 23 | +3.0/-6.0 |
| 32 |  |  |  |  | 23 | +3.0/-5.0 |
| 33 |  |  |  |  | 23 | +3.0/-6.0 |
| 34 |  |  |  |  | 23 | +3.0/-7.0 |
| 35 |  |  |  |  | 23 | +3.0/-14.5 |
| 36 |  |  |  |  | 23 | +3.0/-4.0 |
| 37 |  |  |  |  | 23 | +3.0/-5.0 |
| 38 |  |  |  |  | 23 | +3.0/-6.0 |
| 39 |  |  |  |  | 23 | +3.0/-5.0 |
| 40 |  |  |  |  | 23 | +3.0/-6.0 |
| 41 |  |  |  |  | 23 | +3.0/-7.0 |
| 42 |  |  |  |  | 23 | +3.0/-14.5 |
| 43 |  |  |  |  | 23 | +3.0/-4.0 |
| 44 |  |  |  |  | 23 | +3.0/-5.0 |
| 45 |  |  |  |  | 23 | +3.0/-6.0 |
| 46 |  |  |  |  | 23 | +3.0/-5.0 |
| 47 |  |  |  |  | 23 | +3.0/-6.0 |
| 48 |  |  |  |  | 23 | +3.0/-7.0 |
| 49 |  |  |  |  | 23 | +3.0/-14.5 |
| 50 |  |  |  |  | 23 | +3.0/-4.0 |
| 51 |  |  |  |  | 23 | +3.0/-5.0 |
| 52 |  |  |  |  | 23 | +3.0/-6.0 |
| 53 |  |  |  |  | 23 | +3.0/-5.0 |
| 54 |  |  |  |  | 23 | +3.0/-6.0 |
| 55 |  |  |  |  | 23 | +3.0/-7.0 |
| 56 |  |  |  |  | 23 | +3.0/-14.5 |
| 57 |  |  |  |  | 23 | +3.0/-4.0 |
| 58 |  |  |  |  | 23 | +3.0/-5.0 |
| 59 |  |  |  |  | 23 | +3.0/-6.0 |
| 60 |  |  |  |  | 23 | +3.0/-5.0 |
| 61 |  |  |  |  | 23 | +3.0/-6.0 |
| 62 |  |  |  |  | 23 | +3.0/-7.0 |
| 63 |  |  |  |  | 23 | +3.0/-14.5 |
| 64 |  |  |  |  | 23 | +3.0/-4.0 |
| 65 |  |  |  |  | 23 | +3.0/-5.0 |
| 66 |  |  |  |  | 23 | +3.0/-6.0 |
| 67 |  |  |  |  | 23 | +3.0/-5.0 |
| 68 |  |  |  |  | 23 | +3.0/-6.0 |
| 69 |  |  |  |  | 23 | +3.0/-7.0 |
| 70 |  |  |  |  | 23 | +3.0/-14.5 |
| NOTE 1: Lower limit is assuming ΔTC,c and ΔTIB,c are zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.6.2.1A.1	Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA)

6.6.2.1A.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.6.2.1A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.1.4	Test description

6.6.2.1A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_25@0 | S_50@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 | N/A for this test | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 |  | 75 |  | 75 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@74 |  | - |  | - |  |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 |  | 100 |  | 50 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@49 |  | - |  | - |  |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 |  | 100 |  | 100 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@99 |  | - |  | - |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.1.4.3.

6.6.2.1A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.1.5-1 or 6.6.2.1A.1.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.1.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.1.5-1 or Table.6.6.2.1A.1.5-2 for Bandwidth Class C and Table.6.6.2.1A.1.5-3 or Table.6.6.2.1A.1.5-4 for Bandwidth Class B, as applicable.

Table 6.6.2.1A.1.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-24.95 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 24.95-29.9 | -23.5 |  |  |  |  | 1 MHz |
| 29.9-29.95 |  | -23.5 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  | -23.5 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  | -23.5 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.2 | -20.7 | -20.7 | -21.7 | -22.2 | 30 kHz |
| 1-5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-24.95 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 24.95-29.9 | -23.2 |  |  |  |  | 1 MHz |
| 29.9-29.95 |  | -23.2 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  | -23.2 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  | -23.2 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  | -23.2 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-3: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands ≤ 3GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.90 | -25 | -13 | 1 MHz |
| 19.90-19.95 | -25 | -25 | 1 MHz |
| 19.95-24.90 |  | -25 | 1 MHz |

Table 6.6.2.1A.1.5-4: General E-UTRA spectrum emission mask for CA Bandwidth Class B, 3GHz < E-UTRA bands ≤ 4.2GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.90 | -25 | -13 | 1 MHz |
| 19.90-19.95 | -25 | -25 | 1 MHz |
| 19.95-24.90 |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

< Unchanged sections omitted >

##### 6.6.2.3A.1	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA)

6.6.2.3A.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra band contiguous DL CA and UL CA.

6.6.2.3A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1.4	Test description

6.6.2.3A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_25@0 | S_50@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 |  | 50 |  | 50 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@49 |  | - |  | - |  |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 |  | 75 |  | 25 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@24 |  | - |  | - |  |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 |  | 75 |  | 75 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@74 |  | - |  | - |  |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 |  | 100 |  | 50 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@49 |  | - |  | - |  |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 |  | 100 |  | 100 |  |  |  | QPSK |  | 2 |  | P_1@0 |  | S_1@99 |  | - |  | - |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

6.6.2.3A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.1.5	Test Requirements

6.6.2.3A.1.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.1.5.1-1.

Table 6.6.2.3A.1.5.1-1: UTRA UE ACLR for CA

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| UTRAACLR1 | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.6.2.3A.1.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10, shall be higher than the limits in table 6.6.2.3A.1.5.2-1.

Table 6.6.2.3A.1.5.2-1: CA E-UTRA ACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

< Unchanged sections omitted >

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- |
| 6.2.2 UE Maximum Output Power | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-3 dB 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-4.0 dB Power class 3: 23dBm ±3.3 dB |
| < Unchanged rows omitted > |  |  |  |
| 6.2.3 Maximum Power Reduction | Power class 3 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 4.7dB QPSK: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 6.0dB |
| < Unchanged rows omitted > |  |  |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB MPR = 10dB MPR = 10.5dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB MPR = 10.0dB MPR = 10.0dB | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -max(TL,c, T(PCMAX_L )) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: TL,c=2dB 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +2.7 / - 14.2dB 23dBm +2.7 / - 15.7dB 23dBm +2.7 / - 17.2dB TL,c=3dB 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.0dB 23dBm +3.0 / - 7.0dB 23dBm +3.0 / - 14.5dB 23dBm +3.0 / - 16.0dB 23dBm +3.0 / - 17.5dB |
| < Unchanged rows omitted > |  |  |  |

< End of changes >