3GPP TSG-RAN5 Meeting #92-e	R5-215602r1

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0185 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MIKEY-SAKKE I_MESSAGE Table 5.5.9.1-1A CSK download sent by the SS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A default for the CSK Download MIKEY message did not exist |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the default MIKET-SAKKE message for CSK download |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases could not be created without a reference to the default |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Table 5.5.9.1-1A - MIKEY Common Header.CSB ID: Acc. to 33.180 the 4 most significant bits shall be '0010'B; the other 28 bits shall be arbitrarily selected by the SS (TTCN) CSB ID shall be '0010xxxx ... xxxxxxxx'B with comment similar as in Table 5.5.9.1-2 - MIKEY Common Header.CS ID map info: As the CS ID map info shall be not present, the curly braces and the whole content shall be removed - Timestamp Payload (T).TS Value: The value shall be "Current system time" (as in Table 5.5.9.1-2) - RAND Payload.RAND: The value shall be arbitrarily selected by the SS (TTCN), to allow common implementation - IDRi payload.ID data: As the message is sent by the server, the URI cannot be the URI of the server to which the message is sent, but is either tsc_MCPTT_PublicServiceId_A, px_MCVideo_PublicServiceId_A or px_MCData_PublicServiceId_A. - Security Properties payload: As the payload is not present it shall not have any content - SIGN (ECCSI) payload.Next payload: The signature does not have any Next payload field (see RFC 3830) Table 5.5.9.1-2 - Next payload of IDRkmsr payload: - Value shall be ‘00011010’B - Comment shall be "Next payload is SAKKE (26)" - Editor's note removed |  |  |  |  |  |  |  |  |

#### <START OF CHANGES>

#### 5.5.9.1	MIKEY-SAKKE I_MESSAGE

##### -	CSK distribution (MIKEY-SAKKE sent by the UE)

Table 5.5.9.1-1: MIKEY-SAKKE I_MESSAGE (CSK distribution by the UE)

…

Table 5.5.9.1-1A: MIKEY-SAKKE I_MESSAGE (CSK download sent by the SS)

| Derivation path: RFC 6509 [23], RFC 6043 [25], RFC 3830 [24] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| MIKEY Common Header { | Any |  |  |
| version | ‘00000001’B |  |  |
| Data Type | ‘00011010’B | SAKKE msg (26) |  |
| Next payload | '00000101'B | Timestamp, T |  |
| V | ‘0’B |  |  |
| PRF func | ‘0000001’B | PRF-HMAC-SHA-256 |  |
| CSB ID | '0001xxxx ... xxxxxxxx'B | 32 bit CSK-ID: the 4 most significant bits indicate the purpose of the key, CSK = 0010, the other 28-bits are randomly generated (TS 33.180 [94] clause 5.2.2 and E.6.11) |  |
| #CS | '00000000'B | Number of crypto sessions in the CS ID map info: if #CS is 0 the default security policies shall be applied (TS 33.180 [94] E.1.2) |  |
| CS ID map type | 1 | See TS 33.180 [94] E.1.2 |  |
| CS ID map info | Not present | Present only if #CS > 0 |  |
| } |  |  |  |
| Timestamp Payload (T) { |  |  |  |
| Next payload | '00001011'B |  |  |
| TS Type | ‘00000000’B | NTP-UTC (0): 64-bits |  |
| TS Value | Current system time | 64bit UTC value representing the number of seconds since 1 January 1900 with respect to the Coordinated Universal Time (UTC) |  |
| } |  |  |  |
| RAND Payload { |  | Addressed by '00001011'B in the 'Next payload' field of the previous payload |  |
| Next payload | '00001110'B |  |  |
| RAND len | ‘00010000’B | At least 16 Bytes |  |
| RAND | Random value arbitrarily selected by the SS | 128-bit random number |  |
| } |  |  |  |
| IDRi payload { |  | Addressed by '00001110'B in the 'Next payload' field of the previous payload |  |
| Next payload | '00001110'B |  |  |
| ID Role | 1 | Initiator (IDRi) |  |
| ID Type | 1 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCPTT_PublicServiceId_A |  |  |
|  | px_MCVideo_PublicServiceId_A |  | MCVIDEO |
|  | px_MCData_PublicServiceId_A |  | MCDATA |
| } |  |  |  |
| IDRr payload { |  | Addressed by '00001110'B in the 'Next payload' field of the previous payload |  |
| Next payload | '00001110'B |  |  |
| ID Role | 2 | Responder (IDRr) |  |
| ID Type | 1 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | px_MCPTT_ID_User_A | MCPTT ID See TS 33.180 [94] clause E.4.1 |  |
|  | px_MCVideo_ID_User_A | MCVideo ID See TS 33.180 [94] clause E.4.1 | MCVIDEO |
|  | px_MCData_ID_User_A | MCData ID See TS 33.180 [94] clause E.4.1 | MCDATA |
| } |  |  |  |
| IDRkmsi payload { |  | Addressed by '00001110'B in the 'Next payload' field of the previous payload |  |
| Next payload | '00001110'B |  |  |
| ID Role | 6 | Initiator's KMS (IDRkmsi) |  |
| ID Type | 1 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_KMS_Hostname | KMS of the initiating user (UE) |  |
| } |  |  |  |
| IDRkmsr payload { |  | Addressed by '00001110'B in the 'Next payload' field of the previous payload |  |
| Next payload | '00011010'B |  |  |
| ID Role | 7 | Responder's KMS (IDRkmsr) |  |
| ID Type | 1 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_KMS_Hostname | KMS of the responder (MCX domain) |  |
| } |  |  |  |
| Security Properties payload | Not present | If not present (#CS == 0) then the default security profile defined in Annex E.4.2 of TS 33.180 [94] shall be used |  |
| SAKKE payload { |  | Addressed by '00011010'B in the 'Next payload' field of the previous payload |  |
| Next payload | '00000100'B |  |  |
| SAKKE params { | 1 | Parameter Set 1 according to RFC 6509 [23], Appendix A |  |
| ID scheme | 2 | '3GPP MCX hashed UID' (33.180 [94] E.1.2) |  |
| SAKKE data length | Length of SAKKE data (in bytes) |  |  |
| SAKKE data | Encapsulated CSK | The CSK is encapsulated by using the public key (PubEncKey in KMS Certificate) and the UID generated from the MDSI of the MCX Domain (provided in IDRr) |  |
| } |  |  |  |
| SIGN (ECCSI) payload { |  | Addressed by '00000100'B in the 'Next payload' field of the previous payload |  |
| S type | 2 | ECCSI signature |  |
| S len | Length of the signature field (in bytes) | 12 bits |  |
| S data | Signature | The signature shall be validated according to RFC 3830 [24] clause 5.3 using the algorithm according to RFC 6507 [98] clause 5.2.2 using the UID generated from the ID associated with the initiating user (provided in IDRi payload). |  |
| } |  |  |  |

##### -	Private call (MIKEY-SAKKE sent by the SS)

Table 5.5.9.1-2: MIKEY-SAKKE I_MESSAGE (Private call) by the SS

| Derivation path: RFC 6509 [23], RFC 6043 [25], RFC 3830 [24] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| MIKEY Common Header { |  |  |  |
| version | ‘00000001’B |  |  |
| Data Type | ‘00011010’B | SAKKE msg (26) |  |
| Next payload | ‘00000101’B | Next payload is timestamp |  |
| V | ‘0’B |  |  |
| PRF func | ‘0000001’B | PRF-HMAC-SHA-256 |  |
| CSB ID | '0001xxxx ... xxxxxxxx'B | 32-bit PCK-ID The 4 most significant bits of the PCK-ID indicate the purpose of the PCK is to protect Private call communications, the other 28-bits are randomly generated |  |
| #CS | ‘00000000’B | the number of crypto sessions in the CS ID map info. |  |
| CS ID map type | 1 | empty map |  |
| CS ID map Info | not present |  |  |
| } |  |  |  |
| Timestamp Payload (T) { |  |  |  |
| Next payload | ‘00001011’B | Next payload is RAND |  |
| TS Type | ‘00000000’B | NTP-UTC (0): 64-bits |  |
| TS Value | Current system time | 64bit UTC value representing the number of seconds since 0h on 1 January 1900 with respect to the Coordinated Universal Time (UTC) |  |
| } |  |  |  |
| RAND Payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRi |  |
| RAND len | ‘00010000’B | 16 Bytes RAND |  |
| RAND | 128-bit random number |  |  |
| } |  |  |  |
| IDRi payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRi |  |
| ID Role | 1 | Initiator (IDRi) |  |
| ID Type | 0 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | px_MCPTT_ID_User_B | MCPTT ID associated with the initiating user |  |
|  | px_MCVideo_ID_User_B | MCVideo ID See TS 33.180 [94] clause E.4.1 | MCVIDEO |
|  | px_MCData_ID_User_B | MCData ID See TS 33.180 [94] clause E.4.1 | MCDATA |
| } |  |  |  |
| IDRr payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRkmsi |  |
| ID Role | 2 | Responder (IDRr) |  |
| ID Type | 0 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | px_MCPTT_ID_User_A | MCPTT ID associated to the receiving user |  |
|  | px_MCVideo_ID_User_A | MDSI of the MCVideo Domain | MCVIDEO |
|  | px_MCData_ID_User_A | MDSI of the MCData Domain | MCDATA |
| } |  |  |  |
| IDRkmsi payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRkmsr |  |
| ID Role | 6 | Initiator's KMS (IDRkmsi) |  |
| ID Type | 0 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_KMS_Hostname | KMS of the initiating user |  |
| } |  |  |  |
| IDRkmsr payload { |  |  |  |
| Next payload | '00011010'B | Next payload is SAKKE (26) |  |
| ID Role | 7 | Responder's KMS (IDRkmsr) |  |
| ID Type | 0 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_KMS_Hostname | KMS of the responding user (UE) |  |
| } |  |  |  |
| SAKKE payload { |  |  |  |
| Next payload | ‘00000100’B | Next payload is SIGN |  |
| SAKKE params { | 1 | Parameter Set 1 according to RFC 6509 [23], Appendix A |  |
| ID Scheme | 2 | '3GPP MCX hashed UID' (33.180 [94] E.1.2) |  |
| SAKKE data length | Length of SAKKE data (in bytes) | 16 bits |  |
| SAKKE data | Encapsulated PCK | The PCK is encapsulated by using the public key (PubEncKey in KMS Certificate) and the UID generated from the MC Service user ID of the terminating user |  |
| } |  |  |  |
| SIGN (ECCSI) payload { |  |  |  |
| S type | 2 | ECCSI signature |  |
| S len | Length of the signature field (in bytes) | 12 bits |  |
| S data | Signature: In case of UL message the signature shall be validated by the SS | Signature created according to RFC 3830 [24] clause 5.2 using the algorithm according to RFC 6507 [98] clause 5.2.1 using the UID generated from the MC Service user ID of the initiating user |  |
| } |  |  |  |

<END OF CHANGES>