3GPP TSG-RAN5 Meeting #92e	R5-214429

# Electronic meeting, 16-26 August 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2287 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC reconfiguration Test Case 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 38.331 section 5.3.1.1, when all DRBs are released and SRB2 is still active, the RRC connection must be released. At step 13 of this test, all DRBs are released but the RRC connection isn’t expected to be brought down |  |  |  |  |  |  |  |  |
| Summary of change: |  | Another DRB is brought up before step 13, so atleast one DRB is still active when DRB1 is brought down |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compliant UE will release RRC connection after step 13 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of changes]

8.1.2.1.1.3.2	Test procedure sequence

Table 8.1.2.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | NR RRC: Paging | - | - |
| 2 | UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmit an RRCSetup message | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete | - | - |
| 5 | SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | Check: Does the UE transmit a SecurityModeComplete message? | --> | NR RRC: SecurityModeComplete | 1 | P |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 9 | SS transmits an RRCReconfiguration message to establish SRB2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of signalling radio bearer, SRB2? | --> | NR RRC: RRCReconfigurationComplete | 2 | P |
| 11 | The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | 3 | P |
| 12A | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? | - | - | 3 | - |
| 12B | Cause the UE to request establishment of PDU session.(Note 1) | - | - | - | - |
| 12C | The PDU session establishment procedure as specified in TS 38.508-1 [4] subclause 4.5A.2 takes place | - | - | - | - |
| 13 | The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMMAND | - | - |
| 14 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | 4 | P |
| 16 | The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE | - | - |
| Note 1: The request to establish a PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

### [End of changes]