3GPP TSG-RAN5 Meeting #92-e	R5-214438

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5027 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.2.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | T1 should be calculated when the UE detects that RLC PDU is missing. In Step3 the SS transmits RLC PDU#3 containing RLC SDU#2. UE starts t-Reordering at this time. Note T1 should start to calculate in Step3 instead of Step1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the Note T1 from Step 1 to Step 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current Prose may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact (Covered by R5s210683) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.2.2.8	UM RLC / In sequence delivery of upper layer PDUs without residual loss of RLC PDUs / Maximum re-ordering delay exceeds t-Reordering

7.2.2.8.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {
  when { a PDU is received out of order after t-Reordering has expired }

then { UE discards the corresponding PDU and delivers all correctly received RLC SDUs to upper layer in the correct order }

}

7.2.2.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.322, clause 5.1.2.2.2, 5.1.2.2.3 and 5.1.2.2.4.

[TS 36.322, clause 5.1.2.2.2]

When an UMD PDU with SN = x is received from lower layer, the receiving UM RLC entity shall:

-	if VR(UR) < x < VR(UH) and the UMD PDU with SN = x has been received before; or

-	if (VR(UH) – UM_Window_Size) <= x < VR(UR):

-	discard the received UMD PDU;

-	else:

-	place the received UMD PDU in the reception buffer.

[TS 36.322, clause 5.1.2.2.3]

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if x falls outside of the reordering window:

-	update VR(UH) to x + 1;

-	reassemble RLC SDUs from any UMD PDUs with SN that falls outside of the reordering window, remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer in ascending order of the RLC SN if not delivered before;

-	if VR(UR) falls outside of the reordering window:

-	set VR(UR) to (VR(UH) – UM_Window_Size);

-	if the reception buffer contains an UMD PDU with SN = VR(UR):

-	update VR(UR) to the SN of the first UMD PDU with SN > current VR(UR) that has not been received;

-	reassemble RLC SDUs from any UMD PDUs with SN < updated VR(UR), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer in ascending order of the RLC SN if not delivered before;

-	if t-Reordering is running:

-	if VR(UX) <= VR(UR); or

-	if VR(UX) falls outside of the reordering window and VR(UX) is not equal to VR(UH):

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above):

-	if VR(UH) > VR(UR):

-	start t-Reordering;

-	set VR(UX) to VR(UH).

[TS 36.322, clause 5.1.2.2.4]

When t-Reordering expires, the receiving UM RLC entity shall:

-	update VR(UR) to the SN of the first UMD PDU with SN >= VR(UX) that has not been received;

-	reassemble RLC SDUs from any UMD PDUs with SN < updated VR(UR), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer in ascending order of the RLC SN if not delivered before;

-	if VR(UH) > VR(UR):

-	start t-Reordering;

-	set VR(UX) to VR(UH).

7.2.2.8.3	Test description

7.2.2.8.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18] with the exceptions listed in table 7.2.2.8.3.1-1 applicable for the configured UM DRB.

-	The condition SRB2-DRB(1,1) is used for step 8 in 4.5.3A.3 according to [18].

Table 7.2.2.8.3.1-1: RLC parameters

| Downlink RLC |  |
| --- | --- |
| t-Reordering | ms200 |

7.2.2.8.3.2	Test procedure sequence

Table 7.2.2.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RLC PDU#1 containing first segment of RLC SDU#1. | <-- | UMD PDU#1 | - | - |
| 2 | The SS does not transmit RLC PDU#2 containing last segment of RLC SDU#1. | - | - | - | - |
| 3 | The SS transmits RLC PDU#3 containing RLC SDU#2. Note T1 | <-- | UMD PDU#3 | - | - |
| 3A | Check 1: Does the UE transmit RLC SDU#2 after t-Reordering (200 ms) expiry? Note T2 Check 2: Is (T2 – T1) > t-reordering? (Note 1) | --> | (RLC SDU#2) | 1 | P |
| 3B | The SS transmits RLC PDU#2 containing last segment of RLC SDU#1. | <-- | UMD PDU#2 | - | - |
| 4 | Check: Does the UE transmit RLC SDU#1 within 100 ms? | --> | (RLC SDU#1) | 1 | F |
| Note 1: It is assumed that, at expiry of t-reordering, reassembling RLC SDUs from any UMD PDUs with SN < updated VR(UR), removal of RLC headers when doing so and delivering the reassembled RLC SDUs to upper layer in ascending order of the RLC SN if not delivered before and sending the looped back RLC SDU will not take more than 60ms. |  |  |  |  |  |

7.2.2.8.3.3	Specific message contents

None.