Title: 	Discussion on test point selection for NR Occupied Bandwidth in FR1

Source: 	Keysight Technologies,Huawei, Ericsson

# Introduction

A default baseline for test point generation for TRx test cases was presented in [1] and [2] at RAN5#1-NR adhoc meeting, showing some proposals to environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR1, based on LTE. However, test points definition for frequency, channel bandwidth and SCS were FFS. Specific analysis is required per test case.

The purpose of this contribution is to provide test point selection proposal for NR Occupied Bandwidth measurement in FR1. Firstly single transmission case is done, then UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

# Discussion

The definition of Occupied Bandwidth for FR1 can be found in [3]:

6.5.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 6.5.1-1

Table 6.5.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / NR Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| Channel bandwidth [MHz] | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |

The test case stablishes that the maximum transmission bandwidth configuration transmitted by the UE shall not exceed the channel bandwidth for this transmission bandwidth configuration.

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## Test Environment

Based on the baseline agreed in [1], test environment for Occupied Bandwidth measurement for FR1 can be defined as in LTE.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Occupied Bandwidth measurement in FR1.

## Test Frequencies

In LTE, Occupied Bandwidth measurement is considered an agnostic-frequency measurement, meaning that the maximum channel bandwidth transmitted by the UE remains the same with the independence of the EARFCN being tested.

In NR FR1, there are several bands with a high bandpass, spanning in some case up to 900 MHz at high frequencies. In Table 5.2-1 in [3], the operating band definition for each band is presented. In the following table, band bandwidth per NR operating band is calculated and classified in two different types as follows:

Table 2.2-1 Bandwidth per operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode | Band Bandwidth (MHz) | Type |
| --- | --- | --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD | 60 | 1 |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD | 60 | 1 |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD | 75 | 1 |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD | 25 | 1 |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD | 70 | 1 |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD | 35 | 1 |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD | 30 | 1 |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD | 45 | 1 |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD | 50 | 1 |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD | 194 | 1 |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD | 85 | 1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD | 5 | 1 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD | 90 | 1 |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD | 25 | 1 |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD | 35 | 1 |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD | 43 | 1 |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL | 85 | 1 |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL | 5 | 1 |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD | 900 | 2 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD | 500 | 2 |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD | 600 | 2 |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL | 75 | 1 |
| n81 | 880 MHz – 915 MHz | N/A | SUL | 35 | 1 |
| n82 | 832 MHz – 862 MHz | N/A | SUL | 30 | 1 |
| n83 | 703 MHz – 748 MHz | N/A | SUL | 45 | 1 |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL | 60 | 1 |

Table 2.2-1 shows the band bandwidth per operating band. NR operating bands have been classified, based on their band bandwidth:

- Type 1: Similar bands to the ones defined for LTE, with a band bandwidth lower than 200 MHz.

- Type 2: Bands with wider band bandwidth, higher than 500 MHz.

For type-2 bands, it is needed to consider the frequency response of filters, amplifiers and other active elements in the UE, which can modify the amplitude and the phase of the signal being transmitted. This will derive in a different spectrum transmission depending on the frequency.

Observation 1: LTE test frequencies criteria, MidRange, can be reused for bands with similar bandwidth to LTE band bandwidth (≤ 200 MHz for UE Tx test cases). However, for type-2 bands with wider bandwidth, there is a need to ensure UEs properly work in the whole band bandwidth.

Proposal 2: Select Mid range as test frequency for Type-1 bands. Select Low range, Mid range and High range as test frequencies for Type-2 bands.

## Test Channel Bandwidth

Occupied Bandwidth minimum conformance requirement defined in [3] is specific for each channel bandwidth.

Furthermore, in LTE all channel bandwidths supported in a band are tested.

Observation 2: Based on Occupied Bandwidth minimum conformance requirement definition and LTE test channel bandwidth selection for Occupied Bandwidth, all available channel bandwidths in a band must be tested.

Proposal 3: Select all channel bandwidths defined in a band to measure Occupied Bandwidth.

## Test Subcarrier Spacing

According to Table 5.3.2-1 in [3], there is a relation between channel bandwidth, subcarrier spacing and the maximum number of resource blocks that can be allocated in each channel bandwidth and SCS combination.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | [78] | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | [38] | 51 | 65 | 79 | 107 | 135 |

Considering the maximum number of Resource blocks that can be allocated in each channel bandwidth and subcarrier spacing as shown in Table 5.3.2-1 in [3], the maximum transmission bandwidth can be calculated. Table 2.4-1 summarizes this calculation.

Table 2.4-1 Maximum transmission bandwidth for each UE channel bandwidth and SCS

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 28.8 | 38.88 | 48.6 | N/A | N/A | N/A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 28.08 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N/A | 7.92 | 12.96 | 17.28 | 22.32 | 27.36 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |

Observation 2: Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values.

Table 5.3.3-1 in [3], show the minimum guardband for a UE channel bandwidth and SCS.

Table 5.3.3-1: Minimum Guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | [592.5] | 552.5 | 692.5 | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | [945] | 905 | 1045 | 825 | 925 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | [1290] | 1610 | 1570 | 1530 | 1450 | 1370 |

The guardband is the distance between the channel edge and the transmission bandwidth edge. Narrower guardband implies higher attenuation slope in the UE transmission filter, to avoid subcarriers transmitted out of the channel bandwidth.

Observation 3: Lower guardbands are achieved with lower SCS for the same channel bandwidth.

Proposal 4: Select the lowest SCS supported for the current channel bandwidth to test.

## Uplink configuration

Maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS according to Table 5.3.2-1 in [3] allow the maximum spectrum utilization to measure the occupied bandwidth.

On the other hand, maximum RB allocation depends on OFDM waveform as it is stated in Table 4.5.1.1-1 and Table 4.5.1.2-1 in [4].

Table 4.5.1.1-1: Range 1 NR UE and BS maximum RB allocation for CP-OFDM

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N.A | N.A | N.A |
| 30 | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
| 60 | N.A | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |

Table 4.5.1.2-1: Range 1 NR UE maximum RB allocation for DFT-s-OFDM

| SCS [kHz] | Channel bandwidths [MHz] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 25 | 50 | 75 | 100 | 128 | 216 | 270 | N.A | N.A | N.A |
| 30 | 10 | 24 | 36 | 50 | 64 | 100 | 128 | 162 | 216 | 270 |
| 60 | N.A | 10 | 18 | 24 | 30 | 50 | 64 | 75 | 100 | 135 |

Analyzing above tables, maximum spectrum utilization per channel bandwidth and SCS is obtained for CP-OFDM waveform. RB allocation defined for DFT-s-OFDM is the closest number lower of equal to CP-OFDM satisfying the RB allocation is multiple of 2, 3 or 5.

Observation 4: CP-OFDM waveform allows to maximize the spectrum utilization for the same channel bandwidth and SCS compared to DFT-s-OFDM waveform.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select CP-OFDM waveform to get the maximum spectrum utilization.

In LTE, the lowest modulation scheme is used to measure the occupied bandwidth, as it is the most robust modulation. Similarly to LTE, for 5G NR, the lowest modulation scheme supported for the CP-OFDM is selected.

Proposal 7: Select QPSK modulation to test Occupied Bandwidth.

## Exceptions for OBW UL-MIMO test

Extracted from Clause 6.5D.1 [6]

6.5D.1	Occupied bandwidth for UL-MIMO

For UE supporting UL MIMO, the requirements for occupied bandwidth apply to  the sum of the powers from both UE transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth shall be less than the channel bandwidth specified in table 6.5.1-1. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.1 apply.

Observation 5: Same occupied bandwidth requirements as specified for single transmission apply to each antenna connector for UE supporting UL-MIMO.

The test waveform in Proposal 6 is the CP-OFDM which is mandatory to UL-MIMO transmission according to [5]

Proposal 8: Select same test points as singular transmission case for UL-MIMO OBW case.

The singular transmission test case for OBW (6.5.1) is tested with only CP-OFDM. The same approach can be used for ULFPTx UE, testing OBW for UL-MIMO release 16.

Proposal 9: For ULFPTx UE, select same select same test points as singular transmission.

# Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Occupied Bandwidth test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for Occupied Bandwidth measurement in FR1.

Proposal 2: Select Mid range as test frequency for Type-1 bands. Select Low range, Mid range and High range as test frequencies for Type-2 bands.

Proposal 3: Select all channel bandwidths defined in a band to measure Occupied Bandwidth.

Proposal 4: Select the lowest SCS supported for the current channel bandwidth to test.

Proposal 5: Select maximum transmission bandwidth configuration NRB for the selected channel bandwidth and SCS.

Proposal 6: Select CP-OFDM waveform to get the maximum spectrum utilization.

Proposal 7: Select QPSK modulation to test Occupied Bandwidth.

Proposal 8: Select same test points as singular transmission case for UL-MIMO OBW case.

Proposal 9: For ULFPTx UE, select same select same test points as singular transmission.

# References

[1] R5-180065 , “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, “Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases”, Ericsson, RAN 5 Ad hoc, Munich, Jan. 2018.

[3] R4-1800402, “38.101-1 Implementation of agreed CRs after NR-AH-1801”, Qualcomm Incorporated, RAN4 NR-AH#1801, San Diego, CA, USA, Jan 2018.

[4] TR 38.817-01 v0.3.0, “General aspects for UE RF for NR”.

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)