3GPP TSG-RAN5 Meeting #92-e	R5-214898

# Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2406 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5G NAS TC 9.1.5.1.3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies, Keysight Technologies, Anritsu Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Registration Accept message at steps 34, the IE’s SST and Mapped configured SST under Allowed NSSAI is set. But in subsequent step when SS sends PDU Session Establishment Accept message the IE Mapped configured SST is missing. TS 23.502 4.3.2.2.1 Non-roaming and Roaming with Local Breakout If the NAS message does not contain an S-NSSAI, the AMF determines an S-NSSAI of the Serving PLMN for the requested PDU Session from the current Allowed NSSAI for the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 9.1.5.1.3a.3.3-6A to add Mapped configured SST value of 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the testcase when PDU session establishment accept message is received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 9.1.5.1.3a	Initial registration / 5GS services / NSSAI handling / NSSAI storage

9.1.5.1.3a.1	Test Purpose (TP)

(1)

with { UE is switched off with a valid USIM inserted }

ensure that {

when { UE has configured NSSAI associated with HPLMN, configured NSSAI associated with a current PLMN with mapping to the Configured NSSAI for the HPLMN and UE is powered up or switched on }

then { UE transmits REGISTRATION REQUEST message with Requested NSSAI using the configured NSSAI associated with current PLMN and mapping of each S-NSSAI of the requested NSSAI to the S-NSSAIs of the configured NSSAI for the HPLMN }

}

(2)

with { UE is switched off with a valid USIM inserted }

ensure that {

when { UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a default configured NSSAI and UE is powered up or switched on }

then { UE transmits REGISTRATION REQUEST message with Requested NSSAI using the default configured NSSAI and also includes Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" }

}

(3)

with { UE is switched off with a valid USIM inserted }

ensure that {

when { UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no configured NSSAI not associated with a PLMN and UE is powered up or switched on }

then { UE transmits REGISTRATION REQUEST message and does not include a Requested NSSAI }

}

9.1.5.1.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.5.1.2.2 and 5.5.1.2.4.

[TS 24.501 clause 5.5.1.2.2]

5.5.1.2.1	General

This procedure can be used by a UE for initial registration for 5GS services.

…

The UE shall include the requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wants to register and shall include the mapping of the requested NSSAI which is the mapping of each S-NSSAI of the requested NSSAI to the S-NSSAI(s) of the HPLMN, if available, in the REGISTRATION REQUEST message. If the UE has allowed NSSAI or configured NSSAI for the current PLMN, the requested NSSAI shall be either:

a)	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN;

b)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

c)	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and those are neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

If the UE has neither allowed NSSAI for the current PLMN nor configured NSSAI for the current PLMN and has a default configured NSSAI, the UE shall:

a)	include the S-NSSAI(s) in the Requested NSSAI IE of the REGISTRATION REQUEST message using the default configured NSSAI; and

b)	include the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI" in the REGISTRATION REQUEST message.

If the UE has no allowed NSSAI for the current PLMN, no configured NSSAI for the current PLMN, and no default configured NSSAI, the UE shall not include a requested NSSAI in the REGISTRATION message.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN, if the S-NSSAI is neither in the rejected NSSAI for the current PLMN nor in the rejected NSSAI for the current PLMN and registration area combination.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for the current PLMN.

NOTE 3:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 4:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

[TS 24.501 clause 5.5.1.2.4]

The AMF shall include the allowed NSSAI for the current PLMN and shall include the mapping of each S-NSSAI of the allowed NSSAI to the S-NSSAI(s) of the HPLMN contained in the requested NSSAI from the UE if available, in the REGISTRATION ACCEPT message if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the requested NSSAI but rejected by the network associated with rejection cause(s).

The AMF may include a new configured NSSAI for the current PLMN in the REGISTRATION ACCEPT message if:

a)	the REGISTRATION REQUEST message did not include the requested NSSAI;

b)	the REGISTRATION REQUEST message included the requested NSSAI containing an S-NSSAI that is not valid in the serving PLMN; or

c)	the REGISTRATION REQUEST message included the Network slicing indication IE with the Default configured NSSAI indication bit set to "Requested NSSAI created from default configured NSSAI".

If a new configured NSSAI for the current PLMN is included in the REGISTRATION ACCEPT message, the AMF shall also include the mapping of the configured NSSAI for the current PLMN the S-NSSAI(s) of the to HPLMN if available in the REGISTRATION ACCEPT message. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The AMF shall include the Network slicing indication IE with the Network slicing subscription change indication set to "Network slicing subscription changed" in the REGISTRATION ACCEPT message if the UDM has indicated that the subscription data for network slicing has changed. In this case the AMF shall start timer T3550 and enter state 5GMM-COMMON-PROCEDURE-INITIATED as described in subclause 5.1.3.2.3.3.

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in the current PLMN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message or none of the requested NSSAI are present in the subscribed S-NSSAIs, and one or more subscribed S-NSSAIs (containing one or more S-NSSAIs each of which may be associated with a new S-NSSAI) marked as default are available, the AMF shall put the subscribed S-NSSAIs marked as default in the allowed NSSAI of the REGISTRATION ACCEPT message. The AMF shall determine a registration area such that all S-NSSAIs of the allowed NSSAI are available in the registration area.

9.1.5.1.3a.3	Test description

9.1.5.1.3a.3.1	Pre-test conditions

System Simulator:

-	NGC Cell G is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [4] except that frequency NRf1 replaces NRf2.

-	NGC Cell G belongs to VPLMN, TAI-7 and set as serving cell.

UE:

-	None

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4] and camped on NGC Cell A HPLMN.

9.1.5.1.3a.3.2	Test procedure sequence

Table 9.1.5.1.3a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. |  |  | - | - |
| 2 | The UE transmits REGISTRATION REQUEST message on NGC Cell G. | --> | REGISTRATION REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 12 | The SS transmits REGISTRATION ACCEPT message including Configured and Allowed NSSAI lists. | <-- | REGISTRATION ACCEPT | - | - |
| 13 -18 | Steps 15 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed with ‘connected without release’. | - | - | - | - |
| 19 | The SS transmits NSSAI DELETE REQUEST message to delete the Allowed NSSAI list for all PLMNs (MCC-MNC =000-000) and for 3GPP access so UE has only configured NSSAI. | <-- | NSSAI DELETE REQUEST | - | - |
| 20 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 21 | The SS transmits an RRCRelease message. | - | - | - | - |
| 22 | Switch off procedure in RRC_Idle specified in TS 38.508-1 subclause 4.9.6.1 is performed. | - | - | - | - |
| 23 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 24 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 1 | P |
| 25-33 | Steps 5 to13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 34 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 35-40 | Steps 15 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed with ‘connected without release’. | - | - | - | - |
| 41 | The SS transmits NSSAI DELETE REQUEST message to delete the Default Configured NSSAI list. | <-- | NSSAI DELETE REQUEST | - | - |
| 42 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 43 | Use AT command and set Default Configured NSSAI to 1 & 2. | - | - | - | - |
| 44 | The SS transmits NSSAI DELETE REQUEST message to delete the Configured NSSAI list for all PLMNs (MCC-MNC =000-000). | <-- | NSSAI DELETE REQUEST | - | - |
| 45 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 46 | The SS transmits NSSAI DELETE REQUEST message to delete the Allowed NSSAI list for all PLMNs (MCC-MNC =000-000). | <-- | NSSAI DELETE REQUEST | - | - |
| 47 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 48 | The SS transmits an RRCRelease message. | - | - | - | - |
| 49 | Switch off procedure in RRC_Idle specified in TS 38.508-1 subclause 4.9.6.1 is performed | - | - | - | - |
| 50 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 51 | Check: Does UE transmit a REGISTRATION REQUEST message including Requested NSSAI? | --> | REGISTRATION REQUEST | 2 | P |
| 52- 60 | Steps 5 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 61 | The SS transmits a REGISTRATION ACCEPT message including Allowed NSSAI. | <-- | REGISTRATION ACCEPT | - | - |
| 62-67 | Steps 15 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed with ‘connected without release’. | - | - | - | - |
| 68 | The SS transmits NSSAI DELETE REQUEST message to delete the Default Configured NSSAI list. | <-- | NSSAI DELETE REQUEST | - | - |
| 69 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 70 | The SS transmits NSSAI DELETE REQUEST message to delete the Configured NSSAI list for all PLMNs (MCC-MNC =000-000). | <-- | NSSAI DELETE REQUEST | - | - |
| 71 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 72 | The SS transmits NSSAI DELETE REQUEST message to delete the Allowed NSSAI list for all PLMNs (MCC-MNC =000-000). | <-- | NSSAI DELETE REQUEST | - | - |
| 73 | UE transmits NSSAI DELETE RESPONSE message. | --> | NSSAI DELETE RESPONSE | - | - |
| 74 | The SS transmits an RRCRelease message. | - | - | - | - |
| 75 | Switch off procedure in RRC_Idle specified in TS 38.508-1 subclause 4.9.6.1 is performed. | - | - | - | - |
| 76 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 77 | Check: Does UE transmit a REGISTRATION REQUEST message not including Requested NSSAI? | --> | REGISTRATION REQUEST | 3 | P |
| 78-93 | Steps 5 to 20a1of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2.2-2 are performed. | - | - | - | - |

9.1.5.1.3a.3.3	Specific message contents

Table 9.1.5.1.3a.3.3-1: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.5.1.3a.3.3-2: REGISTRATION ACCEPT (step 12, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000100’B | 4 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000100’B | 4 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000001’B | 1 |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000101’B | 5 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000010’B | 2 |  |
| Mapped configured SD |  | Not Present |  |  |

Table 9.1.5.1.3a.3.3-2A: PDU SESSION ESTABLISHMENT ACCEPT (step 14 , Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0100’B | SST value 4 |  |

Table 9.1.5.1.3a.3.3-3: NSSAI DELETE REQUEST (step 19, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000010’ | Delete Allowed NSSAI |  |
| Allowed NSSAI |  | 000000 | All PLMNs (3 Octets) |  |
|  |  | 00 | 3GPP access |  |

Table 9.1.5.1.3a.3.3-4: NSSAI DELETE RESPONSE (step 20, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-5: REGISTRATION REQUEST (step 24, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000100’B | 4 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000001’B | 1 |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000101’B | 5 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000010’B | 2 |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: UE may include S-NSSAI 4 or 5 or both 4 and 5 from the configured NSSAI list associated with NGC cell G. |  |  |  |  |

Table 9.1.5.1.3a.3.3-6: REGISTRATION ACCEPT (step 34, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000100’B | 4 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000001’B | 1 |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000101’B | 5 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000010’B | 2 |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: SS will send Allowed NSSAIs based on the Requested NSSAI sent by UE in step 24. |  |  |  |  |

Table 9.1.5.1.3a.3.3-6A: PDU SESSION ESTABLISHMENT ACCEPT (step 36, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0010’B | SST |  |
| SST |  | ‘0000 0100’B | SST value 4 |  |
| Mapped configured SST |  | ‘00000001’B | mapped configured SST value 1 |  |

Table 9.1.5.1.3a.3.3-7: NSSAI DELETE REQUEST (step 41, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000000’ | Delete Default Configured NSSAI |  |

Table 9.1.5.1.3a.3.3-8: NSSAI DELETE RESPONSE (step 42, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-9: NSSAI DELETE REQUEST (step 44, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  |  |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000001’ | Delete Configured NSSAI |  |
| Configured NSSAI |  | 00000000 | All PLMNs |  |

Table 9.1.5.1.3a.3.3-10: NSSAI DELETE RESPONSE (step 45, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-11: NSSAI DELETE REQUEST (step 46, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000010’ | Delete Allowed NSSAI |  |
| Allowed NSSAI |  | 000000 | All PLMNs (3 Octets) |  |
|  |  | 00 | 3GPP access |  |

Table 9.1.5.1.3a.3.3-12: NSSAI DELETE RESPONSE (step 47, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-13: REGISTRATION REQUEST (step 51, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Network slicing indication |  |  |  |  |
| Default configured NSSAI indication |  | 1 | Requested NSSAI created from default configured NSSAI |  |
| Note: UE may include either 1 or 2 or both 1 and 2 NSSAIs. |  |  |  |  |

Table 9.1.5.1.3a.3.3-14: REGISTRATION ACCEPT (step 61, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  | Note |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000001’B |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000010’B | SST and mapped configured SST |  |
| SST |  | ‘00000010’B | 2 |  |
| SD |  | Not Present |  |  |
| Mapped configured SST |  | ‘00000010’B |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Note: SS will send Allowed NSSAIs based on the Requested NSSAI sent by UE in Step 51 |  |  |  |  |

Table 9.1.5.1.3a.3.3-15: NSSAI DELETE REQUEST (step 68, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000000’ | Delete Default Configured NSSAI |  |

Table 9.1.5.1.3a.3.3-16: NSSAI DELETE RESPONSE (step 69, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-17: NSSAI DELETE REQUEST (step 70, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000001’ | Delete Configured NSSAI |  |
| Configured NSSAI |  | 000000 | All PLMNs (3 Octets) |  |

Table 9.1.5.1.3a.3.3-18: NSSAI DELETE RESPONSE (step 71, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-19: NSSAI DELETE REQUEST (step 72, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10000110’ |  |  |
| Delete NSSAI type |  | ‘00000010’ | Delete Allowed NSSAI |  |
| Allowed NSSAI |  | 000000 | All PLMNs (3 Octets) |  |
|  |  | 00 | 3GPP access |  |

Table 9.1.5.1.3a.3.3-20: NSSAI DELETE RESPONSE (step 73, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.509 Table 6.7.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1111 |  |  |
| Skip indicator |  | 0000 |  |  |
| Message type |  | ‘10100111’ |  |  |

Table 9.1.5.1.3a.3.3-21: REGISTRATION REQUEST (step 77, Table 9.1.5.1.3a.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
| Requested NSSAI |  | Not Present |  |  |