Title: 	Discussion on test point selection for Time alignment error for UL MIMO in FR2

Source: 	TTA

- Introduction

At RAN5#78, it was agreed a way forward to define test point selection for Tx/Rx test cases in both FR1 and FR2. This agreement included test point selection analysis justifying test environment, test frequencies, test bandwidths, test subcarrier spacing and UL/DL configuration for each test case. This analysis will be captured in TR 38.905.

The purpose of this contribution is to provide test point selection proposal for Time alignment error UL MIMO test case in FR2.

- Discussion

For Time alignment error for UL MIMO in FR2 test case, we use the test point choice from the relevant LTE and NR FR1 time alignment error for UL-MIMO test case from TS 36.521-1 clause 6.8B.4.1 and TS 38.521-1 clause 6.4D.3 as inspiration:

Table 6.8B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [7] clause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] clause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 6 | 6 |
| 3MHz |  |  | QPSK | 15 | 15 |
| 5MHz |  |  | QPSK | 25 | 25 |
| 10MHz |  |  | QPSK | 50 | 50 |
| 15MHz |  |  | QPSK | 75 | 75 |
| 20MHz |  |  | QPSK | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

Table 6.4D.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Time alignment error for UL MIMO | Modulation | RB allocation (NOTE 1) |
| 1 | test case | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

Test environment for Time alignment error for UL MIMO in LTE and FR1 is Normal Condition. This test environment can be also leveraged for FR2.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Time alignment error for UL MIMO in FR2.

  - Test Frequencies

In LTE and NR FR1, Time alignment error for UL MIMO is tested for Mid frequency. For NR FR2 it is proposed to adopt the same selection of test frequencies.

Proposal 2: Select Mid Frequency for Time alignment error for UL MIMO in FR2.

  - Test Channel Bandwidth

In LTE, this test case is tested for Lowest, 5 MHz, and Highest channel bandwidths. In NR FR1, this test case is tested for Lowest, Mid and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to different frequency selectivities with different channel bandwidths. For NR FR2 it is proposed to adopt the same selection of test channel bandwidth.

Proposal 3: Test the Lowest, Mid and Highest Channel Bandwidth for Time alignment error for UL MIMO in FR2.

  - Test Subcarrier Spacing

In NR FR1, Time alignment error for UL MIMO is tested for Lowest and Highest SCS. For NR FR2, it is proposed to adopt the same selection of test SCS.

Proposal 4: Select the Lowest and Highest SCS supported for Time alignment error for UL MIMO in FR2.

  - Modulations

In LTE, only QPSK is tested, which is generating a stable power for testing. For NR it is proposed to also test QPSK for Time alignment error for UL MIMO measurement.

Since DFT-s-OFDM is disable for UL MIMO case. It is proposed to test CP-OFDM for Time alignment error for UL MIMO measurement in FR2

Proposal 5: Test CP-OFDM QPSK in Time alignment error for UL MIMO in FR2.

  - Uplink configuration

In LTE and NR FR1, Time alignment error for UL MIMO is tested with maximum uplink allocation, which will generate the lowest power spectrum density. For NR FR2 the same configuration can be adopted.

Proposal 6: Use Full RB allocation in the Time alignment error for UL MIMO in FR2.

  - Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.7-1 Total test points of Maximum output power

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 2 | 1 | 6 |

- Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR2 Time alignment error for UL MIMO test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for Time alignment error for UL MIMO in FR2.

Proposal 2: Select Mid Frequency for Time alignment error for UL MIMO in FR2.

Proposal 3: Test the Lowest, Mid and Highest Channel Bandwidth for Time alignment error for UL MIMO in FR2.

Proposal 4: Select the Lowest and Highest SCS supported for Time alignment error for UL MIMO in FR2.

Proposal 5: Test CP-OFDM QPSK in Time alignment error for UL MIMO in FR2.

Proposal 6: Use Full RB allocation in the Time alignment error for UL MIMO in FR2.

- References