3GPP TSG-RAN5 Meeting #92-e	R5-215248

# Online, , 16th Aug 2021 - 27th Aug 2021

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.918 | CR | n/a | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Text Proposal on Analysis of mapping application to network slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Text Proposal on 5.4.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to 5.4.1 to add the analysis of Mapping Application to Network Slicing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.918 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

5.4	Statistical Analysis

5.4.1	Overview of Mapping Application to Network Slicing

When the mapping of applications to network slicing is tested, the fundamental goal is to verify the procedure of associating applications to PDU sessions based on URSP rules. URSP rules contain Traffic Descriptors determining which URSP rule is applicable for the specific application and a list of Route Selection Descriptors determining the corresponding PDU session.

With this knowledge, the following aspects are recommended to be checked to verify the procedure of mapping application to Network Slicing

- The selection of applicable URSP rule according to the Traffic descriptors in UE Route Selection Policy Rule within URSP rules and information from the specific application

- The selection of existing PDU session or establishment of new PDU session for specific application based on the selected Route Selection Descriptors within the applicable URSP rule.

The way to trigger URSP rule selection is UE implementation specific and driven by application. To enable testing then the UE is recommended to be equipped with the application simulation client defined in clause 5.3.1 to trigger the required selection procedures

<<< END OF CHANGES >>>