3GPP TSG-RAN5 Meeting #92-e	R5-215296

# Electronic Meeting, 16th – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1137 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to clause 7.3B.2.0.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UL LCRB for DC_3A-41A_n78A in table 7.3B.2.0.3.5.2-1 is 52 which is incorrect. It should be modified to 50 to align with the core protocol. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating UL LCRB for DC_3A-41A_n78A in table 7.3B.2.0.3.5.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL LCRB for DC_3A-41A_n78A is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

<Unchanged Skipped>

7.3B.2.0.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.0.3.5.2-0: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 |  | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |  |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |  |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |  |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |  |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |  |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |  |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A-7A_n78A DC_1A-7C_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |  |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |  |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |  |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |  |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-8A_n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |  |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |  |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |  |
| DC_1A-18A_n77A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |  |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |  |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |  |
| DC_1A-18A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |  |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |  |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |  |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |  |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |  |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |  |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |  |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |  |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |  |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |  |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |  |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |  |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |  |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |  |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | N/A |  |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |  |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |  |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |  |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |  |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |  |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |  |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |  |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |  |
| DC_1A_n40A-n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 ¦3*fB1-fn40¦ |  |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 ¦3*fB1 -fn78¦ |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |  |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |  |
| DC_1A-41A_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
| DC_1A-41A_n78A | 1 | 1955 | 5 | 25 | 2145 | 8.7 | IMD4 |  |
|  | 41 | 2507.5 | 10 | 50 | 2507.5 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A |  |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |  |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |  |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |  |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |  |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |  |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |  |
| DC_1A-SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |  |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |  |
| DC_1A-SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |  |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |  |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |  |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |  |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |  |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |  |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |  |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |  |
| DC_3A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |  |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |  |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |  |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |  |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |  |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |  |
| DC_3A_n7A-n78A DC_3C_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 ¦2*fn7-fB3¦ |  |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |  |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-19A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | 19 | 840 | 5 | 25 | 885 | [18.5] | IMD3 |  |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |  |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |  |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 3 | 1723 | 5 | 25 | 1818 | 9.4 | IMD4 |  |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |  |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |  |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |  |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 3 | N/A | N/A | N/A | N/A | N/A | IMD2 |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |  |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |  |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |  |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |  |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |  |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |  |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |  |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |  |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |  |
| DC_3A-41A_n77A DC_3A-41C_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |  |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |  |
| DC_3A-41A_n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |  |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |  |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |  |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |  |
| DC_3A-SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |  |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |  |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |  |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |  |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |  |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |  |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |  |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |  |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |  |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
| DC_7A-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |  |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |  |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |  |
|  | 20 | 855 | 5 | 25 | 814 | N/A | N/A |  |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |  |
| DC_7A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |  |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |  |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |  |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_7A-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |  |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |  |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |  |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |  |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |  |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |  |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |  |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |  |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |  |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 | IMD2 |  |
|  | n78 | 3350 | 10 | 50 | 3421 | N/A | N/A |  |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |  |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |  |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |  |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |  |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |  |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |  |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |  |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |  |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |  |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |  |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |  |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |  |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |  |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |  |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |  |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |  |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |  |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |  |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |  |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |  |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |  |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |  |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |  |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |  |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |  |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |  |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |  |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |  |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |  |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |  |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |  |
| DC_20A_n28A-n78A DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |  |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |  |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |  |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |  |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |  |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |  |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |  |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |  |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A |  | N/A |  |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A |  | N/A |  |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 |  | IMD5 |  |

<End of Changes>