3GPP TSG-RAN5 Meeting #92-e	R5-215301

# Electronic Meeting, 16th – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1342 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test case 6.3A.4.2 Relative power tolerance for intra-band non-contiguous UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case 6.3A.4.2 can’t be tested for intra-band non-contiguous UL CA and it need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating 6.3A.4.2 for intra-band non-contiguous UL CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.2A.4.2 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0495 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-215302 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3A.4.2	Relative power tolerance for CA

6.3A.4.2.0	Minimum conformance requirements

For intra-band contiguous and non-contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in subclause 6.3A.1.0 and the total power is limited by PUMAX as defined in subclause 6.2A.4.0. The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.4.3.3-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.4.3.3-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subframes;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.4.3.3-1 for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3A.4.

6.3A.4.2.1	Relative power tolerance for CA (2UL CA)

Editor’s Note: For UL non-contiguous intra-band operation when the signalling is absent for dualPA-Architecture IE, the number of test frequency points is under investigation in TS 38.508-1, e.g. “Low” and “High”, or “Mid”.

6.3A.4.2.1.1	Test purpose

To verify the ability of the UE transmitter to set its output power of each component carrier in a target sub-frame(1ms) relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is less than or equal to 20ms.

6.3A.4.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support intra-band contiguous 2UL CA.

6.3A.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.4.2.0.

6.3A.4.2.1.4	Test description

6.3A.4.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.4.2.1.4.1-1 and table 6.3A.4.2.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.4.2.1.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocations (Note 3) (LCRB @ RBstart) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | 5@(NRB-5) 5@(NRB-5) 1@(NRB-1) 8@(NRB-8) | 1@0 8@0 1@0 8@0 |
| NOTE 1: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 3: The UL allocation is changed as part of the test procedure. The Test Configuration Table entries list the combinations used, with the sequence of usage as determined by the test procedure for each sub-test. |  |  |  |  |

Table 6.3A.4.2.1.4.1-2: Test Configuration Table for intra-band non-contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to Range column |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |
| Test ID |  | Range |  |  | DL Config |  | Uplink Configuration |  |  |
|  |  |  |  |  |  |  | Modulation for all CCs | RB allocations (Note 3) (LCRB @ RBstart) |  |
|  | PCC | Wgap | SCC |  |  | PCC | SCC |
| 1 | CC1 | Max Wgap | CC2 | N/A | DFT-s-OFDM QPSK | 5@0 5@0 1@0 8@0 | 1@0 8@0 1@0 8@0 |
| NOTE 1: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.2-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 3: The UL allocation is changed as part of the test procedure. The Test Configuration Table entries list the combinations used, with the sequence of usage as determined by the test procedure for each sub-test. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.2.1.4.1-1 and Table 6.3A.2.1.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.4.2.1.4.3.

6.3A.4.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause5.5.1. Message contents are defined in clause 6.3A.4.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133, clause 9.3).

4.	The procedure is separated in various subtests to verify different aspects of relative power control. The power changes of the subtests are shown by diagrams in the Test Procedure. In this test case, the term PCMAX_L  – MAX{TL, TLOW(PCMAX_L) } defined in TS 38.101 [2] clause 6.2.4A is used, to ensure the UE is not tested outside its power capability.

5.	Sub test: SCC power increase

5.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.4.2.1.4.1-1 for intra-band contiguous CA and Table 6.3A.4.2.1.4.1-2 for intra-band non-contiguous CA on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3A.4.2.1.4.2-1. The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. Measure PCCRefMeas, 0 and SCCRefMeas, 0 in the Reference subframe, and after the SCC allocation is increased, measure PCCTargetMeas, 0 and SCCTargetMeas, 0 in the Target subframe.

Table 6.3A.4.2.1.4.2-1: Power settings and RB allocations for SCC power increase, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +7 | SCCRefSet, 0, dBm/NRB alloc | -17 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +7 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

5.2.	Calculate the Total uplink power across both CCs in dBm as 10log10((PCCTargetMeas, n in mW) + (SCCTargetMeas, n in mW)). If (PCMAX_L  – MAX{TL, TLOW(PCMAX_L) } - Total uplink power) > 1dB, continue to step 5.3. Otherwise, go to step 5.6.

5.3.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3A.4.2.1.5-1. If the result meets the Test requirement, continue to step 5.4. Otherwise, fail the UE for this subtest.

5.4.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3A.4.2.1.5-1. If the result meets the Test requirement, continue to step 5.5. Otherwise, fail the UE for this subtest.

5.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3A.4.2.1.4.2-2. Measure PCCRefMeas, n and SCCRefMeas, n.in the Reference subframe, and after the SCC allocation is increased, measure PCCTargetMeas, n and SCCTargetMeas, n.in the Target subframe.  Repeat steps 5.2 to 5.4.

Table 6.3A.4.2.1.4.2-2: Power settings and RB allocations for SCC power increase, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +7 | SCCRefSet, n+1, dBm/NRB alloc | SCCTargetMeas, n +2dB) |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +7 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

5.6.	If the requirements specified in Table 6.3A.4.2.1.5-1 are all met, pass the UE for this subtest.

6.	Sub test: SCC power decrease

6.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.4.2.1.4.1-1 for intra-band contiguous CA and Table 6.3A.4.2.1.4.1-2 for intra-band non-contiguous CA on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3A.4.2.1.4.2-3.The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. Measure PCCRefMeas, 0 and SCCRefMeas, 0 in the Reference subframe, and after the SCC allocation is decreased, measure PCCTargetMeas, 0 and SCCTargetMeas, 0 in the Target subframe.

Table 6.3A.4.2.1.4.2-3: Power settings and RB allocations for SCC power decrease, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -2 | SCCRefSet, 0, dBm/NRB alloc | PCMAX_L – MAX{TL, TLOW(PCMAX_L) } -5 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -2 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

6.2.	If the uplink (power for each CC – (-20dBm)) is > 1dB, continue to step 6.3. Otherwise, go to step 6.6.

6.3.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3A.4.2.1.5-2. If the result meets the Test requirement, continue to step 6.4.  Otherwise, fail the UE for this subtest.

6.4.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3A.4.2.1.5-2. If the result meets the Test requirement, continue to step 6.5. Otherwise, fail the UE for this subtest.

6.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3A.4.2.1.4.2-4. Measure PCCRefMeas, n and SCCRefMeas, n.in the Reference subframe, and after the SCC allocation is decreased, measure PCCTargetMeas, n and SCCTargetMeas, n.in the Target subframe. Repeat steps 6.2 to 6.4.

Table 6.3A.4.2.1.4.2-4: Power settings and RB allocations for SCC power decrease, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -2 | SCCRefSet, n+1, dBm/NRB alloc | SCCTargetMeas, n -2dB |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -2 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -9 |
| PCC allocation, NRB alloc | 5 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

6.6.	If the requirements specified in Table 6.3A.4.2.1.5-2 are all met, pass the UE for this subtest.

7.	Sub test: PCC and SCC power increase together

7.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.4.2.1.4.1-1 for intra-band contiguous CA and Table 6.3A.4.2.1.4.1-2 for intra-band non-contiguous CA on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3A.4.2.1.4.2-5. The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. Measure PCCRefMeas, 0 and SCCRefMeas, 0 in the Reference subframe, and after the PCC and SCC allocation are increased, measure PCCTargetMeas, 0 and SCCTargetMeas, 0 in the Target subframe.

Table 6.3A.4.2.1.4.2-5: Power settings and RB allocations for PCC and SCC power increase, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | -17 | SCCRefSet, 0, dBm/NRB alloc | -17 |
| PCC allocation, NRB alloc | 1 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (PCCRefSet, 0) +9 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) +9 |
| PCC allocation, NRB alloc | 8 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

7.2.	Calculate the Total uplink power across both CCs in dBm as 10log10((PCCTargetMeas, n in mW) + (SCCTargetMeas, n in mW)). If (PCMAX_L  – MAX{TL, TLOW(PCMAX_L) } - Total uplink power) > 1dB, continue to step 7.3. Otherwise, go to step 7.6.

7.3.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3A.4.2.1.5-3. If the result meets the Test requirement, continue to step 7.4. Otherwise, fail the UE for this subtest.

7.4.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3A.4.2.1.5-3. If the result meets the Test requirement, continue to step 7.5. Otherwise, fail the UE for this subtest.

7.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3A.4.2.1.4.2-6. Measure PCCRefMeas, n and SCCRefMeas, n.in the Reference subframe, and after the PCC and SCC allocation are increased, measure PCCTargetMeas, n and SCCTargetMeas, n.in the Target subframe.  Repeat steps 7.2 to 7.4.

Table 6.3A.4.2.1.4.2-6: Power settings and RB allocations for PCC and SCC power increase, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (Max (PCCTargetMeas, n, SCCTargetMeas, n)) +2dB | SCCRefSet, n+1, dBm/NRB alloc | (Max (PCCTargetMeas, n, SCCTargetMeas, n)) +2dB |
| PCC allocation, NRB alloc | 1 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +9 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) +9 |
| PCC allocation, NRB alloc | 8 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

7.6.	If the requirements specified in Table 6.3A.4.2.1.5-3 are all met, pass the UE for this subtest.

8.	Sub test: PCC and SCC power decrease together

8.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.4.2.1.4.1-1 for intra-band contiguous CA and Table 6.3A.4.2.1.4.1-2 for intra-band non-contiguous CA on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, 0 and SCCRefSet, 0 respectively, as defined in Table 6.3A.4.2.1.4.2-7. The powers and allocations are chosen so the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame. Measure PCCRefMeas, 0 and SCCRefMeas, 0 in the Reference subframe, and after the PCC and SCC allocation are decreased, measure PCCTargetMeas, 0 and SCCTargetMeas, 0 in the Target subframe.

Table 6.3A.4.2.1.4.2-7: Power settings and RB allocations for PCC and SCC power decrease, step n=0

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, 0, dBm/NRB alloc | PCMAX_L – MAX{TL, TLOW(PCMAX_L) } -6 | SCCRefSet, 0, dBm/NRB alloc | PCMAX_L – MAX{TL, TLOW(PCMAX_L) } -6 |
| PCC allocation, NRB alloc | 8 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, 0 | Measured power, dBm/NRB alloc | SCCRefMeas, 0 |
| Target subframe |  |  |  |
| PCCTargetSet, 0, dBm/NRB alloc | (PCCRefSet, 0) -9 | SCCTargetSet, 0, dBm/NRB alloc | (SCCRefSet, 0) -9 |
| PCC allocation, NRB alloc | 1 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, 0 | Measured power, dBm/NRB alloc | SCCTargetMeas, 0 |

8.2.	If the uplink (power for each CC – (-20dBm)) is > 1dB, continue to step 8.3. Otherwise, go to step 8.6.

8.3.	For the PCC, calculate the change in power as (PCCTargetMeas, n - PCCRefMeas, n) and compare to the PCC Test requirement specified in Table 6.3A.4.2.1.5-4. If the result meets the Test requirement, continue to step 8.4. Otherwise, fail the UE for this subtest.

8.4.	For the SCC, calculate the change in power as (SCCTargetMeas, n - SCCRefMeas, n) and compare to the SCC Test requirement specified in Table 6.3A.4.2.1.5-4. If the result meets the Test requirement, continue to step 8.5. Otherwise, fail the UE for this subtest.

8.5.	Send the appropriate TPC commands for PUSCH on each component carrier to the UE to ensure that the UE transmits PUSCH on the PCC and on the SCC with powers nearest to PCCRefSet, n+1 and SCCRefSet, n+1 respectively, as defined in Table 6.3A.4.2.1.4.2-8. Measure PCCRefMeas, n and SCCRefMeas, n.in the Reference subframe, and after the PCC and SCC allocation are decreased, measure PCCTargetMeas, n and SCCTargetMeas, n.in the Target subframe. Repeat steps 8.2 to 8.4.

Table 6.3A.4.2.1.4.2-8: Power settings and RB allocations for PCC and SCC power decrease, step n+1

| PCC |  | SCC |  |
| --- | --- | --- | --- |
| Parameter | Value | Parameter | Value |
| Reference subframe |  |  |  |
| PCCRefSet, n+1, dBm/NRB alloc | (Min (PCCTargetMeas, n, SCCTargetMeas, n)) -2dB | SCCRefSet, n+1, dBm/NRB alloc | (Min (PCCTargetMeas, n, SCCTargetMeas, n)) -2dB |
| PCC allocation, NRB alloc | 8 | SCC allocation, NRB alloc | 8 |
| Measured power, dBm/NRB alloc | PCCRefMeas, n+1 | Measured power, dBm/NRB alloc | SCCRefMeas, n+1 |
| Target subframe |  |  |  |
| PCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -9 | SCCTargetSet, n+1, dBm/NRB alloc | (SCCRefSet, n+1) -9 |
| PCC allocation, NRB alloc | 1 | SCC allocation, NRB alloc | 1 |
| Measured power, dBm/NRB alloc | PCCTargetMeas, n+1 | Measured power, dBm/NRB alloc | SCCTargetMeas, n+1 |

8.6.	If the requirements specified in Table 6.3A.4.2.1.5-4 are all met, pass the UE for this subtest.

6.3A.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exception:

Table 6.3A.4.2.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.3A.4.2.1.5	Test requirement

For intra-band contiguous carrier aggregation bandwidth class C and intra-band non-contiguous CA, the relative power control tolerance per component carrier measured in steps 5, 6, 7 and 8 of the test procedures should satisfy the applicable test requirements specified in Tables 6.3A.4.2.1.5-1 to 6.3A.4.2.1.5-5.

Table 6.3A.4.2.1.5-1: Test requirements for SCC power increase

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -0.7-TT | 0.7+TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | 5.5-TT | 12.5+TT |

Table 6.3A.4.2.1.5-2: Test requirements for SCC power decrease

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -0.7-TT | 0.7+TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | -12.5-TT | -5.5+TT |

Table 6.3A.4.2.1.5-3: Test requirements for PCC and SCC power increase together

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | 5.5-TT | 12.5+TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | 5.5-TT | 12.5+TT |

Table 6.3A.4.2.1.5-4: Test requirements for PCC and SCC power decrease together

| Parameter | Condition | Unit | Minimum | Maximum |
| --- | --- | --- | --- | --- |
| (PCCTargetMeas, n - PCCRefMeas, n) | Normal | dB | -12.5-TT | -5.5+TT |
| (SCCTargetMeas, n - SCCRefMeas, n) | Normal | dB | -12.5-TT | -5.5+TT |

Table 6.3A.4.2.1.5-5: Test Tolerance

|  | f ≤ 6.0GHz |
| --- | --- |
| BW ≤ 100MHz | 0.7 dB |

<End of Changes>