3GPP TSG RAN WG5 Meeting #92-e	R5-215405

Electronic Meeting, 16th Aug 2021 – 27th Aug 2021

Agenda Item:	6.3.2.20

Source:	Ericsson

Title:	URSP and SST

Document for:	Endorsement

# 1	Introduction

Session Management includes UE to provide S-NSSAI according TS 23.501 [1], 5.6 Session Management

“(b)	S-NSSAI of the HPLMN that matches the application (that is triggering the PDU Session Request) within the NSSP in the URSP rules or within the UE Local Configuration as defined in clause 6.1.2.2.1 of TS 23.503 [45].”

The purpose of the S-NSSAI information element is to identify a network slice.

# 2	Discussion

UE policy control

According TS 23.503 [2], 6.1.2.2 UE policy control

“The ANDSP and URSP may be pre-configured in the UE or may be provisioned to UE from PCF. The pre-configured policy shall be applied by the UE only when it has not received the same type of policy from PCF”

The UE Route Selection Policy (URSP) might be pre-configured in the UE and can include any SST.

In TS 38.508-1 [3] the standardised SST values 1 (eMBB), 2 (URLLC) and 4 (V2X) are agreed.

Observation 2-1: The SST value 3 (MIoT) is not yet agreed.

This Slice/Service type is a Slice suitable for the handling of massive IoT.

RAN2 enhancement are added with WI NR_IIOT, e.g. support for PDCP duplication with more than two RLC entities in PDCP config. This is captured in TS 38.523-1 [4] test case 7.1.3.5.6 and added as part of WI NR_IioT-UEConTest.

Proposal 2-1:

Add SST value 3 (MIoT) in TS 38.508-1 [3].

# 3	Proposal

It’s proposed to implement the proposal 2-1.

# 4	References

[1] 3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System".

[2] 3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[3] 3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[4] 3GPP TS 38.523-1: "5GS; User Equipment (UE) conformance specification; Part 1: Protocol".