3GPP TSG RAN Meeting #92-e	 	R5-215485

Electronic Meeting, 16th – 27th August 2021

Source:		Rohde & Schwarz

Title:	On the relative power measurement MU

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

At RAN5 #91-e, the discussion of the relative power measurement uncertainty has been continued [1 – 4]. In this contribution, we discuss this contributor with respect to the relative power tolerance test case.

# Discussion

Relative power measurements are performed for setting the UE output power in several test cases with the power control window method. While in this application the difference between the measured power levels is mainly given by the small last applied step size and associated UE power tolerance, much larger power differences occur in the relative power tolerance test case.

At RAN5 #92-e the following proposals of [2] have been agreed:

Proposal 1: Decide first on the largest testable power step size in the power tolerance test case before finalizing the relative power measurement MU.

Proposal 2: Apply a maximum allocation of 66 RBs in order to enable testing of UEs supporting only 120 kHz SCS.

Proposal 5: Apply a central RB allocation in the power tolerance test case in order to optimize the relative power measurement MU.

In Ref. [6], it has been agreed to limit the allocation in the relative power tolerance test case to 20 RBs.

A RB change from 1 RB to 20 RBs results ideally in a power change of 13 dB. Combined with 1dB due to the TPC command and with the UE power tolerance of 10 dB for a power levels in the lower range (Pmin ≤ P ≤ Pint), this results in an allowed power change of 24 dB.

The measured step size will be smaller than the real step size for low power values due to the influence of noise. At large signal powers the measured power step size will be reduced due to compression. If both distortions are not too large, an intermediate power range exists where the step size can be reasonably measured.

Preliminary analysis at 28 GHz shows that such a range exists with a size of roughly 20 dB based on 1dB influence of noise and a compression of 0.5 dB of the LNA for the higher power levels.

Proposal 1: Consider as an MU contributor the compression of the amplifier in addition of the MU of the power measurement device.

Due to the RB change the signal frequency range is altered. Therefore, the frequency characteristics of the power measurement device and amplifier needs to be considered.

Observation 1: In the power tolerance test, the frequency range of the UL signal is changed due to the RB step. Therefore,  frequency characteristics of the test equipment need be considered.

Proposal 2: Consider the impact of frequency response on the relative power MU.

# Conclusions

The relative power measurement MU has been discussed for the relative power tolerance test case.

Proposal 1: Consider as an MU contributor the compression of the amplifier in addition of the MU of the power measurement device.

Observation 1: In the power tolerance test, the frequency range of the UL signal is changed due to the RB step. Therefore,  frequency characteristics of the test equipment need be considered.

Proposal 2: Consider the impact of frequency response on the relative power MU.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 100 MHz |

# References

- R5-210845, On FR2 relative power measurement uncertainty, Anritsu, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-213313, On the relative power measurement MU, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-213807, On FR2 relative power MU and power control test case testability, Anritsu, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-213814, On FR2 relative and aggregate power tolerance Measurement Uncertainties and test tolerances, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-214070, Correction of power control in 38.905, Anritsu, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021