3GPP TSG-RAN5 Meeting # 92-e	R5-215922

# Online, 16th  - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5332 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Rx Test Cases for CA_66A-66A-66A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Rx Test Cases contain no minimum requirements, test points or test requirements for CA_66A-66A-66A or 3 DL intra-band non-contiguous CA in general, although the band combination has be marked as 100% in the workplan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of minimum requirements. Addtion of new test points for CA_66A-66A-66A Update of test requirements for CA_66A-66A-66A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_66A-66A-66A cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.5, 7.4A.0, 7.4A.5, 7.4A.5_H, 7.5A.0, 7.5A.5, 7.6.1A.0, 7.6.1A.5, 7.6.2A.0, 7.6.2A.5, 7.6.3A.0, 7.6.3A.5, 7.7A.0, 7.8.1A.0, 7.8.1A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.5	Reference sensitivity level for 3DL CA

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	TR 36.905 test point analysis for CA_28A-40C is missing, meaning it is not yet determined if testing of fallback configuration CA_28A_40A can be skipped.

-	Band 70 CA configurations are missing test point analysis.

#### 7.3A.5.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.5.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

#### 7.3A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.5.4	Test description

##### 7.3A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.5.4.1-1 through table 7.3A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.3A.5.4.1-2: Test Configuration Table Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB" and “SCCs NRB ” columns |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  |  |  |  |  | Wgap [MHz] |  | SCC1 |  |  |  |  |  |  |  |  |  |  | Wgap [MHz] |  | SCC2 |  |  |  |  |  |
|  | Band |  | Range |  |  |  | NRB |  |  |  |  |  | Band |  |  | Range |  |  | NRB |  |  |  |  |  |  | Band |  | Range |  |  | NRB |
|  | UL MOD |  | DL MOD |  |  |  | UL alloc (Note 2,3,4) |  | DL alloc |  |  |  | UL MOD |  |  | DL MOD |  |  | UL alloc |  |  | DL alloc |  |  |  | UL MOD |  | DL MOD |  |  | UL alloc | DL alloc |  |
| Group Test Settings for CA_1A-1A-7A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 5 |  | 1 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 7 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 7 |  | Mid |  |  |  | 75@0 |  | All RBs |  | - |  | 1 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | 20 |  | 1 |  | CC2 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 1 |  | CC2 |  |  |  | 90@10 |  | All RBs |  | 35 |  | 1 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 7 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 1 |  | CC2 |  |  |  | 90@10 |  | All RBs |  | 20 |  | 1 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 7 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 1 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 20 |  | 1 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 7 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_1A-7A-7A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Mid |  |  |  | 100@0 |  | All RBs |  | - |  | 7 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | 30 |  | 7 |  | CC2 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 7 |  | CC2 |  |  |  | 75@25 |  | All RBs |  | 30 |  | 7 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 1 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 7 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 30 |  | 7 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 1 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 7 |  | CC2 |  |  |  | 45@55 |  | All RBs |  | 15 |  | 7 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 1 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 7 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 45 |  | 7 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 1 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 7 |  | CC2 |  |  |  | 45@55 |  | All RBs |  | 25 |  | 7 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 1 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_2A-2A-XA or CA_XA-2A-2A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110 | 2 |  | CC2 |  |  |  | 16@57 |  | All RBs |  | 35 |  | 2 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 211 | 2 |  | CC2 |  |  |  | 16@39 |  | All RBs |  | 20 |  | 2 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | 2 |  | CC2 |  |  |  | 16@57 |  | All RBs |  | 20 |  | 2 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 4 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 2 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 20 |  | 2 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_2A-2A-71A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | CC2 |  |  |  | 50@50 |  | All RBs |  | 35 |  | 2 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 71 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 210 | 2 |  | CC2 |  |  |  | 16@57 |  | All RBs |  | 35 |  | 2 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 71 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 2 |  | CC2 |  |  |  | 50@50 |  | All RBs |  | 20 |  | 2 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 71 |  | NOTE 21 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 2 |  | CC2 |  |  |  | 16@57 |  | All RBs |  | 20 |  | 2 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 71 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 2 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 20 |  | 2 |  | CC1 |  |  | N/A |  |  |  | All RBs |  | - |  | 71 |  | High |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 71 |  | Low |  |  |  | 25@0 |  | All RBs |  | - |  | 2 |  | CC2 |  |  | N/A |  |  |  | All RBs |  | 35 |  | 2 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 7 | 71 |  | Low |  |  |  | 25@12 |  | All RBs |  | - |  | 2 |  | CC2 |  |  | N/A |  |  |  | All RBs |  | 30 |  | 2 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 71 |  | Low |  |  |  | 20@25 |  | All RBs |  | - |  | 2 |  | CC2 |  |  | N/A |  |  |  | All RBs |  | 25 |  | 2 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 75 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 75 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 71 |  | Low |  |  |  | 20@40 |  | All RBs |  | - |  | 2 |  | CC2 |  |  | N/A |  |  |  | All RBs |  | 20 |  | 2 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 10 | 71 |  | Mid |  |  |  | 100@0 |  | All RBs |  | - |  | 2 |  | CC2 |  |  | N/A |  |  |  | All RBs |  | 20 |  | 2 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_3A-3A-XA or CA_XA-3A-3A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 118 | 3 |  | CC2 |  |  |  | 16@50 |  | All RBs |  | 50 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 218 | 3 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 15 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 318 | 3 |  | CC2 |  |  |  | 16@50 |  | All RBs |  | 35 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 418 | 3 |  | CC2 |  |  |  | 16@50 |  | All RBs |  | 45 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 518 | 3 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 10 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 618 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 3 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 35 |  | 3 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 718 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 3 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 10 |  | 3 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| Test Settings for CA_3A-3A-8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122 | 3 |  | CC2 |  |  |  | 16@50 |  | All RBs |  | 50 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 8 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 222 | 3 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 15 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 8 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 322 | 3 |  | CC2 |  |  |  | 16@50 |  | All RBs |  | 45 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 8 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 422 | 3 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 10 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 8 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 522 | 3 |  | CC2 |  |  |  | 16@50 |  | All RBs |  | 35 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 8 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 622 | 3 |  | CC2 |  |  |  | 12@25 |  | All RBs |  | 55 |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 8 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 722 | 8 |  | Low |  |  |  | 25@25 |  | All RBs |  | - |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | 45 |  | 3 |  | CC2 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 822 | 8 |  | Low |  |  |  | 25@25 |  | All RBs |  | - |  | 3 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | 45 |  | 3 |  | CC2 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_4A-4A-XA or CA_XA-4A-4A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 20 |  | 4 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 4 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 5 |  | 4 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 4 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 5 |  | 4 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_4A-4A-71AConfigurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 |  | CC2 |  |  | 100@0 |  |  | All RBs |  | 20 |  | 4 |  | CC1 |  |  | N/A |  |  | All RBs |  |  | - |  | 71 |  | Mid |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 25 |  |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 4 |  | CC2 |  |  | 100@0 |  |  | All RBs |  | 5 |  | 4 |  | CC1 |  |  | N/A |  |  | All RBs |  |  | - |  | 71 |  | Mid |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | 71 |  | Mid |  |  | REFSENS |  |  | All RBs |  | - |  | 4 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | 5 |  | 4 |  | CC1 |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_4A-4A-12A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 20 |  | 4 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 12 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 2 | 4 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 20 |  | 4 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 12 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 3 | 12 |  | Mid |  |  |  | 20@30 |  | All RBs |  | - |  | 4 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 20 |  | 4 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 25 |  |
| 4 | 12 |  | Mid |  |  |  | 8@427 |  | All RBs |  | - |  | 4 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 5 |  | 4 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_4A-4A-13A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 4 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 20 |  | 4 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 13 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 4 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 5 |  | 4 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 13 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | 13 |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 4 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 5 |  | 4 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 4 |  | CC1 |  |  |  | 50@0 |  | All RBs |  | 10 |  | 4 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | 13 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| Test Settings for CA_5A-5A-66A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | CC2 |  |  | 50@0 |  |  | All RBs |  | 50 |  | 5 |  | CC1 |  |  | N/A |  |  | All RBs |  |  | - |  | 66 |  | Mid |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 25 |  |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 5 |  | CC2 |  |  | 50@0 |  |  | All RBs |  | 50 |  | 5 |  | CC1 |  |  | N/A |  |  | All RBs |  |  | - |  | 66 |  | Mid |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 50 |  |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | 66 |  | Mid |  |  | REFSENS |  |  | All RBs |  | - |  | 5 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | 50 |  | 5 |  | CC1 |  | N/A |  | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  |  |  |  | 50 |  |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| Group Test Settings for CA_7A-7A-XA or CA_XA-7A-7A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 119 | 7 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 45 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 219 | 7 |  | CC2 |  |  |  | 45@55 |  | All RBs |  | 25 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 319 | 7 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 40 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 419 | 7 |  | CC2 |  |  |  | 45@55 |  | All RBs |  | 20 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 519 | 7 |  | CC2 |  |  |  | 36@64 |  | All RBs |  | 35 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 75 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 619 | 7 |  | CC2 |  |  |  | 50@50 |  | All RBs |  | 15 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 75 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 719 | 7 |  | CC2 |  |  |  | 32@68 |  | All RBs |  | 30 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 819 | 7 |  | CC2 |  |  |  | 45@55 |  | All RBs |  | 15 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 919 | 7 |  | CC2 |  |  |  | 32@18 |  | All RBs |  | 40 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 1019 | 7 |  | CC2 |  |  |  | 50@0 |  | All RBs |  | 15 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 1119 | 7 |  | CC2 |  |  |  | 32@18 |  | All RBs |  | 55 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 1219 | 7 |  | CC2 |  |  |  | 50@0 |  | All RBs |  | 30 |  | 7 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 1319 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 7 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 40 |  | 7 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1419 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 7 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 30 |  | 7 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_12A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | CC1 |  |  | 100@0 |  |  | All RBs |  | 65 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | 12 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 2 | 66 |  | CC1 |  |  | 100@0 |  |  | All RBs |  | 50 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | 12 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 3 | 12 |  | Low |  |  | 20@5 |  |  | All RBs |  | - |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 65 |  | 66 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 25 |  |
| 4 | 12 |  | Mid |  |  | 20@5 |  |  | All RBs |  | - |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 60 |  | 66 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 5 | 12 |  | High |  |  | 20@30 |  |  | All RBs |  | - |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 75 |  | 66 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 75 |  |
| 6 | 12 |  | Mid |  |  | 20@30 |  |  | All RBs |  | - |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 50 |  | 66 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_29A-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | CC1 |  |  |  | 100@0 |  | All RBs |  | 50 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | 29 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| 2 | 66 |  | CC1 |  |  |  | 100@0 |  | All RBs |  | 65 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | 29 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 50 |  |
| Test Settings for CA_66A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | CC1 |  |  |  | 100@0 |  | All RBs |  | 15 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 15 |  | 66 |  | CC3 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 66 |  | CC1 |  |  |  | 100@0 |  | All RBs |  | 22.5 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 22.5 |  | 66 |  | CC3 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 25 |  |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | CC1 |  |  |  | 100@0 |  | All RBs |  | 65 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 66 |  | CC1 |  |  |  | 100@0 |  | All RBs |  | 50 |  | 66 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 66 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | 50 |  | 66 |  | CC2 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Group Test Settings for CA_42A-42A-XA or CA_XA-42A-42A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 42 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 175 |  | 42 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 42 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 160 |  | 42 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | X |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | X |  | Mid |  |  |  | REFSENS |  | All RBs |  | - |  | 42 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 160 |  | 42 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_7A-42A-42A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 42 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 175 |  | 42 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 7 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | 42 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 160 |  | 42 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 7 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | 7 |  | Mid |  |  |  | 75@25 |  | All RBs |  | - |  | 42 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 160 |  | 42 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | Highest NRB |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_20A-42A-42A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Mid |  |  |  | 25@37 |  | All RBs |  | - |  | 42 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 160 |  | 42 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 20 |  | Mid |  |  |  | 20@16 |  | All RBs |  | - |  | 42 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 70 |  | 42 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 42 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 175 |  | 42 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 20 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 25 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 42 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 160 |  | 42 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 20 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for a CA_38A-40A-40A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 38 |  | Mid |  |  |  | 100@0 |  | All RBs |  | - |  | 40 |  |  | CC2 |  |  | N/A |  |  | All RBs |  | 60 |  | 40 |  | CC1 |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 40 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 70 |  | 40 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 38 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 50 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 40 |  | CC2 |  |  |  | 100@0 |  | All RBs |  | 60 |  | 40 |  |  | CC1 |  |  | N/A |  |  | All RBs |  | - |  | 38 |  | Mid |  |  | N/A | All RBs |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 |  |  | 100@0 |  |  | All RBs |  | 149 |  | 41 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | - |  | 41 |  | CC1 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 25 |
| 2 | 41 | CC3 |  |  | 100@0 |  |  | All RBs |  | 134.2 |  | 41 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | - |  | 41 |  | CC1 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for a CA_41A-41C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 |  |  | 100@0 |  |  | All RBs |  | - |  | 41 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | 149 |  | 41 |  | CC1 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 25 |
| 2 | 41 | CC3 |  |  | 100@0 |  |  | All RBs |  | - |  | 41 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | 134.2 |  | 41 |  | CC1 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for a CA_42A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 42 | CC3 |  |  | 100@0 |  |  | All RBs |  | - |  | 42 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | 155.2 |  | 42 |  | CC1 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 25 |
| 2 | 42 | CC3 |  |  | 100@0 |  |  | All RBs |  | - |  | 42 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | 140.2 |  | 42 |  | CC1 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for CA_46A-46A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low |  | 100@0 |  |  |  | All RBs |  | - |  | 46 |  | CC1 |  |  | N/A |  |  | All RBs |  |  | 725 |  | 46 |  | CC2 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |
| 2 | 66 | High |  | 100@0 |  |  |  | All RBs |  | - |  | 46 |  | CC1 |  |  | N/A |  |  | All RBs |  |  | 725 |  | 46 |  | CC2 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |
| Test Settings for a CA_66A-66C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 |  | 100@0 |  |  |  | All RBs |  | 45 |  | 66 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | - |  | 66 |  | CC3 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 25 |
| 2 | 66 | CC1 |  | 100@0 |  |  |  | All RBs |  | 30.2 |  | 66 |  | CC2 |  |  | N/A |  |  | All RBs |  |  | - |  | 66 |  | CC3 |  | N/A |  |  |  | All RBs |
|  | QPSK | QPSK |  |  |  |  |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |  |  |  |  | N/A |  | QPSK |  |  |  |  |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 or Y=5, X =2 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Inter-band: Configuration for the low band for which the 3rd harmonic is within transmission bandwidth of the high band, as specified in Table 7.3A.0-0a Note 5. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: Not applicable if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 12: Void Note 13: Void Note 14: Void Note 15: Void Note 16: Void Note 17 Void Note 18: Applicable test configuratons are specified in Table 7.3A.5.4.1-2a. Note 19: Applicable test configurations are specified in Table 7.3A.5.4.1-2b. Note 20: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 21: Band 71:fUL = 679.3 MHz (NUL = 133285), fDL = 633.3 MHz (NDL = 68749) Note 22: Applicable test configurations are specified in Table 7.3A.5.4.1-2c. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-2a: Test Configuration Applicability for CA_3A-3A-XA or CA_XA-3A-3A

| ID | Supported BCS |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 0,1 |
|  | Associated CA_3A-3A BCS(Note1) |  |  |
|  | 0 | 1 | 0,1 |
| 1 | x | x | x |
| 2 | x | x | x |
| 3 | x | - | x |
| 4 | - | x | - |
| 5 | - | x | - |
| 6 | x | - | x |
| 7 | - | x | - |
| Note 1: Associated CA_3A-3A BCS for each BCS of CA_3A-3A-XA or CA_XA-3A-3A is specified in Table 5.4.2A.1-1 to Table 5.4.2A.1-3 except CA_3A-3A-8A. |  |  |  |

Table 7.3A.5.4.1-2b: Test Configuration Applicability for CA_7A-7A-XA or CA_XA-7A-7A

| ID | Supported BCS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  | 1 |  |  | 0,1 |
|  | Associated CA_7A-7A BCS(Note1) |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 1 | 2 | 3 | 1,2 |
| 1 | x | - | - | x | - | - | x |
| 2 | x | - | - | x | - | - | x |
| 3 | - | - | x | - | - | x | - |
| 4 | - | - | x | - | - | x | - |
| 5 | - | - | - | - | - | - | - |
| 6 | - | - | - | - | - | - | - |
| 7 | x | - | x | x | - | x | x |
| 8 | x | - | x | x | - | x | x |
| 9 | - | x | - | - | x | - | - |
| 10 | - | x | - | - | x | - | - |
| 11 | - | x | - | - | x | - | - |
| 12 | - | x | - | - | x | - | - |
| 13 | - | x | - | - | x | - | - |
| 14 | x | - | x | x | - | x | x |
| Note 1: Associated CA_7A-7A BCS for each BCS of CA_7A-7A-XA or CA_XA-7A-7A is specified in Table 5.4.2A.1-1 to Table 5.4.2A.1-3. |  |  |  |  |  |  |  |

Table 7.3A.5.4.1-2c: Test Configuration Applicability for CA_3A-3A-8A

| ID | Supported BCS |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 0,1 |
|  | Associated CA_3A-3A-8A BCS(Note1) |  |  |
|  | 0 | 1 | 0,1 |
| 1 | x | x | x |
| 2 | x | x | x |
| 3 | - | x | - |
| 4 | - | x | - |
| 5 | x | - | x |
| 6 | - | x | - |
| 7 | - | x | - |
| 8 | x | - | x |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.55 if the UE supports 2 Rx antennas or Figure A.80 if the UE supports 4 Rx antennas in at least one band within the CA configuration.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.3A.5.4.1-1 through Table7.3A.5.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.3A.5.4.3.

##### 7.3A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2. The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.3A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.3A.5.4.1-1 through Table7.3A.5.4.1-2 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.3A.5.4.1-1 through Table7.3A.5.4.1-2 as appropriate on PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the appropriate REFSENS value defined in Table 7.3A.5.5-1 through Table7.3A.5.5-8. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for duration sufficient to achieve statistical significance according to Annex G.2A.

##### 7.3A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

#### 7.3A.5.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

< Unchanged tables omitted >

For combinations of intra-band non-contiguous and intra-band contiguous carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-7. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-7: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + intra-band contiguous

| CA Configuration | Testpoint ID | PCC |  | SCC1 |  | SCC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | Requirement (dBm) | NRB | Requirement (dBm) | NRB | Requirement (dBm) |
| CA_41A-41C | 1 | 100 | -91.3 | 100 | -91.3 | 25 | -97.3 |
|  |  |  | 94.06 |  | 94.06 |  | 100.06 |
|  | 2 | 100 | -91.3 | 100 | -91.3 | 100 | -91.3 |
|  |  |  | 94.06 |  | 94.06 |  | 94.06 |
| CA_41C-41A | 1 | 100 | -91.3 | 100 | -91.3 | 25 | -97.3 |
|  |  |  | 94.06 |  | 94.06 |  | 100.06 |
|  | 2 | 100 | -91.3 | 100 | -91.3 | 100 | -91.3 |
|  |  |  | 94.06 |  | 94.06 |  | 94.06 |
| CA_42A-42C | 1 | 100 | -92.3 | 100 | -92.3 | 25 | -98.3 |
|  |  |  | 94.56 |  | 94.56 |  | 100.56 |
|  | 2 | 100 | -92.3 | 100 | -92.3 | 100 | -92.3 |
|  |  |  | 94.56 |  | 94.56 |  | 94.56 |
| CA_66A-66C | 1 | 100 | -92.8 | 100 | -92.8 | 25 | -98.8 |
|  |  |  | -95.56 |  | -95.56 |  | -101.56 |
|  | 2 | 100 | -92.8 | 100 | -92.8 | 100 | -92.8 |
|  |  |  | -95.56 |  | -95.56 |  | -95.56 |
| Note 1: Requirements with additional relaxation (ΔRIBNC) as per Table 7.3A.0-3. Note 2: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-5. Note 3: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 4: The signal power is specified per port. Note 5: PREFSENS refers to the PCC band’s single carrier reference sensitivity according to table 7.3.5-1. Note 6: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-8. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-8: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous

| CA Configuration | Testpoint ID | PCC |  | SCC1 |  | SCC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | Requirement (dBm) | NRB | Requirement (dBm) | NRB | Requirement (dBm) |
| CA_66A-66A-66A | 1 | 100 | -92.8 | 100 | -92.8 | 100 | -92.8 |
|  |  |  | -95.54 |  | -95.54 |  | -95.54 |
|  | 2 | 100 | -92.8 | 100 | -92.8 | 25 | -95.8 |
|  |  |  | -95.54 |  | -95.54 |  | -98.54 |
| Note 1: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-5. Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 3: The signal power is specified per port. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## 7.4A	Maximum input level for CA

#### 7.4A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part, the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in subclause 7.4.3 for each component carrier while all downlink carriers are active.

For intra-band contiguous carrier aggregation maximum input level is defined as the powers received at the UE antenna port over the Transmission bandwidth configuration of each CC, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel over each component carrier.

The downlink SCC shall be configured at nominal channel spacing to the PCC. For FDD the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.4A.0-1 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels over each component carrier as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.0-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Table 7.4A.0-1.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the maximum input level requirements are defined with the uplink configuration in accordance with Table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in Table 7.4.3-1 and Table 7.4A.0-1 for one component carrier and two component carriers per sub-block, respectively. The throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the specified reference measurement channel as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1). The requirements apply with all downlink carriers active.

Table 7.4A.0-1: Maximum input level for intra-band contiguous CA

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F |
| Power in largestTransmission Bandwidth Configuration CC | dBm |  | -282 | -252 | -252 | -262 | -272 |
|  |  |  | -303 | -273 | -273 | [-28]3 | [-29]3 |
| Power in each other CC | dBm |  | -28 + 10log(NRB,c /NRB,largest BW) 2 | -25 + 10log(NRB,c /NRB,largest BW) 2 | -25 + 10log(NRB,c /NRB,largest BW) 2 | -26 + 10log(NRB,c /NRB,largest BW) 2 | -27 + 10log(NRB,c /NRB,largest BW) 2 |
|  |  |  | -30+ 10log(NRB,c /NRB,largest BW) 3 | -27 + 10log(NRB,c /NRB,largest BW) 3 | -27 + 10log(NRB,c /NRB,largest BW) 3 | [-28] + 10log(NRB,c /NRB,largest BW) 3 | [-29] + 10log(NRB,c /NRB,largest BW) 3 |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in clause 6.2.5A. Note 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 3: Reference measurement channel is Annex A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguous sub-blocks per band and up to four contiguously aggregated carriers per band) and one uplink assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to four component carriers. For these uplink configurations, the UE shall meet the maximum input-level requirements for intra-band non-contiguous carrier aggregation of two downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.4.3. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

The normative references for this requirement are TS 36.101 [2] clauses 7.4.1A and 7.4.1.

< Unchanged sections omitted >

### 7.4A.5	Maximum input level for 3DL CA

#### 7.4A.5.1	Test purpose

Maximum input level for CA test verifies the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

#### 7.4A.5.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

#### 7.4A.5.3	Minimum conformance requirements

The Minimum conformance requirements are defined in clause 7.4A.0.

#### 7.4A.5.4	Test description

##### 7.4A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in tables of 5.4.2A.1 as applicable. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 7.4A.5.4.1-1 to 7.4A.5.4.1-2. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.4A.5.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] | SCC2 |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) | DLalloc |  | UL MOD | DL MOD | ULalloc | DLalloc |  | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for CA_XA-XA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS | All RBs | Max | X | CC1 | N/A | All RBs | - | Y | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB_agg |  | N/A | 64-QAM |  | Highest NRB_agg |  | N/A | 64-QAM |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | - | X | CC2 | N/A | All RBs | Max | X | CC1 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB |  | N/A | 64-QAM |  | Highest NRB_agg |  | N/A | 64-QAM |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS | All RBs | Max | Y | CC1 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB_agg |  | N/A | 64-QAM |  | Highest NRB_agg |  | N/A | 64-QAM |  | Highest NRB |
| 2 | X | Mid | REFSENS | All RBs | - | Y | CC2 | N/A | All RBs | Max | Y | CC1 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB |  | N/A | 64-QAM |  | Highest NRB_agg |  | N/A | 64-QAM |  | Highest NRB_agg |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |  | N/A | 64-QAM |  | Highest NRB |
| 2 | X | Mid | REFSENS | All RBs | - | 66 | CC1 | N/A | All RBs | 50 | 66 | CC2 | N/A | All RBs |
|  | QPSK | 64-QAM |  | Highest NRB |  | N/A | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |
| Test Settings for CA_29A-66A-66A Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | 29 | Mid | N/A | All RBs |
|  | QPSK | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 50 |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 100@0 | All RBs | 134.2 | 41 | CC2 | N/A | All RBs | - | 41 | CC1 | N/A | All RBs |
|  | QPSK | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |
| Test Settings for a CA_41A-41C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 41 | CC3 | 100@0 | All RBs | - | 41 | CC2 | N/A | All RBs | 134.2 | 41 | CC1 | N/A | All RBs |
|  | QPSK | 64-QAM |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 30.2 | 66 | CC2 | N/A | All RBs | - | 66 | CC3 | N/A | All RBs |
|  | QPSK | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |  | N/A | 64-QAM |  | 100 |
| Test Settings for CA_66A-66A-66A Configurations (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 15 | 66 | CC2 | N/A | All RBs | 15 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| 2 | 66 | CC1 | 100@0 | All RBs | 22.5 | 66 | CC2 | N/A | All RBs | 22.5 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 25 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: Intra-band non-contiguous + Inter-band: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Intra-band contiguous: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.55 if the UE supports 2 Rx antennas or Figure A.80 if the UE supports 4 Rx antennas in at least one band within the CA configuration.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1

4.	The UL and DL Reference Measurement channels are set according to Tables 7.4A.5.4.1-1 to 7.4A.5.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.4A.5.4.3.

##### 7.4A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.4A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer to TS 36.321 [13] clauses 5.13 and 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clause 8.3.3.2).

4. 	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables 7.4A.5.4.1-1 to 7.4A.5.4.1-5 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Tables 7.4A.5.4.1-1 to 7.4A.5.4.1-2 as appropriate on PCC. Since the UE has no payload and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value defined in Table 7.4A.5.5-2 or 7.4A.5.5-3 according to the type of CA. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the PCC output power is within +0dB, -3.4 dB of the target level for carrier frequency f ≤ 3.0GHz 
or within +0dB, -4 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

7.	Measure the average throughput for the carrier(s) indicated in table 7.4A.5.4.2-1 for duration sufficient to achieve statistical significance according to Annex G.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.5.4.2-1.

Table 7.4A.5.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with test parameters to select (Note 2) |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.4A.5.5-1 |
|  | 2 |  |  |
| Inter-band | 1 | SCC1 | 7.4A.5.5-2 |
|  |  | SCC2 |  |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
|  | 3 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.4A.5.5-2 |
|  | 2 |  |  |
|  | 3 | SCC1, SCC2 | 7.4A.5.5-1 |
|  | 4 |  |  |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.4A.5.5-2 |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.4A.5.5-2 |
|  |  | SCC1, SCC2 | 7.4A.5.5-1 |
|  | 2 | PCC, SCC1 | 7.4A.5.5-1 |
|  |  | SCC2 | 7.4A.5.5-2 |
| Intra-band non-contiguous | 1 | PCC, SCC1, SCC2 | 7.4A.5.5-2 |
|  | 2 |  |  |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.4A.5.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |  |  |

##### 7.4A.5.4.3	Message contents

Same message contents as in clause 7.4.4.3 for without UL CA.

Same message contents as in clause 7.4A.1.4.3 for UL CA.

#### 7.4A.5.5	Test requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.4A.5.5-1 and 7.4A.5.5-2, as applicable.

Table 7.4A.5.5-1: Maximum input level for 3DL CA (Intra-band contiguous)

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  | For carrier frequency f ≤ 3.0GHz: -28.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -29.0 | For carrier frequency f ≤ 3.0GHz: -25.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -26.0 | For carrier frequency f ≤ 3.0GHz: -25.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -26.0 |  |  |
| Power in each other CC | dBm |  | For carrier frequency f ≤ 3.0GHz: -28.7 + 10log(NRB,c /NRB,largest BW) For carrier frequency 3.0GHz < f ≤ 4.2GHz: -29.0 + 10log(NRB,c /NRB,largest BW) | For carrier frequency f ≤ 3.0GHz: -25.7 + 10log(NRB,c /NRB,largest BW) For carrier frequency 3.0GHz < f ≤ 4.2GHz: -26.0 + 10log(NRB,c /NRB,largest BW) | For carrier frequency f ≤ 3.0GHz: -25.7 + 10log(NRB,c /NRB,largest BW) For carrier frequency 3.0GHz < f ≤ 4.2GHz: -26.0 + 10log(NRB,c /NRB,largest BW) |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5 for one uplink carrier. Note 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

Table 7.4A.5.5-2: Maximum input level for 3DL CA (Inter-band, Intra-band non-contiguous), per CC

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | For carrier frequency f ≤ 3.0GHz: -25.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -26.0 |  |  |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2.5. Note 2: Reference measurement channel is Annex A.3.2 64QAM R=3/4variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

### 7.4A.5_H	Maximum input level for 3DL CA for 256QAM in DL

#### 7.4A.5_H.1	Test purpose

Same test purpose as in clause 7.4A.5.1.

#### 7.4A.5_H.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward that support 256QAM in DL and 3DL CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 256QAM in DL and 3DL CA involving Band 46.

#### 7.4A.5_H.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.1_H.3.

#### 7.4A.5_H.4	Test description

##### 7.4A.5_H.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in tables of 5.4.2A.1 as applicable. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 7.4A.5_H.4.1-1 and 7.4A.5_H.4.1-2. The details of the uplink and downlink reference measurement channels (RMCs) are specified in clauses A.2 and A.3 respectively. The details of the OCNG patterns used are specified in clause A.5. Configurations of PDSCH and PDCCH before measurement are specified in clause C.2.

Table 7.4A.5_H.4.1-1: Test Configuration Table

Initial Conditions

use the content of Table 7.4A.5.4.1-1 raplacing DL MOD by 256QAM

Table 7.4A.5_H.4.1-2: Test Configuration Table with Wgap

Initial Conditions

use the content of Table 7.4A.5.4.1-2 raplacing DL MOD by 256QAM

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.55 if the UE supports 2 Rx antennas or Figure A.80 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to clauses C.0, C.1 and C.3.1, and uplink signals according to clauses H.1 and H.3.1

4.	The UL and DL Reference Measurement channels are set according to Tables 7.4A.5_H.4.1-1 and 7.4A.5_H.4.1-2.

5.	Propagation conditions are set according to clause B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.4A.5_H.4.3.

##### 7.4A.5_H.4.2	Test procedure

1.	Configure SCCs according to clauses C.0, C.1 and clause C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.4A.5_H.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer to TS 36.321 [13] clauses 5.13 and 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clause 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1 for C_RNTI to transmit the DL RMC according to Table 7.4A.5_H.4.1-1 and 7.4A.5_H.4.1-2 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Tables 7.4A.5_H.4.1-1 and 7.4A.5_H.4.1-2 as appropriate on PCC. Since the UE has no payload and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value defined in Table 7.4A.5_H.5-1 or 7.4A.5_H.5-2 according to the type of CA. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the PCC output power is within (+0dB, -3.4dB) of the target level for carrier frequency f ≤ 3.0GHz	or within (+0dB, -4dB) of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for duration sufficient to achieve statistical significance according to clause G.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.5_H.4.2-1.

Table 7.4A.5_H.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with test parameters to select (Note 2) |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.4A.5_H.5-1 |
|  | 2 |  |  |
| Inter-band | 1 | SCC1 | 7.4A.5_H.5-2 |
|  |  | SCC2 |  |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
|  | 3 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.4A.5_H.5-2 |
|  | 2 |  |  |
|  | 3 | SCC1, SCC2 | 7.4A.5_H.5-1 |
|  | 4 |  |  |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.4A.5_H.5-2 |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.4A.5_H.5-2 |
|  |  | SCC1, SCC2 | 7.4A.5_H.5-1 |
|  | 2 | PCC, SCC1 | 7.4A.5_H.5-1 |
|  |  | SCC2 | 7.4A.5_H.5-2 |
| Intra-band non-contiguous | 1 | PCC, SCC1, SCC2 | 7.4A.5_H.5-2 |
|  | 2 |  |  |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.4A.5_H.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |  |  |

##### 7.4A.5_H.4.3	Message contents

Same message contents as defined in clause 7.4A.5.4.3.

#### 7.4A.5_H.5	Test requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in clause A.3.2 with parameters specified in Table 7.4A.5_H.5-1 and Table 7.4A.5_H.5-2.

Table 7.4A.5_H.5-1: Maximum input level for 3DL CA (Intra-band contiguous)

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  | For carrier frequency f ≤ 3.0GHz: -30.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -31.0 | For carrier frequency f ≤ 3.0GHz: -27.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -28.0 | For carrier frequency f ≤ 3.0GHz: -27.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -28.0 |  |  |
| Power in each other CC | dBm |  | For carrier frequency f ≤ 3.0GHz: -30.7 + 10log(NRB,c /NRB,largest BW) For carrier frequency 3.0GHz < f ≤ 4.2GHz: -31.0 + 10log(NRB,c /NRB,largest BW) | For carrier frequency f ≤ 3.0GHz: -27.7 + 10log(NRB,c /NRB,largest BW) For carrier frequency 3.0GHz < f ≤ 4.2GHz: -28.0 + 10log(NRB,c /NRB,largest BW) | For carrier frequency f ≤ 3.0GHz: -27.7 + 10log(NRB,c /NRB,largest BW) For carrier frequency 3.0GHz < f ≤ 4.2GHz: -28.0 + 10log(NRB,c /NRB,largest BW) |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in clause 6.2.5 for one uplink carrier. Note 2: Reference measurement channel is clause A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in clauses A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

Table 7.4A.5_H.5-2: Maximum input level for 3DL CA (Inter-band, Intra-band non-contiguous), per CC

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | For carrier frequency f ≤ 3.0GHz: -27.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -28.0 |  |  |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. Note 2: Reference measurement channel is Annex A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## 7.5A	Adjacent Channel Selectivity (ACS) for CA

#### 7.5A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.5.3 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46, the requirements specified in subclause 7.5.3 are replaced by the requirements in Table 7.5A.0-1a with test parameters in Table 7.5A.0-2a and Table 7.5A.0-3a.

Table 7.5A.0-1a: Adjacent channel selectivity

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | ACS | dB |  |  |  | 33 |  | 27 |

Table 7.5A.0-2a: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA Band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
|  | PInterferer | dBm |  |  |  | REFSENS+45.5dB |  | REFSENS +39.5dB |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075/-15-0.0075 |  | 20+0.0025 / -20-0.0025 |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |  |

Table 7.5A.0-3a: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm |  |  |  | -56.5 |  | -50.5 |
|  | PInterferer | dBm | -25 |  |  |  |  |  |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075/-15-0.0075 |  | 20+0.0025 / -20-0.0025 |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3A.0-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the adjacent channel requirements of subclause 7.5A.0 do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.5A.0-2 or Table 7.5A.0-3 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2. The UE shall fulfil the minimum requirement specified in Table 7.5A.0-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.0-2 and 7.5A.0-3. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.5A.0-2 and 7.5A.0-3.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the adjacent channel selectivity requirements are defined with the uplink configuration in accordance with Table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.5.3 and 7.5A.0 for one component carrier and two component carriers per sub-block, respectively. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5.3-2 and Table 7.5A.0-2 for one component carrier and two component carriers per sub-block, respectively. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5.3-1 and Table 7.5A.0-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer is -25 dBm (Table 7.5.3-3 and Table 7.5A.0-3) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer like for Case 1.

Table 7.5A.0-1: Adjacent channel selectivity

|  |  | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | F |
| ACS | dB | 27 | 24 | 22.2 | 21 | 20 |

Table 7.5A.0-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 25.5 dB | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB | Aggregated power + 19.5 dB | Aggregated power + 18.5 dB |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

Table 7.5A.0-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -50.5 +10log10(NRB,c/ NRB agg) | -47.5 +10log10(NRB,c/NRB agg) | -45.743.9 +10log10(NRB,c/NRB agg) | -44.5 +10log10(NRB,c/NRB agg) 5 | -43.5 +10log10(NRB,c/NRB agg) |
| PInterferer | dBm | -25 |  |  |  |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to four non-contiguously aggregated carriers per band and up to four contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in each band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to four component carriers. For these uplink configurations, the UE shall meet the adjacent channel selectivity requirements for intra-band non-contiguous carrier aggregation of two downlink sub-blocks with RIBNC = 0 dB for all sub-block gaps (Table 7.3A.0-3) for the two non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.5.3. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of downlink carriers are replaced by requirements in Table 7.5A.0-4 with test parameters in Table 7.5A.0-5 and Table 7.5A.0-6. For non-contiguously aggregated component carriers configured in Band 46, the said requirements are applied to each sub-block for in-gap and out-of-gap interferers. For the sub-block with a single component carrier, the requirement is replaced by Table 7.5A.0-1a with test parameters in Table 7.5A.0-1b and Table 7.5A.0-1c. For the sub-block with two or more contiguous component carriers, the requirement is replaced by Table 7.5A.0-4 with test parameters in Table 7.5A.0-5 and Table 7.5A.0-6. All downlink carriers shall be active throughout the tests and the requirements for downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

Table 7.5A.0-4: Adjacent channel selectivity

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | ACS | dB |  | 24 | 22.2 | 21 |  |

Table 7.5A.0-5: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC |  |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
|  | PInterferer | dBm |  | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB | Aggregated power + 19.5 dB |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |
| NOTE 1*: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.3.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

Table 7.5A.0-6: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm |  | -47.5 +10log10(NRB,c/NRB agg) | -45.7 +10log10(NRB,c/NRB agg) | -44.5 +10log10(NRB,c/NRB agg) |  |
|  | PInterferer | dBm | -25 |  |  |  |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.3.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

The normative references for this requirement are TS 36.101 [2] clauses 7.5.1A and 7.5.1.

< Unchanged sections omitted >

### 7.5A.5	Adjacent Channel Selectivity (ACS) for 3DL CA

#### 7.5A.5.1	Test purpose

Adjacent channel selectivity for CA test verifies the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when other e-NodeB transmitters exist in the adjacent channel.

#### 7.5A.5.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

#### 7.5A.5.3	Minimum conformance requirements

The Minimum conformance requirements are defined in clause 7.5A.0.

#### 7.5A.5.4	Test description

##### 7.5A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in tables of 5.4.2A.1 as applicable. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in tables 7.5A.5.4.1-1 to 7.5A.5.4.1-2. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.5A.5.4.1-2: Test Configuration Table Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] | SCC2 |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for CA_XA-XA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS | All RBs | Max | X | CC1 | N/A | All RBs | - | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | - | X | CC2 | N/A | All RBs | Max | X | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS | All RBs | Max | Y | CC1 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS | All RBs | - | Y | CC2 | N/A | All RBs | Max | Y | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |
| Test Settings for CA_29A-66A-66A Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | 29 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 100@0 | All RBs | 134.2 | 41 | CC2 | N/A | All RBs | - | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_41A-41C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 41 | CC3 | 100@0 | All RBs | - | 41 | CC2 | N/A | All RBs | 134.2 | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS | All RBs | - | 66 | CC1 | N/A | All RBs | 50 | 66 | CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 30.2 | 66 | CC2 | N/A | All RBs | - | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66A-66A Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 15 | 66 | CC2 | N/A | All RBs | 15 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: Intra-band non-contiguous + Inter-band: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Intra-band contiguous: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.58 if the UE supports 2 Rx antennas or Figure A.82 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1

4.	The UL and DL Reference Measurement channels are set according to Tables 7.5A.5.4.1-1 to 7.5A.5.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.5A.5.4.3.

##### 7.5A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.5A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer to TS 36.321 [13] clauses 5.13 and 6.1.3.8). Wait for at least 2 seconds (Refer to TS 36.133 [4], clause 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables 7.5A.5.4.1-1 to 7.5A.5.4.1-5 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Tables 7.5A.5.4.1-1 to 7.5A.5.4.1-2 as appropriate on PCC. Since the UE has no payload and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value defined for Case 1 in Table 7.5A.5.4.2-1according to the type of CA. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the PCC output power is within +0dB, -3.4 dB of the target level for carrier frequency f ≤ 3.0GHzor within +0dB, -4 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

7.	Set the Interferer signal level to the value defined for Case 1 in Table 7.5A.5.4.2-1, according to the type of CA, with frequency below the measured carrier(s) according to table 7.5A.5.4.2-1, using a modulated interferer bandwidth as defined in Annex D of the present document. The interferer frequencies are adjusted in analogy with table 7.5A.1.4.2-1.

8.	Measure the average throughput for the carrier(s) indicated in table 7.5A.5.4.2-1 for a duration sufficient to achieve statistical significance according to Annex G.2A.

9.	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to table 7.5A.5.4.2-1.

10.	Set the Downlink signal level for PCC and SCCs to the value defined for Case 2 in Table 7.5A.5.4.2-1, according to the type of CA. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the PCC output power is within +0dB, -3.4 dB of the target level for carrier frequency f ≤ 3.0GHz, 
or within +0dB, -4 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

11.	Set the Interferer signal level to the value defined for Case 2 in Table 7.5A.5.4.2-1, according to the type of CA, with frequency below the measured carrier(s) according to table 7.5A.5.4.2-1, using a modulated interferer bandwidth as defined in Annex D of the present document. The interferer frequencies are adjusted in analogy with table 7.5A.1.4.2-1.

12.	Measure the average throughput for the carrier(s) indicated in table 7.5A.5.4.2-1 for a duration sufficient to achieve statistical significance according to Annex G.2A.

13.	Repeat steps from 10 to 12, using an interfering signal above the measured carrier(s) according to table 7.5A.5.4.2-1.

14.	Repeat steps 6 to 13 for component carriers listed in Table 7.5A.5.4.2-1.

Table 7.5A.5.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with test parameters to select (Note 2) |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.5A.5.5-2 7.5A.5.5-3 |
|  | 2 |  |  |
| Inter-band | 1 | SCC1 | 7.5A.5.5-0e 7.5A.5.5-0f 7.5A.5.5-5 7.5A.5.5-6 |
|  |  | SCC2 |  |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
|  | 3 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.5A.5.5-0e 7.5A.5.5-0f 7.5A.5.5-5 7.5A.5.5-6 |
|  | 2 |  |  |
|  | 3 | SCC1, SCC2 | 7.5A.5.5-0b 7.5A.5.5-0c 7.5A.5.5-2 7.5A.5.5-3 |
|  | 4 |  |  |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.5A.5.5-0e 7.5A.5.5-0f 7.5A.5.5-5 7.5A.5.5-6 |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.5A.5.5-5 7.5A.5.5-6 |
|  |  | SCC1, SCC2 | 7.5A.5.5-0b 7.5A.5.5-0c 7.5A.5.5-2 7.5A.5.5-3 |
|  | 2 | PCC, SCC1 | 7.5A.5.5-0b 7.5A.5.5-0c 7.5A.5.5-2 7.5A.5.5-3 |
|  |  | SCC2 | 7.5A.5.5-5 7.5A.5.5-6 |
| Intra-band non-contiguous | 1 | PCC, SCC1, SCC2 | 7.5A.5.5-5 7.5A.5.5-6 |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.5A.5.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |  |  |

##### 7.5A.5.4.3	Message contents

The message contents are the same as in clause 7.5.4.3 for without UL CA.

The message contents are the same as in clause 7.5A.1.4.3 for UL CA.

#### 7.5A.5.5	Test requirement

The throughput measurement of each carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables below, according to the type of CA.

The test requirements do not apply when the interfering signal overlaps with the channel bandwidth of any downlink CC.

Table 7.5A.5.5-0a: Adjacent channel selectivity for 3DL CA (band 46)

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | ACS | dB |  | 24 | 22.2 |  |  |

Table 7.5A.5.5-0b: Test parameters for Adjacent channel selectivity for 3DL CA (band 46), Case 1

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC |  |  | REFSENS + 14 dB | REFSENS + 14 dB |  |  |
|  | PInterferer | dBm |  | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB |  |  |
|  | BWInterferer | MHz |  | 20 | 20 |  |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |  |
| NOTE 1*: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.3.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

Table 7.5A.5.5-0c: Test parameters for Adjacent channel selectivity for 3DL CA (band46), Case 2

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm |  | -47.5 +10log10(NRB,c/NRB agg) | -45.7 +10log10(NRB,c/NRB agg) |  |  |
|  | PInterferer | dBm | -25 |  |  |  |  |
|  | BWInterferer | MHz |  | 20 | 20 |  |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.3.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

Table 7.5A.5.5-0d: Adjacent channel selectivity for 3DL CA (band 46)

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | ACS | dB |  |  |  | 33 |  | 27 |

Table 7.5A.5.5-0e: Test parameters for Adjacent channel selectivity for 3DL CA 
(band 46), Case 1

| E-UTRA Band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
|  | PInterferer | dBm |  |  |  | REFSENS +45.5dB |  | REFSENS +39.5dB |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075 /-15-0.0075 |  | 20+0.0025 /-20-0.0025 |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.3.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |  |

Table 7.5A.5.5-0f: Test parameters for Adjacent channel selectivity for 3DL CA 
(band 46), Case 2

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm |  |  |  | -56.5 |  | -50.5 |
|  | PInterferer | dBm | -25 |  |  |  |  |  |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075 /-15-0.0075 |  | 20+0.0025 /-20-0.0025 |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.3.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |  |

Table 7.5A.5.5-1: Adjacent channel selectivity for 3DL CA (Intra-band contiguous)

|  |  | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | F |
| ACS | dB | 27 | 24 | 22.2 |  |  |

Table 7.5A.5.5-2: Test parameters for Adjacent channel selectivity, Case 1, 3DL CA (Intra-band contiguous)

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |  |
| PInterferer | dBm | Aggregated power + 22.5 dB | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB |  |  |
| BWInterferer | MHz | 5 | 5 | 5 |  |  |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5 for one uplink carrier. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

Table 7.5A.5.5-3: Test parameters for Adjacent channel selectivity, Case 2, 3DL CA (Intra-band contiguous)

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -50.5 +10log10(NRB,c/ NRB agg) | -47.5 +10log10(NRB,c/NRB agg) | -43.9 +10log10(NRB,c/NRB agg) |  |  |
| PInterferer | dBm | -25 |  |  |  |  |
| BWInterferer | MHz | 5 | 5 | 5 |  |  |
| FInterferer (offset) | MHz | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5 for one uplink carrier. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

Table 7.5A.5.5-4: Adjacent channel selectivity for 3DL CA (Inter-band, Intra-band non-contiguous), per CC

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| ACS | dB | 33.0 | 33.0 | 33.0 | 33.0 | 30 | 27 |

Table 7.5A.5.5-5: Test parameters for Adjacent channel selectivity for 3DL CA (Inter-band, Intra-band non-contiguous), Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration for each CC | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
| PInterferer | dBm | REFSENS for SCC +45.5dB | REFSENS for SCC +45.5dB |  | REFSENS for SCC +45.5dB | REFSENS for SCC +45.5dB | REFSENS for SCC +42.5dB | REFSENS for SCC +39.5dB |
| BWInterferer | MHz | 1.4 | 3 |  | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 | 3+0.0075 / -3-0.0075 |  | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L.at the minimum uplink configuration specified in Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |  |  |

Table 7.5A.5.5-6: Test parameters for Adjacent channel selectivity for 3DL CA (Inter-band, Intra-band non-contiguous), Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration for each CC | dBm | -56.5 | -56.5 | -56.5 | -56.5 | -53.5 | -50.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |
| BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 | 3+0.0075 / -3-0.0075 | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.6.1A	In-band blocking for CA

#### 7.6.1A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the in-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.6.1 for each component carrier while all downlink carriers are active. For the UE which supports inter band CA configuration in Table 7.3.3-1A, PInterferer power defined in Table 7.6.1.3-2 is increased by the amount given by ΔRIB,c in Table 7.3.3-1A. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The requirements for the component carrier configured in the operating band without uplink operation are specified in Table 7.6.1A.0-0, Table 7.6.1A.0-0a and Table 7.6.1A.0-0b.

Table 7.6.1A.0-0: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 29, 32, 67, 69 | FInterferer | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| Note 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. Note 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 Note 3: FInterferer range values for unwanted modulated interfering signal are interferer centre frequencies. |  |  |  |  |

Table 7.6.1A.0-0a: In band blocking parameters for additional operating bands for carrier aggregation

| E-UTRA band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 (NOTE 3) | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 9 |
|  | BWInterferer | MHz |  |  |  |  |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  |  |  | 30+0.0125 |
|  | FIoffset, case 2 | MHz |  |  |  |  |  | 50+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) |  |  |  |  |  |  |  |  |

Table 7.6.1A.0-0b: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -50 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 46 | FInterferer | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz or 60 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer centre frequencies. |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3A.0-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the in-band blocking requirements of subclause 7.6.1A.0 do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.1A.0-1 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.1A.0-1 and Tables 7.6.1A.0-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1A.0-1 and 7.6.1A.0-2. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.6.1A.0-1 and 7.6.1A.0-2.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the in-band blocking requirements are defined with the uplink configuration in accordance with Table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclause 7.6.1.3 and in this subclause for single component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

Table 7.6.1A.0-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 | 16 |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 | 12.5 | 12.5 |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A Note 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 7.6.1A.0-2: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_40D, CA_40E, CA_41C, CA_41D, CA_41E, CA_41F, CA_42C, CA_42D, CA_42E, CA_42F, CA_43C, CA_48C, CA_48D, CA_48E, CA_48F, CA_66B, CA_66C, CA_70C | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| Note 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band Note 2: For each carrier frequency the requirement is valid for two frequencies: a) the carrier frequency -Foffset - FIoffset, case 1 and b) the carrier frequency +Foffset + FIoffset, case 1 Note 3: Foffset is the frequency offset from the centre frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. Note 4: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguously sub-blocks per band and up to four contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in the band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to four component carriers. For these uplink configurations, the UE shall meet the in-band blocking requirements for intra-band non-contiguous carrier aggregation of two downlink sub-blocks with RIBNC = 0 dB for all sub-block gaps (Table 7.3A.0-3) for the two non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.6.1.3. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of uplink operation.

For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of downlink carriers are replaced by requirements in Table 7.6.1A.0-3 and 7.6.1A.0-4.

Table 7.6.1A.0-3: In band blocking parameters

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  | 12 | 13.8 | 15 |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FIoffset, case 1 | MHz |  | 30 | 30 | 30 |  |
|  | FIoffset, case 2 | MHz |  | 50 | 50 | 50 |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |

Table 7.6.1A.0-4: In-band blocking

| E-UTRA Band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -50 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| 46 | FInterferer (Range) | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the centre frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 7.6.1.1A.

< Unchanged sections omitted >

#### 7.6.1A.5	In-band blocking for 3DL CA

##### 7.6.1A.5.1	Test Purpose

Same test purpose as in clause 7.6.1A.1.

##### 7.6.1A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA or 3DL with Intra-band contiguous and Inter-band CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous and Inter-band CA or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL with FDD-TDD CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.7.6.1A.5.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.1A.0.

##### 7.6.1A.5.4	Test Description

###### 7.6.1A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.1A.5.4.1-1 through table 7.6.1A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.6.1A.5.4.1-2: Test Configuration Table Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] | SCC2 |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for CA_XA-XA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS | All RBs | Max | X | CC1 | N/A | All RBs | - | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS | All RBs | - | X | CC2 | N/A | All RBs | Max | X | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS | All RBs | Max | Y | CC1 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS | All RBs | - | Y | CC2 | N/A | All RBs | Max | Y | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |
| Test Settings for CA_29A-66A-66A Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | 29 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 100@0 | All RBs | 134.2 | 41 | CC2 | N/A | All RBs | - | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_41A-41C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 41 | CC3 | 100@0 | All RBs | - | 41 | CC2 | N/A | All RBs | 134.2 | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 50 | 66 | CC2 | N/A | All RBs | - | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS | All RBs | - | 66 | CC1 | N/A | All RBs | 50 | 66 | CC2 | N/A | All RBs |
|  | QPSK | QPSK |  | Highest NRB |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 30.2 | 66 | CC2 | N/A | All RBs | - | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66A-66A Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs | 15 | 66 | CC2 | N/A | All RBs | 15 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: Intra-band non-contiguous + Inter-band: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Intra-band contiguous: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.58 as appropriate if the UE supports 2 Rx antennas or Figure A.82 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6.1A.5.4.1-1 through Table 7.6.1A.5.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.6.1A.5.4.3.

###### 7.6.1A.5.4.2	Test Procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.6.1A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.6.1A.5.4.1-1 through Table 7.6.1A.5.4.1-5 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.6.1A.5.4.1-1 through Table 7.6.1A.5.4.1-2 on PCC. Since the UE has no payload data to send, the UE sends uplink MAC padding bits on the UL RMC.

6	Set the parameters of the signal generator for an interfering signal below the aggregated component carriers in Case 1 according to Table 7.6.1A.5.4.2-1.

7	Set the downlink signal level according to the Table 7.6.1A.5.4.2-1. Send uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in table 7.6.1A.5.4.2-1 for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the throughput measurement.

8	Measure the average throughput for the carrier(s) indicated in Table 7.6.1A.5.4.2-1for duration sufficient to achieve statistical significance according to Annex G.2A.

9	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to Table 7.6.1A.5.4.2-1 in Case 1 at step 6.

10	Repeat steps from 6 to 9, using interfering signals in Case 2 at step 6 and 9. The ranges of case 2 are covered in steps equal to the interferer bandwidth. The test frequencies are chosen in analogy to table 7.6.1A.1.4.2-1.

11.	Repeat steps 1 to 10 for all component carriers listed in Table 7.6.1A.5.4.2-1.

Table 7.6.1A.5.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with test parameters to select (Note 2) |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.6.1A.5.5-2 7.6.1A.5.5-3 |
|  | 2 |  |  |
| Inter-band | 1 | SCC1 | 7.6.1A.5.5-1 7.6.1A.5.5-1a 7.6.1A.5.5-1b 7.6.1A.5.5-4 7.6.1A.5.5-5 |
|  |  | SCC2 |  |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
|  | 3 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.6.1A.5.5-1 7.6.1A.5.5-1a 7.6.1A.5.5-1b 7.6.1A.5.5-4 7.6.1A.5.5-5 |
|  | 2 |  |  |
|  | 3 | SCC1, SCC2 | 7.6.1A.5.5-2 7.6.1A.5.5-3 7.6.1A.5.5-3a 7.6.1A.5.5-3b |
|  | 4 |  |  |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.6.1A.5.5-1 7.6.1A.5.5-1a 7.6.1A.5.5-1b 7.6.1A.5.5-4 7.6.1A.5.5-5 |
|  | 2 | SCC1, | 7.6.1A.5.5-1a 7.6.1A.5.5-1b 7.6.1A.5.5-4 7.6.1A.5.5-5 |
|  |  | SCC2 |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.6.1A.5.5-4 7.6.1A.5.5-5 |
|  |  | SCC1, SCC2 | 7.6.1A.5.5-2 7.6.1A.5.5-3 7.6.1A.5.5-3a 7.6.1A.5.5-3b |
|  | 2 | SCC2 | 7.6.1A.5.5-1a 7.6.1A.5.5-1b 7.6.1A.5.5-4 7.6.1A.5.5-5 |
|  |  | PCC, SCC1 | 7.6.1A.5.5-2 7.6.1A.5.5-3 |
| Intra-band non-contiguous | 1 | PCC, SCC1, SCC2 | 7.6.1A.5.5-4 7.6.1A.5.5-5 |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.6.1A.5.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |  |  |

< Unchanged sections omitted >

### 7.6.2A	Out-of-band blocking for CA

#### 7.6.2A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The throughput in the downlink measured shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.3-1 and 7.6.2A.0-0. For E-UTRA CA configurations including an operating band without uplink operation (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the uplink active in the band(s) capable of UL operation. The UE shall meet these requirements for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the out-of-band blocking requirements specified above shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,c for each serving cell c.

For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3A.0-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the out-of-band blocking requirements of subclause 7.6.2A.0 do not apply.

Table 7.6.2A.0-0: Out-of-band blocking for inter-band carrier aggregation

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pw | dBm | Table 7.6.2.3-1 for all component carriers |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 or 15 < f – FDL_High(j) < 60 | -85 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(1) – 85 or FDL_High(j) + 85 ≤ f ≤ FDL_Low(j+1) – 85 with j < X or FDL_High(X) + 85 ≤ f ≤ 12750 |
| Note 1: FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination (X ≤ 5 for the present version of this specification). Note 2: For FDL_Low(j+1) – FDL_High(j) < 145 MHz and FInterferer in FDL_High(j) < f < FDL_Low(j+1), FInterferer can be in both Range 1 and Range 2. Then the lower of the PInterferer applies. Note 3: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier j. Note 4: RIB,c according to Table 7.3.3-1A applies when serving cell c is measured. Note 5 For inter-band CA combinations containing Bands 42 ,43 or B48, the interferer with respect to Band 42 , Band 43 or Band 48 shall have power level (PInterferer) for Range 3 modified to -20 + RIB,c dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 6: For inter-band CA combinations containing Bands 7 and 38 simultaneously, for FInterferer Bands 7 and 38 are considered as one single band as follows: FDL_Low = 2570 MHz and FDL_High = 2690 MHz. For Range 2, the following applies for FDL_Low: -95 < f – FDL_Low ≤ -60 or 60 ≤ f – FDL_High < 85. For Range 3 the following applies 1 ≤ f ≤ FDL_Low -95 or FDL_High + 85 ≤ f ≤ 12750. NOTE 7: For CA_20A-28A, CA_1A-3A-7A-20A-28A, CA_1A-3A-20A-28A, CA_1A-3A-3A-20A-28A, CA_1A-7A-20A-28A, CA_1A-20A-28A, CA_3A-7A-20A-28A, CA_3A-20A-28A and CA_7A-20A-28A the FDL_low(1) is given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high(2) by Band 20 (Table 5.2A-2). |  |  |  |  |

Table 7.6.2A.0-0a: out-of-band blocking for inter-band carrier aggregation with band 46 and with one active uplink

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pwanted | dBm | Table 7.6.2.1-1 for all component carriers |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c (NOTE 5) |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 with j ≤ K or 15 < f – FDL_High(j) < 60 with j ≤ K | -85 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(j) – 85 or FDL_High(j) + 85 ≤ f ≤ 12750 |
| NOTE 1: FDL_Low(j) and FDL_High(j), j = 1,…,K,…N, denote the respective lower and upper frequency limits of the (non-overlapping) operating bands of the CA configuration numbered in increasing order of frequency, with N the number of bands in the band combination and K the number of bands with FDL_High ≤ 3800 MHz. NOTE 2: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier j ≤ K. NOTE 3: For FDL_Low(j) – 60 MHz ≤ f ≤ FDL_High(j) + 60 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier K < j ≤ N. NOTE 4: RIB,c according to Table 7.3.1-1A applies when serving cell c is measured. NOTE 5: The power level (PInterferer) for Range 3 is modified to -20 dBm for FInterferer > 4400 MHz except for band combinations with Band 42 or Band 43 for which PInterferer for Range 3 is modified to -20 dBm for FInterferer > 2800 MHz. |  |  |  |  |

For Table 7.6.2A.0-0 in frequency ranges 1, 2 and 3, up to  exceptions per downlink are allowed for spurious response frequencies for one active uplink when measured using a step size of 1 MHz.

For Table 7.6.2A.0-0 in frequency ranges 1, 2 and 3, up to 2∙ exceptions per downlink are allowed for spurious response frequencies for two active uplinks when measured using a step size of 1 MHz. For these exceptions the requirements in clause 7.7A.0 apply.

For intra-band contiguous carrier aggregations the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.2A.0-1 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.2A.0-1 and Tables 7.6.2A.0-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2A.0-1 and 7.6.2A.0-2. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.6.2A.0-1 and 7.6.2A.0-2.

For Table 7.6.2A.0-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7 Spurious response are applicable.

Table 7.6.2A.0-1: Out-of-band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 | 9 | 9 |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |

Table 7.6.2A.0-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_40C, CA_40D, CA_40E, CA_41C, CA_41D, CA_41E, CA_41F, CA_42C1, CA_42D1, CA_42E1, CA_42F1, CA_43C1, CA_48C1, CA_48D1, CA_48E1, CA_48F1, CA_66B, CA_66C, CA_66D, CA_70C | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| Note 1: For CA_42C, the power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the out-of-band blocking requirements are defined with the uplink configuration in accordance with table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.6.2.3 and 7.6.2A.0 for one component carrier and two component carriers per sub-block, respectively. The requirements apply with all downlink carriers active.

For Table 7.6.2.3-2 in frequency range 1, 2 and 3, up to exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7A.4.3 spurious response are applicable.

For Table 7.6.2.3-2 in frequency range 4, up to exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7A.4.3 spurious response are applicable.

For intra-band non-contiguous carrier aggregation with two uplink carriers and two downlink carriers, the out-of-band blocking requirements are defined with the uplink configuration of the PCC and SCC being in accordance with Table 7.3A.0-4 and powers of both carriers set to PCMAX_L,c – 7 dBm. The UE shall meet the requirements specified in subclause 7.6.2A.0 for each component carrier while both downlink carriers are active.

For Table 7.6.2.3-2 in frequency range 1, 2 and 3, up to 2∙ exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies for two active uplinks in the same operating band when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7 spurious response are applicable.

For Table 7.6.2.3-2 in frequency range 4, up to 2∙ exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies for two active uplinks in the same operating band when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7 spurious response are applicable.

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguously sub-blocks per band and up to four contiguously aggregated carriers per band) and the uplink assigned to one E-UTRA band, the requirement is defined with the uplink active a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to four component carriers. For the two non-contiguous component carriers within the same band, Pwanted in Table 7.6.2A.0-0 is set using RIBNC = 0 dB for all sub-block gaps (Table 7.3A.0-3) while a band supporting contiguously aggregated carriers the out-of-band blocking parameters in Table 7.6.2.3-1 are replaced by those specified in Table 7.6.2A.0-1. For each downlink the UE shall meet the out-of-band blocking requirements applicable for inter-band carrier aggregation with one component carrier per operating band but with up to four component carriers assigned to the same band with the following exception. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

The normative reference for this requirement is TS 36.101 [2] clause 7.6.2.1A.

< Unchanged sections omitted >

#### 7.6.2A.5	Out-of-band blocking for 3DL CA

##### 7.6.2A.5.1	Test Purpose

Same test purpose as in clause 7.6.2A.3.1.

##### 7.6.2A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE and forward that support 3DL with CA configurations in Table 7.1-2a.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

##### 7.6.2A.5.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.6.2A.0.

##### 7.6.2A.5.4	Test Description

###### 7.6.2A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.2A.5.4.1-1 through table 7.6.2A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.6.2A.5.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  | Wgap [MHz] |  | SCC1 |  |  |  |  |  |  | Wgap [MHz] |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) |  | DL alloc |  |  |  | UL MOD |  | DL MOD |  | UL alloc | DL alloc |  |  |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for CA_XA-XA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS |  | All RBs |  | Max |  | X |  | CC1 |  | N/A | All RBs |  | - |  | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS |  | All RBs |  | - |  | X |  | CC2 |  | N/A | All RBs |  | Max |  | X | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS |  | All RBs |  | Max |  | Y |  | CC1 |  | N/A | All RBs |  | - |  | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS |  | All RBs |  | - |  | Y |  | CC2 |  | N/A | All RBs |  | Max |  | Y | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB_agg |
| Test Settings for CA_29A-66A-66A Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 50 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | 29 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 50 |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 100@0 |  | All RBs |  | 134.2 |  | 41 |  | CC2 |  | N/A | All RBs |  | - |  | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_41A-41C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 41 | CC3 | 100@0 |  | All RBs |  | - |  | 41 |  | CC2 |  | N/A | All RBs |  | 134.2 |  | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 50 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS |  | All RBs |  | - |  | 66 |  | CC1 |  | N/A | All RBs |  | 50 |  | 66 | CC2 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 30.2 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66A-66A Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs |  | 15 |  | 66 |  | CC2 |  | N/A |  |  | All RBs |  | 15 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  | N/A | QPSK |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: Intra-band non-contiguous + Inter-band: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Intra-band contiguous: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.58 as appropriate if the UE supports 2 Rx antennas or Figure A.82 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6.2A.5.4.1-1 through Table 7.6.2A.5.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.6.2A.5.4.3.

###### 7.6.2A.5.4.2	Test Procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.6.2A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.6.2A.5.4.1-1 through Table 7.6.2A.5.4.1-2 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.6.2A.5.4.1-1 through Table 7.6.2A.5.4.1-2 on PCC. Since the UE has no payload data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the parameters of the CW signal generator for an interfering signal below the lowest CA Band according to the table indicated in the column "Table with interferer parameters to select" of Table 7.6.2A.5.4.2-1. The frequency step size is 1MHz. Use the highest RIB,c among CA bands for Pinterferer calculation.

7.	Set the downlink signal level according to the table indicated in the column "Table with transmission parameters to select" of Table 7.6.2A.5.4.2-1. Send uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in table 7.6.2A.5.5-1 for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the throughput measurement.

8.	Measure the average throughput for all the carrier(s) indicated in the Table 7.6.2A.7.4.2-1 simultaneously for duration sufficient to achieve statistical significance according to Annex G.2. Note that the statistical decision for each component carrier should be postponed until the statistical decisions for all component carriers are obtained simultaneously according to table G.2.4-1.

9.	Record the frequencies for which the throughput doesn't meet the requirements and for each frequency, the carriers for which the throughput was not met.

10.	Repeat steps 6 to 8 for each recorded frequency-carrier pair, with exception of pairs for which RIB,c is the same as RIB used in Step 6. In Step 6 use only recorded frequencies for interferer placement and use RIB,c relevant to recorded carrier for Pinterferer calculation. Remove the frequency-carrier pairs that meet the throughput requirements from the record.

11.	If the CA Configuration includes multiple bands, repeat steps from 6 to 10 using an interfering signal in each gap between the CA bands, according to the table indicated in the column "Table with interferer parameters to select" of Table 7.6.2A.5.4.2-1.

12.	Repeat steps from 6 to 10, using an interfering signal above the highest CA Band according to the table indicated in the column "Table with interferer parameters to select" of Table 7.6.2A.5.4.2-1.

13.	Repeat steps 1 to 12 for each row of "Throughput measured on" column listed in the Table 7.6.2A.5.4.2-1.

Table 7.6.2A.5.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with transmission parameters to select | Table with interferer parameters to select (Note 2) |
| --- | --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.6.2A.5.5-1 | 7.6.2A.5.5-2 |
|  | 2 |  |  |  |
| Inter-band | 1 | SCC1, SCC2 | 7.6.2A.5.5-3 | 7.6.2A.5.5-4 |
|  | 2 | SCC1, SCC2 |  |  |
|  | 3 | SCC1, SCC2 |  |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.6.2A.5.5-3 | 7.6.2A.5.5-4 (Note 4) |
|  | 2 | SCC1, SCC2 | 7.6.2A.5.5-1 | 7.6.2A.5.5-2 (Note 5) |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.6.2A.5.5-3 | 7.6.2A.5.5-4 (Note 6) |
|  | 2 | SCC1, | 7.6.2A.5.5-3 | 7.6.2A.5.5-5 (Note 5) |
|  |  | SCC2 |  |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.6.2A.5.5-3 | 7.6.2A.5.5-5 (Note 4) |
|  |  | SCC1, SCC2 | 7.6.2A.5.5-1 | 7.6.2A.5.5-2 (Note 6) |
|  | 2 | SCC2 | 7.6.2A.5.5-3 | 7.6.2A.5.5-5 (Note 4) |
|  |  | PCC, SCC1 | 7.6.2A.5.5-1 | 7.6.2A.5.5-2 (Note 6) |
| Intra-band non-contiguous | 1 | PCC, SCC1, SCC2 | 7.6.2A.5.5-3 | 7.6.2A.5.5-5 |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.6.2A.5.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. Note 4: When interferer position is within range 1 or 2 frequency of other CC block, Pinterfer is selected from table 7.6.2A.5.5-2. When interferer position is closer than 15 MHz from other CC block interferer is OFF. Note 5: When interferer position is within range 1 or 2 frequency of other CC block, Pinterfer is selected from table 7.6.2A.5.5-4. When interferer position is closer than 15 MHz from other CC block interferer is OFF. Note 6: When interferer position is within range 1 or 2 frequency of other CC block, Pinterfer is selected from table 7.6.2A.5.5-5. When interferer position is closer than 15 MHz from other CC block interferer is OFF. |  |  |  |  |

< Unchanged sections omitted >

### 7.6.3A	Narrow band blocking for CA

#### 7.6.3A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the narrow-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.6.3 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3A.0-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the narrow-band blocking requirements of subclause 7.6.3A.0 do not apply. For E-UTRA CA configurations with a component carrier assigned in Band 46, narrow-band blocking requirements do not apply in the presence of a narrow-band interferer in Band 46.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.3A.0-1 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6.3A.0-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3A.0-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Table 7.6.3A.0-1.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two downlink sub-blocks, the narrow band blocking requirements are defined with the uplink configuration in accordance with Table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.6.3.3 and 7.6.3A.0 for one component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

Table 7.6.3A.0-1: Narrow-band blocking

| Parameter | Unit | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 16 | 164 | 16 | 16 | 16 |
| Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 |
| Fuw (offset for f = 15 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |  |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 3: The Fuw (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. Note 4: The requirement is applied for the band combinations whose component carriers’ BW≥5 MHz. |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguously sub-blocks per band and up to four contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to four component carriers. For these uplink configurations, the UE shall meet the narrow-band blocking requirements for intra-band non-contiguous carrier aggregation of two downlink sub-blocks with RIBNC = 0 dB for all sub-block gaps (Table 7.3A.0-3) for the two non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.6.3. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

For E-UTRA CA configurations with a component carriers assigned in Band 46, narrow-band blocking requirements do not apply in the presence of a narrow-band interferer in Band 46.

The normative reference for this requirement is TS 36.101 [2] clause 7.6.3.1A.

< Unchanged sections omitted >

#### 7.6.3A.5	Narrow band blocking for 3DL CA

##### 7.6.3A.5.1	Test Purpose

Same test purpose as 7.6.3A.1.

##### 7.6.3A.5.2	Test Applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA or 3DL with Intra-band contiguous and  Inter-band CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous and  Inter-band CA or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL with FDD-TDD CA.

##### 7.6.3A.5.3	Minimum Conformance Requirements

Same as in clause 7.6.3A.1.

##### 7.6.3A.5.4	Test Description

###### 7.6.3A.5.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6.3A.5.4.1-1 through table 7.6.3A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.6.3A.5.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  | Wgap [MHz] |  | SCC1 |  |  |  |  |  |  | Wgap [MHz] |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) |  | DL alloc |  |  |  | UL MOD |  | DL MOD |  | UL alloc | DL alloc |  |  |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for CA_XA-XA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS |  | All RBs |  | Max |  | X |  | CC1 |  | N/A | All RBs |  | - |  | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS |  | All RBs |  | - |  | X |  | CC2 |  | N/A | All RBs |  | Max |  | X | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS |  | All RBs |  | Max |  | Y |  | CC1 |  | N/A | All RBs |  | - |  | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS |  | All RBs |  | - |  | Y |  | CC2 |  | N/A | All RBs |  | Max |  | Y | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB_agg |
| Test Settings for CA_29A-66A-66A Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 50 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | 29 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 50 |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 100@0 |  | All RBs |  | 134.2 |  | 41 |  | CC2 |  | N/A | All RBs |  | - |  | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_41A-41C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 41 | CC3 | 100@0 |  | All RBs |  | - |  | 41 |  | CC2 |  | N/A | All RBs |  | 134.2 |  | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 50 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS |  | All RBs |  | - |  | 66 |  | CC1 |  | N/A | All RBs |  | 50 |  | 66 | CC2 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 30.2 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66A-66A Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs |  | 15 |  | 66 |  | CC2 |  | N/A |  |  | All RBs |  | 15 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  | N/A | QPSK |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: Intra-band non-contiguous + Inter-band: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Intra-band contiguous: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.58 as appropriate if the UE supports 2 Rx antennas or Figure A.82 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6.3A.5.4.1-1 through Table 7.6.3A.5.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.6.3A.5.4.3.

###### 7.6.3A.5.4.2	Test Procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.6.3A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.6.3A.5.4.1-1 through Table 7.6.3A.5.4.1-5 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.6.3A.5.4.1-1 through Table 7.6.3A.5.4.1-2 on PCC. Since the UE has no payload data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the parameters of the CW signal generator for an interfering signal below the aggregated component carriers according to Table 7.6.3A.5.4.2-1.

7.	Set the downlink signal level according to the table 7.6.3A.5.4.2-1. Send uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in table 7.6.3A.5.4.2-1 for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the throughput measurement.

8.	Measure the average throughput for the carrier(s) indicated in Table 7.6.3A.5.4.2-1 for duration sufficient to achieve statistical significance according to Annex G.2A.

9.	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to Table 7.6.3A.5.4.2-1 at step 6.

10.	Repeat steps 1 to 9 for component carriers listed in Table 7.6.3A.5.4.2-1.

Table 7.6.3A.5.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with test parameters to select (Note 2) |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.6.3A.5.5-1 |
|  | 2 |  |  |
| Inter-band | 1 | SCC1 | 7.6.3A.5.5-2 |
|  |  | SCC2 |  |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
|  | 3 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.6.3A.5.5-2 |
|  | 2 |  |  |
|  | 3 | SCC1, SCC2 | 7.6.3A.5.5-1 |
|  | 4 |  |  |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.6.3A.5.5-2 |
|  | 2 | SCC1, | 7.6.3A.5.5-2 |
|  |  | SCC2 |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.6.3A.5.5-2 |
|  |  | SCC1, SCC2 | 7.6.3A.5.5-1 |
|  | 2 | SCC2 | 7.6.3A.5.5-2 |
|  |  | PCC, SCC1 | 7.6.3A.5.5-1 |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.6.3A.5.5-2 |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.6.3A.5.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |  |  |

< Unchanged sections omitted >

### 7.7A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the spurious response requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The throughput measured in each downlink with Finterferer in Table 7.6.2A.0-0 and Table 7.6.2A.0-0a at spurious response frequencies shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.7.3-1 and 7.7.3-2. The UE shall meet these requirements for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the spurious response requirements applicable specified above shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,c for each serving cell c.

For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3A.0-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the spurious response requirements of subclause 7.7A.0 do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall configure closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.7A.0-1 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.7A.0-1 and 7.7A.0-2. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.7A.0-1 and 7.7A.0-2.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the spurious response requirements are defined with the uplink configuration in accordance with Table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.7.3 and 7.7A.0 for one component carrier and two component carriers per sub-block, respectively. The requirements apply with all downlink carriers active.

For intra-band non-contiguous carrier aggregation with two uplink carriers and two downlink carriers, the spurious response requirements applicable specified above shall be met with the transmitter powers for the uplinks set to PCMAX_L,c – 7 dBm.

Table 7.7A.0-1: Spurious response parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 | 9 | 9 |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.7A.0-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguously sub-blocks per band and up to four contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation of up to four component carriers. For the two non-contiguous component carriers within the same band, Pwanted in Table 7.6.2A.0-0 is set using RIBNC = 0 dB for all sub-block gaps (Table 7.3A.0-3) while a band supporting contiguously aggregated carriers the out-of-band blocking parameters in Table 7.7.3-1 are replaced by those specified in Table 7.7A.0-1. For each downlink the UE shall meet the spurious-response requirements applicable for inter-band carrier aggregation with one component carrier per operating band but with up to three component carriers assigned to the same band. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

The normative reference for this requirement is TS 36.101 [2] clause 7.7.1A.

< Unchanged sections omitted >

#### 7.8.1A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.8.1.3 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46, the requirements specified in subclause 7.8.1.3 are replaced by the requirements in Table 7.8.1A.0-0. For E-UTRA CA configurations listed in Table 7.3A.0-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the wideband intermodulation requirements of subclause 7.8.1A.0 do not apply.

Table 7.8.1A.0-0: Wide band intermodulation

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 6 |  |  |  | 9 |  |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  |  | 20 |  |  |  | 20 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  |  | -BW/2-30 / +BW/2+30 |  |  |  | -BW/2 – 30 / +BW/2 + 30 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table Table 7.3.3-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.8.1A.0-1 with the uplink configuration set according to Table 7.3A.0-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.3-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.8.1A.0-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.1A.0-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.8.1A.0-1.

Table 7.8.1A.0-1: Wide band intermodulation

| Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 | 16 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
| BWInterferer 2 | MHz | 5 | 5 | 5 | 5 | 5 |
| FInterferer 1 (Offset) | MHz | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| Note 1: The transmitter shall be set to 4dB below PCMAX_L or PCMAX_L_CA as defined in subclause 6.2.5A. Note 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1. Note 4: The interfering modulated signal is 5MHz E-UTRA signal as described in Annex D for channel bandwidth 5MHz Note 5: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the wide band intermodulation requirements are defined with the uplink configuration in accordance with Table 7.3A.0-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclause 7.8.1.3 and in this subclause for one component carrier and two  component carriers per sub-block, respectively. The requirements apply for out-of-gap interferers while all downlink carriers are active.

For combinations of intra-band and inter-band carrier aggregation with up to five downlink carriers (up to two non-contiguously sub-blocks per band and up to four contiguously aggregated carriers per band) and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3A.0-3 when the uplink is active in the band supporting two non-contiguous component carriers, Table 7.3A.0-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.3-2 when the uplink is active in a band supporting one carrier per band. For these uplink configurations, the UE shall meet the wide-band intermodulation requirements for intra-band non-contiguous carrier aggregation of two downlink sub-blocks with RIBNC = 0 dB for all sub-block gaps (Table 7.3A.0-3) for the two non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.8.1. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of two downlink carriers are replaced by requirements in Table 7.8.1A.0-2. For non-contiguously aggregated component carriers configured in Band 46, the said requirements are applied to each sub-block for out-of-gap interferers. For the sub-block with a single component carrier, the requirement is replaced by Table 7.8.1A.0-0. For the sub-block with two or more contiguous component carriers, the requirement is replaced by Table 7.8.1A.0-1. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

Table 7.8.1A.0-2: Wide band intermodulation

| E-UTRA Band | Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  | 12 | 13.8 | 15 |  |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
|  | BWInterferer 2 | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer 1 (Offset) | MHz |  | –Foffset-30 / + Foffset+30 | –Foffset-30 / + Foffset+30 | –Foffset-30 / + Foffset+30 |  |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1. NOTE 4: The interfering modulated signal is 20 MHz E-UTRA signal as described in Annex D interference setting 2; NOTE 5: The Finterferer 1 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the modulated interferer. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 7.8.1A and TS 36.101 [2] Annexes A and D.

< Unchanged sections omitted >

#### 7.8.1A.5	Wideband intermodulation for 3DL CA

##### 7.8.1A.5.1	Test purpose

Same as 7.8.1A.1

##### 7.8.1A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support 3DL with Intra-band contiguous CA or 3DL with Inter-band CA, or 3DL with Inter-band and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 11 and forward that support 3DL with Intra-band non-contiguous CA and Inter-band CA, or 3DL with Intra-band non-contiguous and Intra-band contiguous CA.

This test case also applies to all types of E-UTRA UE release 12 and forward that support 3DL  with FDD-TDD CA.

This test case also applies to all types of E-UTRA UE release 13 and forward that support 3DL CA involving Band 46.

##### 7.8.1A.5.3	Minimum conformance requirements

Same as 7.8.1A.1.

##### 7.8.1A.5.4	Test description

7.8.1A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.8.1A.5.4.1-1 through table 7.8.1A.5.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

< Unchanged tables omitted >

Table 7.8.1A.5.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  |  | Wgap [MHz] |  | SCC1 |  |  |  |  |  |  | Wgap [MHz] |  | SCC2 |  |  |  |
|  | Band | Range | NRB |  |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) |  | DL alloc |  |  |  | UL MOD |  | DL MOD |  | UL alloc | DL alloc |  |  |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for CA_XA-XA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS |  | All RBs |  | Max |  | X |  | CC1 |  | N/A | All RBs |  | - |  | Y | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | Y | Mid | REFSENS |  | All RBs |  | - |  | X |  | CC2 |  | N/A | All RBs |  | Max |  | X | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for CA_XA-YA-YA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | CC2 | REFSENS |  | All RBs |  | Max |  | Y |  | CC1 |  | N/A | All RBs |  | - |  | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS |  | All RBs |  | - |  | Y |  | CC2 |  | N/A | All RBs |  | Max |  | Y | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | Highest NRB_agg |  |  |  | N/A | QPSK |  | Highest NRB_agg |
| Test Settings for CA_29A-66A-66A Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 50 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | 29 | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 50 |
| Test Settings for a CA_41C-41A Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 41 | CC3 | 100@0 |  | All RBs |  | 134.2 |  | 41 |  | CC2 |  | N/A | All RBs |  | - |  | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_41A-41C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 41 | CC3 | 100@0 |  | All RBs |  | - |  | 41 |  | CC2 |  | N/A | All RBs |  | 134.2 |  | 41 | CC1 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Group Test Settings for CA_66A-66A-XA or CA_XA-66A-66A Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 50 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | X | Mid | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | Highest NRB |
| 2 | X | Mid | REFSENS |  | All RBs |  | - |  | 66 |  | CC1 |  | N/A | All RBs |  | 50 |  | 66 | CC2 | N/A | All RBs |
|  | QPSK | QPSK |  |  | Highest NRB |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66C Configuration (Intra-band non-contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 |  | All RBs |  | 30.2 |  | 66 |  | CC2 |  | N/A | All RBs |  | - |  | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  |  | 100 |  |  |  | N/A |  | QPSK |  |  | 100 |  |  |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_66A-66A-66A Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | 100@0 | All RBs |  | 15 |  | 66 |  | CC2 |  | N/A |  |  | All RBs |  | 15 | 66 | CC3 | N/A | All RBs |
|  | QPSK | QPSK |  | 100 |  |  |  | N/A |  | QPSK |  |  |  |  | 100 |  |  | N/A | QPSK |  | 100 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band non-contiguous + Inter-band: Use CA Configuration – specific test points if present in the table. Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use Default Test Settings test points. Note 3: Intra-band non-contiguous + Intra-band contiguous: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_2A-2A-5A,X=2,Y=5 Note 6: Intra-band non-contiguous + Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41C-41A,X=41. Note 7: Intra-band non-contiguous + Inter-band: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 8: Intra-band non-contiguous + Intra-band contiguous: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band non-contiguous + Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_CC1 = NRB_CC2 for testing. If no such combination is supported, choose Combination with maximum NRB_CC1 for testing. Note 10: The orders and numbering of SCCs in this table does not imply any order in test implementation of SCCs. Note 11: ID/test points that result in no refsens requirement shall be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.58 as appropriate if the UE supports 2 Rx antennas or Figure A.82 if the UE supports 4 Rx antennas in the band.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.8.1A.5.4.1-1 through Table 7.8.1A.5.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.8.1A.5.4.3.

7.8.1A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2. The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.8.1A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.8.1A.5.4.1-1 through Table 7.8.1A.5.4.1-2 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.8.1A.5.4.1-1 through Table 7.8.1A.5.4.1-2 as appropriate on PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the value as defined in Table 77.8.1A.5.4.2-1. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

7.	Set the Interfering signal levels to the values as defined in Table 7.8.1A.5.4.2-1 and frequency below the  measured carrier(s) according to table 7.8.1A.5.4.2-1, using a modulated interferer bandwidth as defined in Annex D of the present document.

8.	Measure the average throughput for the carrier(s) indicated in table 7.8.1A.5.4.2-1 for a duration sufficient to achieve statistical significance according to Annex G.2A.

9.	Repeat steps from 6 to 8, using an interfering signal above the measured carrier(s) according to table 7.8.1A.5.4.2-1.

10.	Repeat steps 6 to 9 for component carriers listed in table 7.8.1A.5.4.2-1.

Table 7.8.1A.5.4.2-1: Test repetition and measurement configuration

| CA configuration | ID (Note 1) | Throughput measured on (Note 3) | Table with test parameters to select (Note 2) |
| --- | --- | --- | --- |
| Intra-band contiguous | 1 | PCC, SCC1, SCC2 | 7.8.1A.5.5-2 |
|  | 2 |  |  |
| Inter-band | 1 | SCC1 | 7.8.1A.5.5-0 7.8.1A.5.5-1 |
|  |  | SCC2 |  |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
|  | 3 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band contiguous + Inter-band | 1 | SCC2 | 7.8.1A.5.5-0 7.8.1A.5.5-1 |
|  | 2 |  |  |
|  | 3 | SCC1, SCC2 | 7.8.1A.5.5-2 7.8.1A.5.5-3 |
|  | 4 |  |  |
| Intra-band non-contiguous + Inter-band | 1 | SCC2 | 7.8.1A.5.5-0 7.8.1A.5.5-1 |
|  | 2 | SCC1 |  |
|  |  | SCC2 |  |
| Intra-band non-contiguous + Intra-band contiguous | 1 | PCC | 7.8.1A.5.5-1 |
|  |  | SCC1, SCC2 | 7.8.1A.5.5-2 7.8.1A.5.5-3 |
|  | 2 | SCC2 | 7.8.1A.5.5-1 |
|  |  | PCC, SCC1 | 7.8.1A.5.5-2 7.8.1A.5.5-3 |
| Intra-band non-contiguous | 1 | PCC, SCC1, SCC2 | 7.8.1A.5.5-1 |
| Note 1: Test point ID for the particular CA configuration as defined in section 7.8.1A.5.4.1. Note 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. Note 3: Where there are multiple rows for a single Test point ID, the test is repeated for each row. Note 4: Skip the parts of the test procedure configuring in-gap interferer positions for a non-contiguous component carrier |  |  |  |

< End of changes >