3GPP TSG-RAN5 Meeting #92-e	R5-216120

# Electronic Meeting, 16th – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1116 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.5B.2.2.3 ACLR for intra-band non-contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The notation of E-UTRA uplink RB allocation was not in line with that in other test cases, and the corresponding explanation was missing. - The initial procedure was wrongly described as LTE anchor agnostic. - In test procedure, there was a lack of the step to test maximum power reduction. And the procedure to enable DFT-s-OFDM modulation was missing. - The whole subclause of Message Contents was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Unify the notation of E-UTRA uplink RB allocation to align with other test cases. - Correct initial procedures to enable transmission on E-UTRA carrier. - Add the step to measure maximum power reduction and the procedure to enable DFT-s-OFDM modulation. - Supplement the subclause of Message Contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would not be performed properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-215212r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5B.2.2.3	Adjacent channel leakage ratio for intra-band non-contiguous EN-DC

Editor's note:	The following aspects are either missing or not yet determined:

-	Testing of none simultaneous LTE-NR transmission is FFS.

-	The reference to 6.2B.2.2.5 MPR in Step 4 is incomplete until test points for MPR and ACLR are aligned.

6.5B.2.2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5B.2.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.3.3	Minimum conformance requirements

For intra-band non-contiguous EN-DC, the EN-DC Adjacent Channel Leakage power Ratio (EN-DCACLR) is the ratio of the sum of the filtered mean powers centred on the assigned E-UTRA and NR sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than an E-UTRA or NR sub-block bandwidth, no EN-DCACLR requirement is set for the corresponding sub-block for the gap. The assigned EN-DC sub-block power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in TS 36.101 [5] for the E-UTRA sub-block, and TS 38.101-1 [2] for the NR sub-block. If the measured adjacent channel power is greater than –50dBm then the EN-DCACLR shall be higher than the value specified in for E-UTRAACLR and NRACLR.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.3.

Adjacent Channel Leakage power Ratio must be measured for both NR and E-UTRA and therefore LTE anchor agnostic approach is not applied.

6.5B.2.2.3.4	Test description

6.5B.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in TS 38.508-1 [6] clause 4.3.1.4.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in TS 38.508-1 [6] clause 4.3.1.4.3 and are shown in Table 6.5B.2.2.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.2.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1.4.3 for cases with single E-UTRA carrier |  |  | Low with maxWgap (NR low – E-UTRA high); High with maxWgap (E-UTRA low – NR high) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 |  |  | Lowest for E-UTRA, lowest for NR; Highest for E-UTRA, highest for NR |  |  |  |
| Test SCS as specified as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 |  |  | Lowest, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | NR Uplink Configuration |  | E-UTRA Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) | Modulation | RB allocation (NOTE 3) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | QPSK | Outer_1RB_Right |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | QPSK | Outer_1RB_Left |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full | QPSK | Outer_Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | QPSK | Outer_1RB_Right |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | QPSK | Outer_1RB_Left |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer_Full | QPSK | Outer_Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer_Full | 16QAM | Outer_Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer_Full | 16QAM | Outer_Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer_Full | 16QAM | Outer_Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | QPSK | Outer_1RB_Right |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right | QPSK | Outer_1RB_Left |
| 18 | Default |  | CP-OFDM QPSK | Outer_Full | QPSK | Outer_Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 21 | Default |  | CP-OFDM 16 QAM | Outer_Full | 16QAM | Outer_Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 24 | Default |  | CP-OFDM 64 QAM | Outer_Full | 16QAM | Outer_Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | 16QAM | Outer_1RB_Right |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | 16QAM | Outer_1RB_Left |
| 27 | Default |  | CP-OFDM 256 QAM | Outer_Full | 16QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2. The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.2.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.5B.2.2.3.4.2	Test Procedure

1.	For NR carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.3.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	For E-UTRA carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously power control “up” commands to the UE for NR and E-UTRA until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements in clause 6.2B.2.2.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

5.	Measure the rectangular filtered mean power for the assigned NR channel.

6.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR channel, respectively. Skip the measurement within the Wgap in case Wgap < NR sub-block bandwidth.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NRACLR, respectively.

8.	Measure the rectangular filtered mean power for E-UTRA channel.

9.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel, respectively. Skip the measurement within the Wgap in case Wgap < E-UTRA sub-block bandwidth.

10.	Calculate the ratios of the power between the values measured in step 7 over step 8 for lower and upper E-UTRAACLR, respectively.

NOTE 1: When switching to DFT-s-OFDM waveform, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.2.3.4.3	Message Content

Message contents are according to TS 36.508-1 [11] clause 4.6 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.5B.2.2.3.4.3-1: RRCConnectionReconfiguration: nr-Config-r15.

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.3.4.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 6.5B.2.2.3.4.3-3: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

6.5B.2.2.3.5	Test requirement

For NR ACLR, if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR shall be higher than the limits in Table 6.5.2.4.1.5-2 defined in clause 6.5.2.4.1.5 in TS 38.521-1 [8] for the NR carrier.

For E-UTRA ACLR, if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRA ACLR shall be higher than the limits in Table 6.6.2.3.5.1-1 defined in clause 6.6.2.3.5 in TS 36.521-1 [10] for the E-UTRA carrier.

<End of Changes>