3GPP TSG-RAN WG5 Meeting #93-e	R5-216812

Electronic Meeting, 8-19th November 2021

Source:	Qualcomm Incorporated

Title:	Applicability for 5G SA UE not supporting VoiceoverNR

# Document for:	Agreement

Agenda Item:	6.3.1.20

## 1.	Introduction

As per TS 38.306, an R-15 device can indicate to NW that it doesn't support VoNR (VoiceoverNR) using below capability.

| voiceOverNR Indicates whether the UE supports IMS voice over NR. It is mandated to the UE if the UE is capable of IMS voice over NR. Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

For such a device, NW shall not trigger VoNR call, if the UE supports IMS, then NW may redirect it to perform an EPS FB. Such a UE is still capable of supporting IMS registration over NR.

TS 34.229-2, IMS Applicability Specification, doesn't allow such a device to only pick IMS Registration Test cases and skip voice\MTSI test cases.

## 2. Discussion

A Rel-15 UE that does not support VoNR shall still be able to execute several IMS test cases from TS34.299-5, like the IMS registration test case below. Based on the applicability definition, such a UE can pick up the tests case.

| 6.2 | Initial Registration Failures / 5GS | Rel-15 | C01 | UE supports IMS security and NR |
| --- | --- | --- | --- | --- |

However, a Rel-15 UE that does not support VoNR won't be able to selectively skip voice tests cases (for ex 7.1 and 7.10) due to the current definition of applicability. A Rel-15 UE that does not support VoNR may support MTSI speech in EUTRA i.e VoLTE and below condition will be incorrectly met.

| 7.1 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 | C02 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and initiating a session and preconditions |
| --- | --- | --- | --- | --- |
| 7.10 | MTSI MT Voice call without preconditions and without SDP offer in MT INVITE / 5GS | Rel-15 | C28 | UE supports IMS security and NR and MTSI and MTSI speech and AMR-WB and EVS and preconditions and disabling preconditions |

Further, the current overall applicability assume UE complies to NG.114, which mandates the UE shall support voice. In such a scenario, a Rel-15 UE which doesn't support VoNR, will have to entirely skip executing TS 34.229-5, which will result in loss of test coverage. The test specification and applicability shall be chosen to cater for broad device type as well ecosystem requirements.

One option would be to introduce PICS like pc_voiceoverNR  and modify the applicability as below

| 7.1 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 | C02 | UE supports IMS security and NR and MTSI and MTSI speech and pc_voiceoverNR and AMR-WB and EVS and initiating a session and preconditions |
| --- | --- | --- | --- | --- |

## 3.	Proposal/Way Forward

1. 	Introduce a PICS in TS 38.508-2 for R-15 VoiceoverNR capability - pc_voiceoverNR, use that PICS in applicability statement of TS 34.229-2 for 5GS test cases, wherever applicable.

## 4.	References

[1]		3GPP TS 34.229-2 : "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification" - current document.

[2]		3GPP TS 34.229-5 : "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)"

[3]		3GPP TS 38.306 : "User Equipment (UE) radio access capabilities"