3GPP TSG-RAN5 Meeting #93-e	R5-217423

# Online, 8th Nov – 19th Nov 2021

Title:							Proposal on the increase of untested CCs for FR2 DL CA EIS

Source:			Anritsu

Agenda Item:			5.3.1.17

Document for:			Endorsement

# 1.	Introduction

For testing FR2 intra-band DL CA Reference Sensitivity, it was agreed to keep the power level of PCC and remaining SCCs above the CC under test [1]. This paper propose the concrete value for the increase of the untested CCs.

# 2.	Discussion

The motivation of keeping PCC and remaining SCCs above the CC under test is to avoid the concern that EIS level cannot be measured accuracy. If the power level of PCC and the CC under test is the same, when the PCC reaches its EIS level before the CC under test reaches its EIS level, UE may lose its connection to TE and the test stops as a result of the undefined scenario as mentioned in [1]. However, it increases test time due to not measure all CCs at the same time. In order to optimize the test time, we propose to adopt 0 dB for the lower limit of the increase of the untested CCs if UE keeps its connection to TE. For now, in the actual testing, UE does not lose its connection to TE even if all CCs have the same level. Of course, it is possible that the REFSENS of PCC is much higher than other CCs in the future, so we propose 5 dB for the upper limit of the increase of the untested CCs.

Observation 1 : UE does not lose its connection to TE even if all CCs have the same level in the actual testing.

Proposal 1 : Adopt 0 dB for the lower limit of the increase of the untested CCs if UE keeps its connection to TE.

Proposal 2 : Adopt 5 dB for the upper limit of the increase of the untested CCs.

# 4.	Conclusion

Observation 1 : UE does not lose its connection to TE even if all CCs have the same level in the actual testing.

Proposal 1 : Adopt 0 dB for the lower limit of the increase of the untested CCs if UE keeps its connection to TE.

Proposal 2 : Adopt 5 dB for the upper limit of the increase of the untested CCs.

# 5.	References

[1] R5-215819, “Discussion on FR2 DL CA test procedure”, Rohde & Schwarz, RAN5#92-e