3GPP TSG-RAN5 Meeting #93-e	R5-217632

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0229 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Clause 5.5.8.3 MCPTT User Profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The user profile was missing parameters for remote group initiation |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the parameter RemoteGroupSelectionURIList - Added general parameters for future test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT clients will not be configured correctly for various features and could then falsely fail test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <START OF CHANGES>

#### 5.5.8.3	MCPTT User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 8.3, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 5.2.

The structure of the configuration document is based on the XML Schema in clause 8.3.2.3 of TS 24.484 [14] and XML "ruleset" schema according to IETF RFC 4745 [103]. To distinguish the schemas the prefix "cp" ("common policy") is used for the ruleset.

Table 5.5.8.3-1: MCPTT User Profile Defaults

| Derivation Path: TS 24.484 [14] clause 8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| mcptt-user-profile |  |  |  |  |  |  |  |  |  |
| XUI-URI attribute |  | px_MCPTT_User_XUI_URI |  |  |  |  |  |  |  |
| user-profile-index attribute |  | "49" |  | value arbitrarily selected |  |  |  |  |  |
| Status |  | true |  | MCPTT user profile is enabled |  |  |  |  |  |
| ProfileName |  | px_MCPTT_User_A_Profile_Name |  | Profile name for the MCPTT user |  | TS 24.483 [13] clause 5.2.7B |  |  |  |
| Common |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  | Index for the particular MCPTT user profile |  |  |  |  |  |
| MCPTTUserID |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_A |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 5.2.7 |  |  |  |
| UserAlias |  |  |  | Alphanumeric aliases of MCPTT user |  | TS 24.483 [13] clause 5.2.8 |  |  |  |
| alias-entry |  | px_MCPTT_User_A_Alias |  |  |  |  |  |  |  |
| ParticipantType |  | px_MCPTT_User_A_ParticipantType |  | Participant type of the MCPTT user |  | TS 24.483 [13] clause 5.2.10 |  |  |  |
| MissionCriticalOrganization |  | px_MCX_DomainName_Organization_A |  | Indicates the organization an MCPTT user belongs to |  | TS 24.483 [13] clause 5.2.11 |  |  |  |
| PrivateCall |  |  |  |  |  |  |  |  |  |
| PrivateCallList |  |  |  |  |  |  |  |  |  |
| PrivateCallURI[1] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_B |  | MCPTT user(s) who can be called in a MCPTT private call |  | TS 24.483 [13] clause 5.2.17 |  |  |  |
| display-name |  | "User B Name" |  | a human readable name for this User |  | TS 24.483 [13] clause 5.2.18 |  |  |  |
| PrivateCallURI[2] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "1" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_C |  | MCPTT user(s) who can be called in a MCPTT private call |  | TS 24.483 [13] clause 5.2.17 |  |  |  |
| display-name |  | "User C Name" |  | a human readable name for this User |  | TS 24.483 [13] clause 5.2.18 |  |  |  |
| PrivateCallProSeUser[1] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | "1234" |  | Discovery group ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19 |  |  |  |
| User-Info-ID |  | "5555" |  | Prose user Info ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19A |  |  |  |
| PrivateCallProSeUser[2] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "1" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | "1234" |  | Discovery group ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19 |  |  |  |
| User-Info-ID |  | "6666" |  | Prose user Info ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19A |  |  |  |
| EmergencyCall |  |  |  |  |  |  |  |  |  |
| MCPTTPrivateRecipient |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UsePreConfigured" |  | Indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. |  | TS 24.483 [13] clause 5.2.29F |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_B |  | The MCPTT private recipient for an MCPTT emergency private call |  | TS 24.483 [13] clause 5.2.29B |  |  |  |
| display-name |  | "User B Name" |  | a human readable name for this User |  | TS 24.483 [13] clause 5.2.29E |  |  |  |
| ProSeUserID-entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | "1234" |  | Discovery group ID in the ProSe discovery procedures |  | TS 24.483 [13] clause 5.2.29C |  |  |  |
| User-Info-ID |  | "5555" |  | ProSe user Info ID in the ProSe discovery procedures |  | TS 24.483 [13] clause 5.2.29D |  |  |  |
| MCPTT-group-call |  |  |  |  |  |  |  |  |  |
| MaxSimultaneousCallsN6 |  | "3" |  | Indicates the maximum number of simultaneously received MCPTT group calls |  | TS 24.483 [13] clause 5.2.31 |  |  |  |
| EmergencyCall |  |  |  |  |  |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  | Use currently selected MCPTT group for an on-network MCPTT emergency group call |  | TS 24.483 [13] clause 5.2.34D |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | The group used upon certain criteria on initiation of an MCPTT emergency group call |  | TS 24.483 [13] clause 5.2.34B |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | The display name for group used for emergency |  | TS 24.483 [13] clause 5.2.34C |  |  |  |
| ImminentPerilCall |  |  |  |  |  |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  | Use currently selected MCPTT group for an on-network MCPTT imminent peril group call |  | TS 24.483 [13] clause 5.2.39D |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | the group used on initiation of an MCPTT imminent peril group call. |  | TS 24.483 [13] clause 5.2.39B |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | display name for group used for the imminent peril call |  | TS 24.483 [13] clause 5.2.39C |  |  |  |
| EmergencyAlert |  |  |  |  |  |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  | Use currently selected MCPTT group for emergency alert |  | TS 24.483 [13] clause 5.2.43E |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | Indicates the MCPTT group used upon certain criteria on initiation of an MCPTT emergency alert. |  | TS 24.483 [13] clause 5.2.43B |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | Optional; name of emergency alert group |  | TS 24.483 [13] clause 5.2.43D |  |  |  |
| Priority |  | "10" |  | Indicates the priority of the MCPTT group calls, 0-255 |  | TS 24.483 [13] clause 5.2.43F |  |  |  |
| OffNetwork |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| MCPTTGroupInfo |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | Indicates an off-network MCPTT group for use by an MCPTT user |  | TS 24.483 [13] clause 5.2.53 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | The display name corresponding to off-network group id |  | TS 24.483 [13] clause 5.2.53A |  |  |  |
| User-Info-ID |  | "5555" |  | ProSe user info ID |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.58 |  |  |  |
| OnNetwork |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| MCPTTGroupInfo |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  | Group 1 the MCPTT user is allowed to affiliate to |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  | TS 24.483 [13] clause 5.2.48B4 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | The display name for the group |  | TS 24.483 [13] clause 5.2.48B5 |  |  |  |
| MaxAffiliationsN2 |  | 20 |  |  |  |  |  |  |  |
| MaxSimultaneousTransmissionsN7 |  | 20 |  |  |  |  |  |  |  |
| ImplicitAffiliations |  |  |  | Group 1 the MCPTT user is implicitly affiliated to |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | indicates a MCPTT group ID to which the MCPTT user is implicitly affiliated to |  | TS 24.483 [13] clause 5.2.48C4 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | display name for implicitly affiliated group |  | TS 24.483 [13] clause 5.2.48C5 |  |  |  |
| PrivateEmergencyAlert |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UsePreConfigured" |  | Indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. |  | TS 24.483 [13] clause 5.2.48O |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_B |  | Indicates the default MCPTT user ID to be used upon certain criteria on initiation of an MCPTT private emergency alert for on-network |  | TS 24.483 [13] clause 5.2.48M |  |  |  |
| display-name |  | px_MCPTT_User_A_Alias |  | The display name corresponding to private emergency call id |  | TS 24.483 [13] clause 5.2.48N |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| RemoteGroupSelectionURIList |  |  |  |  |  | TS 24.483 [13] clause 5.2.48U2 |  |  |  |
| entry |  | px_MCPTT_ID_User_A |  |  |  | TS 24.483 [13] clause 5.2.48U4 |  |  |  |
| entry |  | px_MCPTT_ID_User_B |  |  |  | TS 24.483 [13] clause 5.2.48U4 |  |  |  |
| entry |  | px_MCPTT_ID_User_C |  |  |  | TS 24.483 [13] clause 5.2.48U4 |  |  |  |
| FunctionalAliasList |  |  |  |  |  | TS 24.483 [13] clause 5.2.48W6 |  |  |  |
| entry |  | px_MCPTT_ID_FA_A |  |  |  |  |  |  |  |
| cp:ruleset |  |  |  |  |  |  |  |  |  |
| cp:rule |  |  |  |  |  |  |  |  |  |
| cp:id attribute |  | "rule1" |  |  |  |  |  |  |  |
| cp:actions |  |  |  |  |  |  |  |  |  |
| allow-create-delete-user-alias |  | "true" |  | Indicates authorisation to create and delete aliases of other MCPTT users |  | TS 24.483 [13] clause 5.2.9 |  |  |  |
| allow-private-call |  | "true" |  | Indicates the authorisation to make a MCPTT private call |  | TS 24.483 [13] clause 5.2.13 |  |  |  |
| allow-private-call-to-any-user |  | "true" |  | indicates the authorisation to make a MCPTT private call to any MCPTT user |  | TS 24.483 [13] clause 5.2.14 |  |  |  |
| allow-manual-commencement |  | "true" |  | Indicates the authorisation to make a MCPTT private call with manual commencement |  | TS 24.483 [13] clause 5.2.20 |  |  |  |
| allow-automatic-commencement |  | "true" |  | Indicates the authorisation to make a MCPTT private call with automatic commencement |  | TS 24.483 [13] clause 5.2.21 |  |  |  |
| allow-force-auto-answer |  | "true" |  | Indicates the authorisation of MCPTT user to force automatic answer for a MCPTT private call |  | TS 24.483 [13] clause 5.2.22 |  |  |  |
| allow-failure-restriction |  | "false" |  | Indicates the authorisation to restrict the provision of a notification of call failure reason for a MCPTT private call |  | TS 24.483 [13] clause 5.2.23 |  |  |  |
| allow-private-call-media-protection |  | "true" |  | Indicates authorisation to protect confidentiality and integrity of media for MCPTT private calls |  | TS 24.483 [13] clause 5.2.24 |  |  |  |
| allow-private-call-floor-control-protection |  | "true" |  | Indicates authorisation to protect confidentiality and integrity of floor control signalling for MCPTT private calls. |  | TS 24.483 [13] clause 5.2.25 |  |  |  |
| allow-emergency-private-call |  | "true" |  | Indicates the authorisation to make an MCPTT emergency private call. |  | TS 24.483 [13] clause 5.2.27 |  |  |  |
| allow-cancel-private-emergency-call |  | "true" |  | Indicates the authorisation to cancel emergency priority in an MCPTT emergency private call by an authorised MCPTT user |  | TS 24.483 [13] clause 5.2.28 |  |  |  |
| allow-emergency-group-call |  | "true" |  | Indicates the authorisation to make an MCPTT emergency group call functionality enabled for MCPTT user |  | TS 24.483 [13] clause 5.2.33 |  |  |  |
| allow-cancel-group-emergency |  | "true" |  | Indicates the authorisation to cancel an in progress MCPTT emergency call associated with a group. |  | TS 24.483 [13] clause 5.2.35 |  |  |  |
| allow-imminent-peril-call |  | "true" |  | Indicates the authorisation to make an Imminent Peril group call |  | TS 24.483 [13] clause 5.2.37 |  |  |  |
| allow-cancel-imminent-peril |  | "true" |  | Indicates the authorisation for in-progress MCPTT imminent peril cancelation |  | TS 24.483 [13] clause 5.2.38 |  |  |  |
| allow-activate-emergency-alert |  | "true" |  | Indicates the authorisation to activate an MCPTT emergency alert |  | TS 24.483 [13] clause 5.2.41 |  |  |  |
| allow-cancel-emergency-alert |  | "true" |  | Indicates the authorisation to cancel an MCPTT emergency alert |  | TS 24.483 [13] clause 5.2.42 |  |  |  |
| allow-create-group-broadcast-group |  | "true" |  | Indicates the authorisation to create a group-broadcast group. |  | TS 24.483 [13] clause 5.2.46 |  |  |  |
| allow-create-user-broadcast-group |  | "true" |  | Indicates the authorisation to create a user-broadcast group |  | TS 24.483 [13] clause 5.2.48 |  |  |  |
| allow-offnetwork |  | "true" |  | Indicates the authorisation for off-network services |  | TS 24.483 [13] clause 5.2.50 |  |  |  |
| allow-listen-both-overriding-and-overridden |  | "false" |  | Indicates whether the MCPTT user is allowed to listen both overriding and override |  | TS 24.483 [13] clause 5.2.54 |  |  |  |
| allow-transmit-during-override |  | "false" |  | Indicates whether the MCPTT user is allowed to transmit in case of override (overriding and/or overridden) |  | TS 24.483 [13] clause 5.2.55 |  |  |  |
| allow-off-network-group-call-change-to-emergency |  | "true" |  | Indicates the authorisation for a participant to change an off-network group call in-progress to an off-network MCPTT emergency group call |  | TS 24.483 [13] clause 5.2.56 |  |  |  |
| allow-imminent-peril-change |  | "true" |  | Indicates the authorisation for a participant to change an off-network group call in-progress to an off-network MCPTT imminent peril group call |  | TS 24.483 [13] clause 5.2.57 |  |  |  |
| allow-regroup |  | "true" |  | Indicates whether the MCPTT user is authorised to perform dynamic regrouping operations |  | TS 24.483 [13] clause 5.2.48D |  |  |  |
| allow-presence-status |  | "true" |  | Indicates the presence status on the network of this MCPTT user is available |  | TS 24.483 [13] clause 5.2.48E |  |  |  |
| allow-request-presence |  | "true" |  | Indicates whether the MCPTT user is authorised to obtain whether a particular MCPTT User is present on the network |  | TS 24.483 [13] clause 5.2.48F |  |  |  |
| allow-private-call-participation |  | "true" |  | Indicates whether the MCPTT user is allowed to participate in MCPTT private calls that they are invited to |  | TS 24.483 [13] clause 5.2.48G |  |  |  |
| allow-override-of-transmission |  | "true" |  | Indicates whether the MCPTT user is authorised to override transmission in a MCPTT private call |  | TS 24.483 [13] clause 5.2.48H |  |  |  |
| allow-manual-off-network-switch |  | "true" |  | Indicates whether the MCPTT user is authorised to manually switch to off-network operation while in on-network operation |  | TS 24.483 [13] clause 5.2.48I |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| allow-request-private-call-call-back |  | "true" |  | Indicates whether the MCPTT user is allowed to request a private call call-back |  | TS 24.483 [13] clause 5.2.48P |  |  |  |
| allow-cancel-private-call-call-back |  | "true" |  | Indicates whether the MCPTT user is allowed to cancel an outstanding private call call-back request |  | TS 24.483 [13] clause 5.2.48Q |  |  |  |
| allow-request-remote-initiated-ambient-listening |  | "true" |  | Indicates whether the MCPTT user is allowed to request a remote initiated ambient listening call |  | TS 24.483 [13] clause 5.2.48R |  |  |  |
| allow-request-locally-initiated-ambient -listening |  | "true" |  | Indicates whether the MCPTT user is allowed to request a locally initiated ambient listening call |  | TS 24.483 [13] clause 5.2.48S |  |  |  |
| allow-request-first-to-answer-call |  | "true" |  | Indicates whether the MCPTT user is authorised to request a first to answer call |  | TS 24.483 [13] clause 5.2.48T |  |  |  |
| allow-request-remote-init-private-call |  | "true" |  | Indicates whether the MCPTT user is authorised to request remotely initiated private calls |  | TS 24.483 [13] clause 5.2.48W1 |  |  |  |
| allow-request-remote-init-group-call |  | "true" |  | Indicates whether the MCPTT user is authorised to request a remotely initiated group call |  | TS 24.483 [13] clause 5.2.48W2 |  |  |  |
| allow-query-functional-alias-other-user |  | "true" |  | Indicates whether the MCPTT user is authorised to query the functional alias(es) activated by another MCPTT user |  | TS 24.483 [13] clause 5.2.48W8 |  |  |  |
| allow-takeover-functional-alias-other-user |  | "true" |  | Indicates whether he MCPTT user is authorised to take over the functional alias(es) previously activated by another MCPTT user |  | TS 24.483 [13] clause 5.2.48W9 |  |  |  |
| allow-location-info-when-talking |  | "false" |  | When set to "true" the MCPTT user is authorised to send its location information when it is requesting the floor. When set to "false" the MCPTT user is not authorised to send its location information when it is requesting the floor. |  | TS 24.483 [13] clause 5.2.48W10 |  |  |  |

<END OF CHANGES>