3GPP TSG-RAN5 Meeting #93-e	R5-217870

# Electronic Meeting, 8th November 2021 – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1361 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for new eMTC4 test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Two new test cases: 7.1.3.23 Enhanced Coverage / DL Fexible starting PRB and 7.1.4.42 Enhanced Coverage / UL Fexible starting PRB, Was added according to the eMTC4 Work Plan. The corresponding applicability statements needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability statements for the new test cases, 7.1.3.23 and 7.1.4.42 were added. New conditions for the test cases were also added. Two new UE capabilities was added in Table A.4.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability for the new eMTC4 TCs will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, Annex A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-217507 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

# 4	Recommended Test Case Applicability

The applicability of each individual test is identified in Table 4-1. This is just a recommendation based on the purpose for which the test case was written.

The applicability of every test is formally expressed by the use of Boolean expression that are based on parameters (ICS) included in annex A of the present document.

Additional information related to the Test Case (TC), e.g. affecting its dynamic behaviour or its execution may be provided as well.

When a test case is to be executed against a category M1 UE and with IMS enabled, it is assumed that the UE is compliant to GSMA profile NG.108 [55].

The columns in Table 4-1 have the following meaning:

Clause

The clause column indicates the clause number in TS 36.523-1 [19] that contains the test body.

Title

The title column describes the name of the test and contains the clause title of the clause in TS 36.523-1 [19] that contains the test body.

Release

The release column indicates the earliest release from which the test case is applicable. In some specific cases it may indicate the release(s) for which the TC is only applicable.

Note:	Some exceptions to this interpretation may be indicated in Notes in column 'Number of TC Executions' e.g. see Note 3 Table 4-1.

Applicability - Condition

The following notations are used for the applicability column:

R	recommended - the test case is recommended

O	optional – the test case is optional

N/A	not applicable - in the given context, the test case is not recommended.

Ci	conditional - the test is recommended ("R") or not ("N/A") depending on the support of other items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

NOTE 1:	The conditions are defined in Table 4-1a.

Applicability - Comments

This column contains a verbal description of the condition.

Additional Information - Specific ICS

This column contains the mnemonics of ICS(s) affecting the dynamic behaviour of the TC.

NOTE 1A:	ICS items specified in 3GPP TS 34.123-2 [8] and 3GPP TS 34.229-2 [45] can be referred, to avoid redundant definitions.

NOTE 1B:	The ICS items pc_eFDD and pc_eFDD, as well as pc_NB_FDD and pc_NB_TDD, specified in the present document (Table A.4.1-1) are used to identify that a test case can be run in FDD or/and TDD branch. When none of them is provided it is assumed that the test case requires both FDD and TDD.

Additional Information - Specific IXIT

This column contains the mnemonics of IXIT(s) affecting the dynamic behaviour of the TC.

Additional Information - Number of TC Executions

This column contains, wherever applicable, the recommended for certification purposes number of TC executions. It may contain also other information e.g. exceptions to the release applicable to the test. Clarifying notes are listed in Table 4-1b.

Additional Information - Release other RAT

In regard to a particular test case, this column provides information on the release which is used by the simulated network in the other (i.e. non E-UTRA) RAT(s) where applicable. For each applicable RAT the release shall be indicated in the format 'Rel-X RAT'. When multiple RATs are applicable the entries per RAT shall be separated by a comma. When a value for a 3GPP RAT is not provided but the RAT is in the scope of the test case then for this RAT the release indicated in the Release column applies (per default).

EXAMPLES:

Rel-9 UTRA FDD, Rel-8 GERAN or simply as Rel-9 UTRA FDD
(meaning that the UTRA FDD will simulate Rel-9 and the GERAN Rel-8 behaviours)

Rel-9 UTRA TDD
(meaning that the UTRA LCR TDD network will simulate Rel-9 behaviours)

NOTE 2:	To meet the validation requirements from certification bodies then there is a need to uniquely reference the FDD and TDD branch of common FDD and TDD test cases. The FDD and TDD branches of common FDD and TDD test cases can be referenced by amending a "FDD" or "TDD" suffix to the test case clause number. For example for AM RLC test case 7.2.3.13 the FDD and TDD branches can be identified by "7.2.3.13 FDD" and "7.2.3.13 TDD".

Table 4-1: Applicability of tests and additional information for testing

| Clause |  | TC Title |  | Release |  | Applicability |  |  | Additional Information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment | Specific ICS |  |  | Specific IXIT |  | Number of TC Executions |  | Release other RAT |  |
| 6 |  | Idle mode operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <<< ROWS SKIPPED >>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  | Layer 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <<< ROWS SKIPPED >>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.3.20 |  | Short TTI / Correct handling of DL assignment / multiplexing of SPDCCH and slot/subslot-PDSCH |  | Rel-15 |  | C381 |  | UE supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs and L1-based SPDCCH reuse |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.3.21 |  | Short TTI / Correct handling of DL assignment / DMRS sharing |  | Rel-15 |  | C380 |  | UEs supporting E-UTRA and {subslot, subslot} combination in downlink and uplink CCs and minimum processing timeline |  |  | pc_eFDD |  |  |  |  |  |  |
| 7.1.3.22 |  | Short Processing Time / Correct handling of DL assignment / HARQ process sharing |  | Rel-15 |  | C378 |  | UE supporting E-UTRA and short processing time |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.3.23 |  | Enhanced Coverage / DL Fexible starting PRB |  | Rel-15 |  | Cxxx |  | UEs supporting E-UTRA and CE Mode A and flexible starting PRB for PDSCH |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.4.1 |  | Correct handling of UL assignment / Dynamic case |  | Rel-8 |  | R |  | UEs supporting E-UTRA |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.4.1a |  | Correct handling of UL assignment / Dynamic case / Skip padding transmissions |  | Rel-14 |  | C325 |  | UE supporting skip of uplink transmissions if no data is available |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.4.2 |  | Correct handling of UL assignment / Semi-persistent case |  | Rel-8 |  | C100F |  | UEs supporting E-UTRA and semi-persistence scheduling and Feature Group Indicator 7 |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  | C100T |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| <<< ROWS SKIPPED >>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.4.40 |  | Short TTI / Correct handling of MAC control information / Scheduling requests and SPUCCH |  | Rel-15 |  | C379a |  | UEs supporting E-UTRA and {slot, slot} combination in downlink and uplink CCs |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.4.41 |  | Short TTI / Correct handling of UL assignment / HARQ sharing between PUSCH and slot/subslot-PUSCH |  | Rel-15 |  | C383 |  | UEs supporting E-UTRA and short processing time and {slot, slot} combination in downlink and uplink CCs |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.4.42 |  | Enhanced Coverage / UL Fexible starting PRB |  | Rel-15 |  | Cyyy |  | UEs supporting E-UTRA and CE Mode A and flexible starting PRB for PUSCH |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| 7.1.4a.1 |  | Correct downlink reception and uplink transmission when specific valid subframes are signalled for BL UE |  | Rel-13 |  | C254 |  | UEs supporting E-UTRA and (CE Mode A or CE Mode B) |  |  | pc_eFDD |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | pc_eTDD |  |  |  |  |  |  |
| <<< ROWS SKIPPED >>> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4-1a: Applicability of tests Conditions

<<< ROWS SKIPPED >>>

C403 IF A.4.1-1/9 AND A.4.4-1A/17 THEN R ELSE N/A

C404 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/220 THEN R ELSE N/A

C405 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/216 AND A.4.4-1/221

Cxxx IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/XX THEN R ELSE N/A

Cyyy IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/YY THEN R ELSE N/A

<<< TEXT SKIPPED >>>

## A.4.4	Additional information

Table A.4.4-1: Additional information

| Item | Additional information | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of USIM removal without power down |  | Rel-8 | pc_USIM_Removal |  |
| 2 | Support of Allowed CSG list | 36.331 Annex B.2 | Rel-8 | pc_Allowed_CSG_list | For Rel-8: CSG autonomous search is optional. For Rel-9 or later releases: CSG autonomous search is mandatory for UEs supporting CSG full functionality. |
| 3 | Support of Short Message Service (SMS) MT over SGs | 23.272, 8.2.4, 8.2.5 | Rel-8 | pc_SMS_SGs_MT |  |
| 4 | Support of Short Message Service (SMS) MO over SGs | 23.272, 8.2.2, 8.2.3 | Rel-8 | pc_SMS_SGs_MO |  |
| 5 | Support of ISR | 23.401, 4.3.5.6 | Rel-8 | pc_ISR |  |
| 6 | Support of Mobility management based on Dual-Stack Mobile IPv6 | 24.303 | Rel-8 | pc_DSMIPv6 |  |
| 7 | Support for being configured to discover the Home Agent address via DNS | 24.303 | Rel-8 | pc_HAAddress_via_DNS |  |
| 8 | Support of inter-RAT PS handover to E-UTRA (FDD) from UTRA | 25.306, 4.7 | Rel-8 | pc_HO_from_UTRA_to_eFDD |  |
| 9 | Support of EMM information message | 24.301, 5.4.5.3 | Rel-8 | pc_EMM_Information |  |
| 10 | Support for being configured to discover the Home Agent address via DHCPv6 | 24.303 | Rel-8 | pc_HAAddress_via_DHCPv6 |  |
| 11 | Void |  |  |  |  |
| 12 | Upon reception of ‘Full name for network’ information the UE stores/updates the network full name | 24.301, 8.2.13 | Rel-8 | pc_FullNameNetwork |  |
| 13 | Upon reception of ‘Short name for network’ information the UE stores/updates the network short name | 24.301, 8.2.13 | Rel-8 | pc_ShortNameNetwork |  |
| 14 | Upon reception of ‘Local time zone’ information the UE stores/updates the local time zone | 24.301, 8.2.13 | Rel-8 | pc_LocalTimeZone |  |
| 15 | Upon reception of ‘Universal time and local time zone’ information the UE stores/updates the universal time and local time zone | 24.301, 8.2.13 | Rel-8 | pc_UniversalAndLocalTimeZone |  |
| 16 | Void |  |  |  |  |
| 17 | Void |  |  |  |  |
| 18 | Support of ESM UE requested bearer resource allocation procedure | 24.301, 6.5.3 | Rel-8 | pc_ESM_MO_Bearer_Allocation |  |
| 19 | Support of ESM UE requested bearer resource modification procedure | 24.301, 6.5.4 | Rel-8 | pc_ESM_MO_Bearer_Modification |  |
| 20 | Support of ETWS message | 23.401, 5.12.2 | Rel-8 | pc_ETWS_message |  |
| 21 | Supports E-UTRAN Neighbour Cell measurements and MS autonomous cell reselection to E-UTRAN | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_meas |  |
| 22 | Support for being configured to request the IPv6 address of the Home Agent during Attach procedure | 24.303 | Rel-8 | pc_RequestIPv6HAAddress_DuringAttach |  |
| 23 | Support for being configured to request the IPv4 address of the Home Agent during Attach procedure | 24.303 | Rel-8 | pc_RequestIPv4HAAddress_DuringAttach |  |
| 24 | Void |  |  |  |  |
| 25 | Support of IMS | 24.229 | Rel-8 | pc_IMS |  |
| 26 | Supports of disabling the EPS services | 24.301, 3.1, 5.5.2.1 | Rel-8 | pc_EPS_Services_Disable |  |
| 27 | Support of automatic re-activation of the EPS bearer(s) during Network Initiated Detach with detach type set to "re-attach required" | 24.301, 5.5.2.3.2 | Rel-8 | pc_Automatic_Re_Attach |  |
| 28 | Support of Compressed mode | 25.306 | Rel-8 | pc_UTRA_CompressedModeRequired |  |
| 29 | Support of GERAN to E-UTRAN PS Handover | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_PSHO |  |
| 30 | Support for multiple PDN connections | 23.401, 5.10 | Rel-8 | pc_Multiple_PDN |  |
| 31 | Support of use of the UTRA system information provided by RRCConnectionRelease upon redirection | 36.306 | Rel-9 | pc_eRedirectionUTRA |  |
| 32 | Support for SRVCC from E-UTRAN to GERAN/UTRAN | 24.301, 8.2.4 | Rel-8 | pc_SRVCC_GERAN_UTRAN |  |
| 33 | Support for VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS" | 24.173, 24.229, 26.114, 5.2.1, GSMA PRD IR.92 | Rel-8 | pc_VoLTE | Multimedia telephony service participant initiating a speech session. UE supports sending DTMF events over RTP. |
| 34 | Support of detach for non-EPS services | 24.301, 5.5.2.1 | Rel-8 | pc_IMSI_Detach |  |
| 35 | Support for establishing the emergency call using the CS domain in UTRA after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_UTRA |  |
| 36 | Support for establishing the emergency call using the CS domain in GERAN after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_GERAN |  |
| 37 | Support for establishing the emergency call using the CS domain in 1xRTT after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_1xRTT |  |
| 38 | Support for EDTM | 44.060 8.9.1.2 | Rel-8 | pc_EDTM |  |
| 39 | Supports CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_measreporting_CCN |  |
| 40 | Support for ROHC profile0x0001 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0001 | 'IMS capable UEs supporting voice' shall set this PICS to true. |
| 41 | Support for ROHC profile0x0002 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0002 | 'IMS capable UEs supporting voice' shall set this PICS to true. |
| 42 | Support for ROHC profile0x0003 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0003 |  |
| 43 | Support for ROHC profile0x0004 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0004 |  |
| 44 | Support for ROHC profile0x0006 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0006 |  |
| 45 | Support for ROHC profile0x0101 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0101 |  |
| 46 | Support for ROHC profile0x0102 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0102 |  |
| 47 | Support for ROHC profile0x0103 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0103 |  |
| 48 | Support for ROHC profile0x0104 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0104 |  |
| 49 | Support of manual CSG selection | 36.331, Annex B2 | Rel-8 | pc_Manual_CSG_Selection | For Rel-8: manual CSG selection is optional. For Rel-9 or later releases: manual CSG selection is mandatory for UEs supporting CSG full functionality. |
| 50 | Support of semi-persistence scheduling | 36.331, Annex B1 | Rel-8 | pc_Semi_Persistence_Scheduling | For Rel-8: semi-persistence scheduling is mandatory if pc_FeatrGrp_3 is set to true. For Rel-9 or later releases: semi-persistence scheduling is mandatory if pc_FeatrGrp_29 is set to true. |
| 51 | Support of TTI bundling | 36.331, Annex B1 | Rel-8 | pc_TTI_Bundling | For Rel-8: TTI bundling is mandatory if pc_FeatrGrp_3 is set to true. For Rel-9 or later releases TDD: TTI bundling is mandatory if pc_FeatrGrp_28 is set to true. For Rel-9 or later releases FDD: TTI bundling is mandatory. |
| 52 | Support for inter-RAT PS handover from E-UTRAN to GERAN. | 36.306, 4.3.7.11 | Rel-8 | pc_E_UTRAN_2_GERAN_PSHO |  |
| 53 | Support of inter-RAT PS handover to E-UTRA (TDD) from UTRA | 25.306, 4.7 | Rel-8 | pc_HO_from_UTRA_to_eTDD |  |
| 54 | Support for UE requested modification of network allocated TFTs | 24.301, 6.5.4 | Rel-8 | pc_ESM_UE_Modification_NW_TFT |  |
| 55 | Support of automatic re-activation of the EPS bearer(s) during Network Initiated Detach even though UE has initiated a detach procedure with detach type set to "EPS detach" or "combined EPS/IMSI detach" | 24.301, 5.5.2.2.4 | Rel-8 | pc_Re_Attach_AfterDetachColl |  |
| 56 | Support of Squal based cell reselection to UTRAN from E-UTRAN | 25.304, 5.2.6.1.4a | Rel-9 | pc_Squal_based_CellReselection_to_UTRAN_from_E_UTRAN |  |
| 57 | Support of Squal based cell reselection to E-UTRAN from UTRAN | 36.304, 5.2.4.5 | Rel-9 | pc_Squal_based_CellReselection_to_E_UTRAN_from_UTRAN |  |
| 58 | Support of CMAS message | 36.331, 5.2.1.5 | Rel-9 | pc_CMAS_Message |  |
| 59 | Void |  |  |  |  |
| 60 | Void |  |  |  |  |
| 61 | Void |  |  |  |  |
| 62 | Support of logged measurements in RRC_IDLE | 36.306, 4.3.13.1 | Rel-10 | pc_LoggedMeasurementsIdle |  |
| 63 | Support of standalone GNSS receiver to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE | 36.306, 4.3.13.2 | Rel-10 | pc_standaloneGNSS_Location |  |
| 64 | Support of automatic re-activation of the EPS bearer(s) | 24.301 | Rel-8 | pc_Automatic_EPS_Re_Attach |  |
| 65 | Support of UTRAN ANR | 25.306, 4.15 | Rel-10 | pc_UTRAN_ANR |  |
| 66 | Void |  |  |  |  |
| 67 | Support of PWS upper layer | 23.041 clause 9.1.3.4.2 | Rel-9 | pc_PWS_UpperLayer |  |
| 68 | Support of automatic PDN connectivity in EUTRAN (i.e. UE upper layer provides PDN connectivity parameters) | 24.301, 6.5.1.1 | Rel-8 | pc_Auto_PDN_Connectivity |  |
| 69 | Support user initiated PLMN reselection in automatic mode | 23.122 | Rel-8 | pc_UserInitiatedPLMN_Reselection |  |
| 70 | Support of UL MIMO | 36.306, clause 4.3.4.6 | Rel-10 | pc_UL_MIMO |  |
| 71 | Support of ESM Notification procedure | 24.301, 6.6.2 | Rel-9 | pc_ESM_Notification |  |
| 72 | Support of sending concatenated multiple Short Message over SGs | 23.272, 8.2.3a | Rel-9 | pc_SMS_SGs_Multi_MO |  |
| 73 | Support TAU in connected mode | 23.221, 7.2a | Rel-8 | pc_TAU_connected_in_IMS | Applicable when configured to pc_voice_PS_1_CS_2 and pc_Attach |
| 74 | Support TAU in idle mode | 23.221, 7.2a | Rel-8 | pc_TAU_idle_in_IMS |  |
| 75 | Support of Intra Frequency Proximity Indication | 36.306, clause 4.3.10.1 | Rel-9 | pc_IntraFreq_ProximityIndication |  |
| 76 | Support of Inter Frequency Proximity Indication | 36.306, clause 4.3.10.2 | Rel-9 | pc_InterFreq_ProximityIndication |  |
| 77 | Support of UTRAN Proximity Indication | 36.306, clause 4.3.10.3 | Rel-9 | pc_UTRAN_ProximityIndication |  |
| 78 | Support of Access Technology Indication in available PLMNs list | 23.122, clause 4.4.3.1.2 | Rel-8 | pc_Available_PLMNs_AcT_Ind |  |
| 79 | Support of Squal based cell reselection between E-UTRAN and GERAN | 36.304, clause 5.2.4.5, 45.008, clause 6.6.6 | Rel-9 | pc_Squal_based_CellReselection_between_E_UTRAN_and_GERAN |  |
| 80 | Support of AttachWithIMSI | 24.368, 5.4 | Rel-10 | pc_eAttachWithIMSI |  |
| 81 | Support of T3412 extended value IE | 24.301, 8.2.1.12, 8.2.26.15 | Rel-10 | pc_T3412Extended |  |
| 82 | Void |  |  |  |  |
| 83 | Void |  |  |  |  |
| 84 | Support of MinimumPeriodicSearchTimer | 23.122, 4.4.3.3 | Rel-10 | pc_eMinimumPeriodicSearchTimer |  |
| 85 | Support of delivery of rachReport upon request from the network | 36.306, 4.3.12.1 | Rel-9 | pc_Rach_Report |  |
| 86 | Support of Power Preference Indication | 36.306 4.3.15.3, 36.331, 5.6.10 | Rel-11 | pc_PPI_Support |  |
| 87 | Support of ePDCCH | 36.306, 4.3.4.18 36.331, 6.3.6 | Rel-11 | pc_ePDCCH |  |
| 88 | Void |  |  |  |  |
| 89 | Void |  |  |  |  |
| 90 | Void |  |  |  |  |
| 91 | Support of Extended Access Barring Override | 24.368, 5.10, 31.102, 4.2.94 | Rel-11 | pc_EAB_override |  |
| 92 | Void |  |  |  |  |
| 93 | Upon reception of ‘Daylight saving time’ information the UE stores/updates the daylight saving time | 24.301, 8.2.13 | Rel-8 | pc_DaylightSavingTime |  |
| 94 | Support of Radio Link Failure Report for inter-RAT MRO | 36.306, clause 6.10.1 | Rel-11 | pc_RLF_ReportForInterRAT_MRO |  |
| 95 | Support of IPv4 | 23.221, 5.1 | Rel-5 | pc_IPv4 |  |
| 96 | Support of IPv6 | 23.221, 5.1 | Rel-5 | pc_IPv6 |  |
| 97 | Support of Automatic Mode EF_LRPLMSI PLMN Selection exception | 23.122, 4.4.3.1 | Rel-8 | pc_PLMN_EF_LRPLMNSI_Automatic_Mode_Exception |  |
| 98 | Support of Manual Mode PLMN Selection exception | 23.122, 4.4.3.1 | Rel-8 | pc_PLMN_Manual_Mode_Exception |  |
| 99 | Support of ZUC algorithm | 33.401,5.1.3.2 | Rel-11 | pc_ZUC |  |
| 100 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring UMTS cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.3 | Rel-9 | pc_SI_Neighbour_UMTS_Autonomous_Gaps |  |
| 101 | Support of reception of requestedFrequencyBands | 36.306, 4.3.5.6 | Rel-11 | pc_reqFreqBands |  |
| 102 | Support of more than 128 CA Band Combinations | 36.331, 5.6.3.3, 6.4 | Rel-11 | pc_More_Than_128_CAbandComb |  |
| 103 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring intra-frequency cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.1 | Rel-9 | pc_SI_Neighbour_intraFreq_Autonomous_Gaps |  |
| 104 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring inter-frequency cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.2 | Rel-9 | pc_SI_Neighbour_interFreq_Autonomous_Gaps |  |
| 105 | Support of Type B Half-duplex FDD operation | 36.211, 6.2.5 36.306, 4.2.6 | Rel-12 | pc_FDD_TypeB_HalfDuplex | Only applicable for UE supporting Category 0 and Category M1 and M2. When set transmission scheduling is performed in accordance to Half-Duplex operation Type B else in accordance to Full-Duplex operation. |
| 106 | Void |  |  |  |  |
| 107 | Support of enhanced HARQ pattern for TTI bundling operation for FDD | 36.306, 4.3.4.27 | Rel-12 | pc_eHARQ_Pattern_for_TTI_bundling |  |
| 108 | Support of tdd-FDD-CA-PCellDuplex-r12 with the first bit setting to "1" | 36.306, 4.3.4.28 | Rel-12 | pc_tdd_FDD_CA_TDD_PCell |  |
| 109 | Support of tdd-FDD-CA-PCellDuplex-r12 with the second bit setting to "1" | 36.306, 4.3.4.28 | Rel-12 | pc_tdd_FDD_CA_FDD_PCell |  |
| 110 | Support of ProSe direct communication | 36.306, 4.3.21.1 | Rel-12 | pc_commSupportedBands | 36.306, 4.3.21.1: If a UE supports sidelink communication on at least one band, the UE shall support sidelink communication transmission based on UE autonomous resource selection and eNB scheduled resource allocation. |
| 111 | Support of ProSe direct discovery | 36.306, 4.3.21.3 | Rel-12 | pc_discSupportedBands |  |
| 112 | Support of ProSe EPC level discovery | 24.334, 7.2 | Rel-12 | pc_Prose_EPC_Discovery |  |
| 113 | Support of ProSe discovery SLSS transmission and reception | 36.306, 4.3.21.6 | Rel-12 | pc_discSLSS |  |
| 114 | Support of uplink 64QAM | 36.306, 4.3.4.39 | Rel-12 | pc_UL_64QAM |  |
| 115 | Support of Power Saving Mode | 24.301, 5.3.11 | Rel-12 | pc_ePSM |  |
| 116 | Support of downlink 256QAM | 36.306, 4.1, 4.1A | Rel-12 | pc_DL_256QAM | Applicable for UEs of category 11-12 and UEs of DL category 11 and onwards. It is mandatory for UEs of DL category 13-14. |
| 117 | Support for GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" | IEEE Std 802.11 GSMA PRD IR.51 | Rel-11 | pc_WLAN_voice | The IR.51 is based on 3GPP Rel-11. |
| 118 | Support of CSI-RS based discovery signals measurement | 36.306 4.3.6.10 | Rel-12 | pc_CSI_RS_DS_Meas |  |
| 119 | Support of simultaneous transmission of EUTRA and sidelink communication (on different carriers) in all bands for which the UE indicated simultaneous sidelink and EUTRA support in a band combination (using commSupportedBandsPerBC) | 36.306, 4.3.21.2 | Rel-12 | pc_commSimultaneousTx |  |
| 120 | ProSe Discovery for Public Safety supported | 24.334, 4.1 | Rel-12 | pc_disc_public_safety | If Support of ProSe direct discovery (entry 111) is indicated then if the present entry is set to FALSE this shall be understood as ProSe Discovery for non-Public Safety supported |
| 121 | Support of extended DRX | 24.301, 5.3.12 | Rel-13 | pc_edrx |  |
| 122 | Support of CE mode A | 36.306, 4.3.29.1 | Rel-13 | pc_CEmodeA | Mandatory for CAT M1 and M2 UEs |
| 123 | Support of CE mode B | 36.306, 4.3.29.2 | Rel-13 | pc_CEmodeB |  |
| 124 | Support of TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations | 36.306, 4.3.4.31 | Rel-12 | pc_eIMTA_TDD |  |
| 125 | Support of prioritization of the frequency bands in multiBandInfoList over the band in freqBandIndicator as defined by freqBandIndicatorPriority-r12 | 36.306, 4.3.5.11 | Rel-12 | pc_freqBandPriorityAdjustment |  |
| 126 | Support of MBMS reception via SC-PTM on configured SCell | 36.306, 4.3.5.2 | Rel-13 | pc_scptm_SCell |  |
| 127 | Support of MBMS reception via SC-PTM on a cell that may be additionally configured as an SCell | 36.306, 4.3.5.2 | Rel-13 | pc_scptm_NonServingCell |  |
| 128 | Support of extended Long DRX cycle | 36.306, 4.3.19.4 | Rel-13 | pc_extendedLongDRX |  |
| 129 | Supports downlink LAA operation | 36.306, 4.3.23.1 | Rel-13 | pc_downlink_LAA |  |
| 130 | Supports measurement and reporting for RSSI and channel occupancy | 36.306, 4.3.6.19 | Rel-13 | pc_rssiAndChannelOccupancyReporting |  |
| 131 | Support of QCI1 indication in Radio Link Failure Report | 36.306, 6.8.2 | Rel-13 | pc_qci1Indication_inRLF |  |
| 132 | Support of user plane CIoT optimisation in WB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_User_Plane_CIoT_Optimisation |  |
| 133 | Support of EMM-REGISTERED without PDN | 24.301, 5.3.15 | Rel-13 | pc_AttachWithoutPDN |  |
| 134 | Support of EMM-REGISTERED with PDN | 24.301, 5.3.15 | Rel-13 | pc_AttachWithPDN |  |
| 135 | Void |  |  |  |  |
| 136 | Void |  |  |  |  |
| 137 | Support of multiple DRBs in NB-IoT | 36.306, 4.3.8.5 | Rel-13 | pc_NB_MultiDRB |  |
| 138 | Support of Fast First Higher Priority PLMN search | 23.122, 4.4.3.3.1 | Rel-12 | pc_Fast_First_HPPLMN_Search |  |
| 139 | Support of TDD Bands38, 40, 41 or 42 Power class 2 operation | 36.101, 6.2.2 | Rel-14 | pc_TDD_band_UE_PC2 |  |
| 140 | Support for PDCP Packet Delay per QCI | 36.331, 5.5.2 | Rel-13 | pc_PDCP_PktDelay |  |
| 141 | Void |  |  |  |  |
| 142 |  |  |  |  |  |
| 143 | Support of Control plane CIoT in WB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_Control_Plane_CIoT_Optimisation |  |
| 144 | Support of S1-U data transfer | 24.301, 5.3.15 | Rel-13 | pc_S1_U_DataTransfer | An UE supporting user plane CIoT optimization shall set this PICS to true. |
| 145 | Support for GSMA PRD NG.108: "IMS Profile for Voice and SMS for UE category M1" | GSMA PRD NG.108 | Rel-13 | pc_Category_M1_voice |  |
| 146 | Support of automatic PDN connection trigger on HRPD cell reselection | X.s0057, 6.4.1 | Rel-8 | pc_AutomaticHRPD_PDN_Connection |  |
| 147 | Support for Dual RM Coding | 36.331, 6.3.6 | Rel-10 | pc_DualRM_Coding |  |
| 148 | Support of V2X sidelink communication | 36.300, 23.14.1.1 | Rel-14 | pc_v2xCommSidelink |  |
| 149 | Support of V2X communication Via Uu | 36.300, 23.14.1.1 | Rel-14 | pc_v2xCommUu |  |
| 150 | Support of simultaneous transmission of EUTRA and V2X sidelink communication | 36.306, 4.3.5.27 | Rel-14 | pc_v2xSimultaneousTx |  |
| 151 | Support of simultaneous reception of EUTRA and V2X sidelink communication | 36.306, 4.3.5.27 | Rel-14 | pc_v2xSimultaneousRx |  |
| 152 | Support of transmitting PSCCH/PSSCH using dynamic scheduling | 36.306, 4.3.21.14 | Rel-14 | pc_v2xScheduling |  |
| 153 | Support of transmitting PSCCH/PSSCH using UE autonomous resource selection mode with full sensing | 36.306, 4.3.21.15 | Rel-14 | pc_v2xFullSensing |  |
| 154 | Support of transmitting PSCCH/PSSCH using UE autonomous resource selection mode with partial sensing | 36.306, 4.3.21.16 | Rel-14 | pc_v2xPartialSensing |  |
| 155 | Support of SLSS transmission and reception for V2X sidelink communication | 36.306, 4.3.21.17 | Rel-14 | pc_v2xSLSS |  |
| 156 | Support of CBR measurement and reporting | 36.306, 4.3.21.18 | Rel-14 | pc_v2xCBRMeas |  |
| 157 | Support of zone based transmission resource pool selection for V2X sidelink communication | 36.306, 4.3.21.12 | Rel-14 | pc_v2xZoneBasedPoolSelection |  |
| 158 | Require intra-frequency measurement gaps for operating in CE Mode A or CE Mode B | 36.306, 4.3.5.1.2 | Rel-13 | pc_intraFreq-CE-NeedForGaps |  |
| 159 | Support of 4 layer spatial multiplexing with transmission mode 3 and transmission mode 4 | 36.306, 4.3.4.7 | Rel-10 | pc_4Layer_spatial_mux_tm3_tm4 |  |
| 160 | Support of delay budget reporting for MMTEL voice and video enhancements | 36.306, 4.3.32.1 | Rel-14 | pc_delayBudgetReporting |  |
| 161 | Support of PUSCH enhancement for MMTEL voice and video enhancements mode | 36.306, 4.3.32.2 | Rel-14 | pc_PUSCH_Ehn_MMTEL |  |
| 162 | Void |  |  |  |  |
| 163 | Support of PUCCH transmission on SCell in CA | 36.306, 4.3.4.47 | Rel-13 | pc_PUCCH_SCell |  |
| 164 | Support high speed enhancement for random access preambles generated from restricted set type B in high speed scenoario as specified in TS 36.211 | 36.306 | Rel-14 | pc_Highspeed_Enh_Prach |  |
| 165 | Support of RRC connection re-establishment | 36.306, 6.7.5 | Rel-14 | pc_RRC_re-establishment_CP_CIoT | An UE supporting S1-U data transfer shall set this PICS to true. |
| 166 | Support of SRS switching between a band pair | 36.306, 4.3.5.24, 4.3.5.25 | Rel-14 | pc_SRS_switching | Support of SRS switching between a band pair |
| 167 | Support of 2 HARQ processes in DL and UL in NB-IoT | 36.306, 4.3.4.62 | Rel-14 | pc_NB_TwoHARQ_Processes |  |
| 168 | Support of Release Assistance Indication (RAI) in NB-IoT | 36.306, 4.3.19.10 | Rel-14 | pc_NB_Rai_Support |  |
| 169 | Support of Announcing for ProSe Group Member Discovery | 24.334, 10A.2.6 | Rel-13 | pc_ProSeAnnForGroupMemberDiscovery |  |
| 170 | Support of SPS interval shorter than 10 subframes in FDD mode | 36.306, 4.3.19.5 | Rel-14 | pc_shortSPS_intervalFDD |  |
| 171 | Support of SPS interval shorter than 10 subframes in TDD mode | 36.306, 4.3.19.6 | Rel-14 | pc_shortSPS_intervalTDD |  |
| 172 | Support of skipping SPS UL transmissions if no data is available | 36.306, 4.3.19.8 | Rel-14 | pc_skipUplinkSPS | An UE supporting SPS interval shorter than 10 (pc_shortSPS_intervalFDD or pc_shortSPS_intervalTDD) shall set this PICS to true. |
| 173 | Support of skipping UL transmissions if no data is available | 36.306, 4.3.19.7 | Rel-14 | pc_skipUplinkDynamic |  |
| 174 | Supports uplink LAA operation | 36.306, 4.3.23.8 | Rel-14 | pc_uplink_LAA | Support of Enhanced LAA operations |
| 175 | Void |  |  |  |  |
| 176 | Supports two step uplink scheduling using PUSCH trigger A and PUSCH trigger B | 36.306, 4.3.23.10 | Rel-14 | pc_twoStepScheduling_uplink_LAA | UE supports two step uplink scheduling using PUSCH trigger A and PUSCH trigger B, applying to the UE supports uplink LAA operation |
| 177 | Supports multiple uplink SPS and reporting SPS assistance information | 36.306, 4.3.19.11 | Rel-14 | pc_multipleUplinkSPS | Support of multiple uplink SPS and reporting SPS assistance information |
| 178 | Support of V2X communication as Pedestrian UE | 36.300, 23.14.1.1 | Rel-14 | pc_P2X_UE |  |
| 179 | Support of the uplink data compression operation | 36.306, 4.3.1.7 | Rel-15 | pc_UDC |  |
| 180 | Support of UL data compression with SIP static dictionary | 36.306, 4.3.1.8 | Rel-15 | pc_UDC_SIP |  |
| 181 | Support of QoE Measurement Collection for Streaming Service | 36.306, 4.36.30 | Rel-15 | pc_qoe_MeasReport |  |
| 182 | Support of QoE Measurement Collection for MTSI Service | 36.306, 4.36.33 | Rel-15 | pc_qoe_MTSI_MeasReport |  |
| 183 | Support of 256QAM in UL | 36.306, 4.3.4.73 | Rel-14 | pc_UL_256QAM |  |
| 184 | Support of Bluetooth Measurement Collection in logged MDT | 36.306, 4.3.13.6 | Rel-15 | pc_BT_Meas_logged_MDT |  |
| 185 | Support of WLAN Measurement Collection in logged MDT | 36.306, 4.3.13.7 | Rel-15 | pc_WLAN_Meas_logged_MDT |  |
| 186 | Support of Bluetooth Measurement Collection in Immediate MDT | 36.306, 4.3.13.8 | Rel-15 | pc_BT_Meas_Imm_MDT |  |
| 187 | Support of WLAN Measurement Collection in Immediate MDT | 36.306, 4.3.13.9 | Rel-15 | pc_WLAN_Meas_Imm_MDT |  |
| 188 | Support of ce-PUSCH-NB-MaxTBS-r14 | 36.306, 4.3.4.63 | Rel-15 | pc_ce_PUSCH_NB_MaxTBS |  |
| 189 | Support of height-based measurement reporting | 36.306, 4.3.6.35 | Rel-15 | pc_heightMeas |  |
| 190 | Support of GNSS for height measurement |  | Rel-15 | pc_gnss_heightMeas |  |
| 191 | Support of measurement reporting triggered based on a number of cells | 36.306, 4.3.6.34 | Rel-15 | pc_Multiple_Cells_Meas_Ext |  |
| 192 | Support of flight path plan reporting | 36.306, 4.3.15.14 | Rel-15 | pc_FlightPathPlan |  |
| 193 | Void |  |  |  |  |
| 194 | Support of HARQ-ACK bundling | 36.213, 7.3.1 | Rel-14 | pc_ce_HARQ_AckBundling | Support of HARQ-ACK bundling |
| 195 | Support of eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode. | 36.306, 4.3.6.31 | Rel-15 | pc_idleModeMeasurement |  |
| 196 | Support of the dormant SCell state. | 36.306, 4.3.19.16 | Rel-15 | pc_dormantSCellState |  |
| 197 | Support of having SCell configured in dormant SCell state | 36.306 4.3.19.18 | Rel-15 | pc_directSCellHibernation |  |
| 198 | Support of having SCell configured in activated SCell state | 36.306, 4.3.19.17 | Rel-15 | pc_directSCellActivation |  |
| 199 | Support of user plane CIoT optimisation in NB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_NB_User_Plane_CIoT_Optimisation |  |
| 200 | Support of Control Plane Early Data Transmission | 36.306, 6.8.4 | Rel-15 | pc_Control_Plane_CIoT_Optimisation_EDT |  |
| 201 | Support of User Plane Early Data Transmission | 36.306, 4.3.8.7 | Rel-15 | pc_User_Plane_CIoT_Optimisation_EDT |  |
| 202 | Support of RLC UM mode in NB-IoT | 36.306, 4.3.2.5 | Rel-15 | pc_NB_RLC_UM |  |
| 203 | Support of short TTI and/or short processing time | 36.306, 4.3.4.150 | Rel-15 | pc_sTTI_SPT |  |
| 204 | Support of short processing time for the corresponding frame structure types | 36.306, 4.3.4.100 | Rel-15 | pc_spt_Parameters |  |
| 205 | Support of sTTI in downlink CCs and uplink CCs | 36.306, 4.3.4.103 | Rel-15 | pc_sTTI_Combinations |  |
| 206 | Support of {subslot, subslot} combinations in downlink CCs and uplink CCs | 36.306, 4.3.4.103 | Rel-15 | pc_subslot_Combinations |  |
| 207 | Support of L1-based SPDCCH reuse | 36.306, 4.3.4.147 | Rel-15 | pc_SPDCCH_Reuse |  |
| 208 | Support of SRS trigerring via DCI format 7 for FS2 | 36.306, 4.3.4.181 | Rel-15 | pc_SRS_DCI7_Triggering |  |
| 209 | Support of UL asynchronous HARQ sharing between different TTI lengths for an UL serving cell. | 36.306, 4.3.4.156 | Rel-15 | pc_ul_AsyncHarqSharingDiffTTI |  |
| 210 | Support of Wake Up Signal | 36.306, 4.3.4.113 | Rel-15 | pc_wakeUpSignal |  |
| 211 | Support of physical layer SR with HARQ ACK | 36.306, 4.3.4.117 | Rel-15 | pc_SR_WithHARQ_ACK |  |
| 212 | Support of physical layer SR without HARQ ACK | 36.306, 4.3.4.118 | Rel-15 | pc_SR_WithoutHARQ_ACK |  |
| 213 | UE supports Ethernet header compression and decompression using EHC protocol | 36.306, 4.3.1.12 | Rel-16 | pc_EUTRAN_EHC |  |
| 214 | UE supports DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.40 | Rel-16 | pc_EUTRA_intraFreqDAPS |  |
| 215 | Support of RACS | 24.301, 5.3.20 | Rel-16 | pc_EPC_RACS |  |
| 216 | Support of RRC message Segmentation in the UL | 36.306, 6.8.12 | Rel-16 | pc_LTE_UL_Segmentation | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |
| 217 | UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. | 36.306, 4.3.30.3 | Rel-16 | pc_EUTRA_cho_r16 |  |
| 218 | Support of Mixed Operation Mode in NB-IoT | 36.306, 4.3.4.115 | Rel-15 | pc_NB_mixedOperationMode |  |
| 219 | Support of NPRACH resources using preamble format 2 for FDD in NB-IoT | 36.306, 4.3.4.119 | Rel-15 | pc_NB_nprach_Format2 |  |
| 220 | UE supports DAPS handover in source PCell and inter-frequency target PCell | 36.306, 4.3.5.43 | Rel-16 | pc_EUTRA_interFreqDAPS |  |
| 221 | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over LTE | 36.509, 5.10 | Rel-16 | pc_Set_UE_Cap_Info_LTE |  |
| XX | Support of flexible starting PRB for PDSCH | 36.306, 4.3.4.121 and 4.3.4.122 | Rel-15 | pc_FlexibleStartPRB_PDSCH |  |
| YY | Support of flexible starting PRB for PUSCH | 36.306, 4.3.4.123 and 4.3.4.124 | Rel-15 | pc_FlexibleStartPRB_PUSCH |  |