3GPP TSG-RAN5 Meeting #93-e	R5-218291

# Electronic Meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0213 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add assistance data for OTDOA feMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The assistance data for OTDOA feMTC test cases for measurement period with longer PRS occasions is not included in TS 37.571-5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new assistance data tables for test cases 9.4.13 to 9.4.18 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The OTDOA eMTC4 work plan cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 37.571-1 CR 0348 to 0354 TS/TR 37.571-3 CR 0148 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-217144 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 7.5.2	OTDOA Assistance Data

This subclause defines the OTDOA assistance data elements which shall be provided to the UE in the eMTC OTDOA measurement tests defined in TS 37.571-1 [6].

OTDOA REFERENCE CELL INFO:

Table 7.5.2-1: OTDOA-ReferenceCellInfo for eMTC intra-frequency RSTD reporting delay test cases 9.3.1.1 to 9.3.6.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.3.x.y.4.1-1 in TS 37.571-1 [6], where x and y represent part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 311 TDD tests: 304 |  |
| numDL-Frames | Test 1: sf-6 Test 2, tests 9.3.1.2, 9.3.2.2 and 9.3.3.2: sf-2 Test 2, tests 9.3.4.2, 9.3.5.2 and 9.3.6.2: sf-4 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | ‘1111 0000’ |  |

Table 7.5.2-2: OTDOA-ReferenceCellInfo for eMTC intra-frequency RSTD reporting accuracy test cases 9.3.7.1 to 9.3.12.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.3.x.y.4.1-1 1 in TS 37.571-1 [6], where x and y represent part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 151 TDD tests: 154 |  |
| numDL-Frames | Test 1, Test 2: sf-6 Test 3, Test 4, tests 9.3.7.2, 9.3.8.2 and 9.3.9.2: sf-2 Test 3, Test 4, tests 9.3.10.2, 9.3.11.2 and 9.3.12.2: sf-4 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | ‘1111 0000’ |  |

Table 7.5.2-3: OTDOA-ReferenceCellInfo for eMTC inter-frequency RSTD reporting delay test cases 9.4.1.1 to 9.4.6.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.4.x.y.4.1-1 in TS 37.571-1 [6], where x and y represent part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 142 TDD tests: 304 |  |
| numDL-Frames | Test 1: sf-4 Test 2, tests 9.4.1.2, 9.4.2.2 and 9.4.3.2: sf-2 Test 2, tests 9.4.4.2, 9.4.5.2 and 9.4.6.2: sf-4 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | ‘11111111 00000000’ |  |

Table 7.5.2-4: OTDOA-ReferenceCellInfo for eMTC inter-frequency RSTD reporting accuracy test cases 9.4.7.1 to 9.4.12.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.4.x.y.4.1-1 1 in TS 37.571-1 [6], where x and y represent part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | 142 |  |
| numDL-Frames | Test 1: sf-4 Test 2: sf-2 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | ‘1111 0000’ |  |

OTDOA NEIGHBOUR CELL INFO LIST:

Table 7.5.2-5: OTDOA-NeighbourCellInfoList for eMTC intra-frequency RSTD reporting delay test cases 9.3.1.1 to 9.3.6.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See tables of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see tables of Sequence data values below |  |
| earfcn | Not present | Same as for the reference cell |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD, HD-FDD: 311 TDD: 304 |  |
| numDL-Frames | Test 1: sf-6 Test 2, tests 9.3.1.2, 9.3.2.2 and 9.3.3.2: sf-2 Test 2, tests 9.3.4.2, 9.3.5.2 and 9.3.6.2: sf-4 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | See tables of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | Not present | Same as for reference cell |
| prs-SubframeOffset | Not present |  |
| expectedRSTD | See tables of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-6: Sequence data values for 15 instances of sequence for eMTC intra-frequency RSTD reporting delay test cases 9.3.1.1 to 9.3.6.2

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value po8-r9 | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |
| Cell 2 | 6 (Note 1) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘0000 1111’ | 8222 | Note 2 |
| Cell 3 | 12 (Note 1) | '0000 0000 0000 0000 0010'B | ‘0000 1100’B | ‘1111 0000’ | 8222 | Note 3 |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘0000 1111’ | 8162 | Note 4 |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘1111 0000’ | 8218 | Note 4 |
| Dummy cell | 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘0000 1111’ | 8211 | Note 4 |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘1111 0000’ | 8175 | Note 4 |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘1111 0000’ | 8190 | Note 4 |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘0000 1111’ | 8200 | Note 4 |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘1111 0000’ | 8182 | Note 4 |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘0000 1111’ | 8207 | Note 4 |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘0000 1111’ | 8182 | Note 4 |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘1111 0000’ | 8218 | Note 4 |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘0000 1111’ | 8182 | Note 4 |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘1111 0000’ | 8192 | Note 4 |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘0000 1111’ | 8162 | Note 4 |
| Note 1: Set according to sub-clause 4.7.1 and Table 9.3.x.y.4.1-1 in TS 37.571-1 [6] Note 2: Data for cell 2 is used at a random position in the first 7 instances of the sequence Note 3: Data for cell 3 is used at a random position in the final 8 instances of the sequence Note 4: Data for this cell is used at any position in the 15 instances of the sequence |  |  |  |  |  |  |

Table 7.5.2-7: OTDOA-NeighbourCellInfoList for eMTC intra-frequency RSTD reporting accuracy test cases 9.3.7.1 to 9.3.12.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See table of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see table of Sequence data values below |  |
| earfcn | Not present | Same as for the reference cell |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 151 TDD tests: 154 |  |
| numDL-Frames | Test 1, 2: sf-6 Test 3, Test 4, tests 9.3.7.2, 9.3.8.2 and 9.3.9.2: sf-2 Test 3, Test 4, tests 9.3.10.2, 9.3.11.2 and 9.3.12.2: sf-4 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | See table of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |
| prs-SubframeOffset | Not present |  |
| expectedRSTD | See table of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-8: Sequence data values for 15 instances of sequence for eMTC intra-frequency RSTD reporting accuracy test cases 9.3.7.1 to 9.3.12.2

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value po8-r9 | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |
| Cell 2 (Test 1) | 6 (Note) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘1111 0000’ | 8202 |  |
| Cell 2 (Test 2) | 7 (Note) | '0000 0000 0000 0000 0110'B | ‘0000 0111’B | ‘1111 0000’ | 8182 |  |
| Cell 2 (Test 3) | 6 (Note) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘1111 0000’ | 8182 |  |
| Cell 2 (Test 4) | 9 (Note) | '0000 0000 0000 0000 0100'B | ‘0000 1001’B | ‘1111 0000’ | 8202 |  |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘0000 1111’ | 8162 |  |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘1111 0000’ | 8218 |  |
| Dummy cell | 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘0000 1111’ | 8211 |  |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘1111 0000’ | 8175 |  |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘1111 0000’ | 8190 |  |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘0000 1111’ | 8200 |  |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘1111 0000’ | 8182 |  |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘0000 1111’ | 8207 |  |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘0000 1111’ | 8182 |  |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘1111 0000’ | 8218 |  |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘0000 1111’ | 8182 |  |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘1111 0000’ | 8192 |  |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘0000 1111’ | 8162 |  |
| Dummy cell | 126 | '0000 0000 0000 0000 1100'B | ‘0111 1110’B | ‘1111 0000’ | 8208 |  |
| Note: Set according to sub-clause 4.7.1 and Table 9.3.x.y.4.1-1 in TS 37.571-1 [6] |  |  |  |  |  |  |

Table 7.5.2-9: OTDOA-NeighbourCellInfoList for eMTC inter-frequency RSTD reporting delay test cases 9.4.1.1 to 9.4.6.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See tables of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see tables of Sequence data values below |  |
| earfcn | 2 |  |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | 152 |  |
| numDL-Frames | Test 1: sf-4 Test 2, tests 9.4.1.2, 9.4.2.2 and 9.4.3.2: sf-2 Test 2, tests 9.4.4.2, 9.4.5.2 and 9.4.6.2: sf-4 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po16-r9 | See tables of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | 0 |  |
| prs-SubframeOffset | 10 |  |
| expectedRSTD | See tables of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-10: Sequence data values for 15 instances of sequence for eMTC inter-frequency RSTD reporting delay test cases 9.4.1.1 to 9.4.6.2

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value po16-r9 | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |
| Cell 2 | 6 (Note 1) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘00000000 11111111’ | 8222 | Note 2 |
| Cell 3 | 12 (Note 1) | '0000 0000 0000 0000 0010'B | ‘0000 1100’B | ‘11111111 00000000’ | 8222 | Note 3 |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘00000000 11111111’ | 8162 | Note 4 |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘11111111 00000000’ | 8218 | Note 4 |
| Dummy cell | 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘00000000 11111111’ | 8211 | Note 4 |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘11111111 00000000’ | 8175 | Note 4 |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘00000000 11111111’ | 8190 | Note 4 |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘11111111 00000000’ | 8200 | Note 4 |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘00000000 11111111’ | 8182 | Note 4 |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘11111111 00000000’ | 8207 | Note 4 |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘00000000 11111111’ | 8182 | Note 4 |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘11111111 00000000’ | 8218 | Note 4 |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘00000000 11111111’ | 8182 | Note 4 |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘11111111 00000000’ | 8192 | Note 4 |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘00000000 11111111’ | 8162 | Note 4 |
| Note 1: Set according to sub-clause 4.7.1 and Table 9.3.x.y.4.1-1 in TS 37.571-1 [6] Note 2: Data for cell 2 is used at a random position in the first 7 instances of the sequence Note 3: Data for cell 3 is used at a random position in the final 8 instances of the sequence Note 4: Data for this cell is used at any position in the 15 instances of the sequence |  |  |  |  |  |  |

Table 7.5.2-11: OTDOA-NeighbourCellInfoList for eMTC inter-frequency RSTD reporting accuracy test cases 9.4.7.1 to 9.4.12.2

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See table of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see table of Sequence data values below |  |
| earfcn | 2 |  |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | 152 |  |
| numDL-Frames | Test 1: sf-4 Test 2: sf-2 |  |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | See table of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | 0 |  |
| prs-SubframeOffset | 10 |  |
| expectedRSTD | See table of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-12: Sequence data values for 15 instances of sequence for eMTC inter-frequency RSTD reporting accuracy test cases 9.4.7.1 to 9.4.12.2

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value po8-r9 | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |
| Cell 2 (Test 1) | 6 (Note) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘1111 0000’ | 8202 |  |
| Cell 2 (Test 2) | 6 (Note) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘1111 0000’ | 8182 |  |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘1111 0000’ | 8162 |  |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘1111 0000’ | 8218 |  |
| Dummy cell | 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘0000 1111’ | 8211 |  |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘1111 0000’ | 8175 |  |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘0000 1111’ | 8190 |  |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘1111 0000’ | 8200 |  |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘1111 0000’ | 8182 |  |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘0000 1111’ | 8207 |  |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘1111 0000’ | 8182 |  |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘0000 1111’ | 8218 |  |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘0000 1111’ | 8182 |  |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘1111 0000’ | 8192 |  |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘0000 1111’ | 8162 |  |
| Dummy cell | 126 | '0000 0000 0000 0000 1100'B | ‘0111 1110’B | ‘1111 0000’ | 8208 |  |
| Note: Set according to sub-clause 4.7.1 and Table 9.4.x.y.4.1-1 in TS 37.571-1 [6] |  |  |  |  |  |  |

OTDOA REFERENCE CELL INFO:

Table 7.5.2-13: OTDOA-ReferenceCellInfo for eMTC intra-frequency RSTD reporting delay test cases 9.3.13 to 9.3.15

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.3.x.4.1-1 in TS 37.571-1 [6], where x represents part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n6 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 311 TDD tests: 304 |  |
| add-numDL-Frames | sf-12 | UE with additional-prs-config capability |
|  | sf-20 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | ‘1111 0000’ |  |

Table 7.5.2-14: OTDOA-ReferenceCellInfo for eMTC intra-frequency RSTD reporting delay test cases 9.3.16 to 9.3.18

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.3.x.4.1-1 in TS 37.571-1 [6], where x represents part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | Test 1: n6 Test 2: n25 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 311 TDD tests: 304 |  |
| add-numDL-Frames | Test 1: sf-30 Test 2: sf-8 | UE with additional-prs-config capability |
|  | Test 1: sf-40 Test 2: sf-10 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | ‘1111 0000’ |  |

OTDOA NEIGHBOUR CELL INFO LIST:

Table 7.5.2-15: OTDOA-NeighbourCellInfoList for eMTC intra-frequency RSTD reporting delay test cases 9.3.13 to 9.3.15

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See tables of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see tables of Sequence data values below |  |
| earfcn | Not present | Same as for the reference cell |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n6 |  |
| prs-ConfigurationIndex | FDD, HD-FDD: 311 TDD: 304 |  |
| add-numDL-Frames | sf-12 | UE with additional-prs-config capability |
|  | sf-20 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | See tables of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | Not present | Same as for reference cell |
| prs-SubframeOffset | Not present |  |
| expectedRSTD | See tables of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-15: OTDOA-NeighbourCellInfoList for eMTC intra-frequency RSTD reporting delay test cases 9.3.16 to 9.3.18

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See tables of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see tables of Sequence data values below |  |
| earfcn | Not present | Same as for the reference cell |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | Test 1: n6 Test 2: n25 |  |
| prs-ConfigurationIndex | FDD, HD-FDD: 311 TDD: 304 |  |
| add-numDL-Frames | Test 1: sf-30 Test 2: sf-8 | UE with additional-prs-config capability |
|  | Test 1: sf-40 Test 2: sf-10 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | See tables of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | Not present | Same as for reference cell |
| prs-SubframeOffset | Not present |  |
| expectedRSTD | See tables of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-16: Sequence data values for 15 instances of sequence for eMTC intra-frequency RSTD reporting delay test cases 9.3.13 to 9.3.18

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value po8-r9 | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |
| Cell 2 | 6 (Note 1) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘0000 1111’ | 8222 | Note 2 |
| Cell 3 | 12 (Note 1) | '0000 0000 0000 0000 0010'B | ‘0000 1100’B | ‘1111 0000’ | 8222 | Note 3 |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘0000 1111’ | 8162 | Note 4 |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘1111 0000’ | 8218 | Note 4 |
| Dummy cell | 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘0000 1111’ | 8211 | Note 4 |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘1111 0000’ | 8175 | Note 4 |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘1111 0000’ | 8190 | Note 4 |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘0000 1111’ | 8200 | Note 4 |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘1111 0000’ | 8182 | Note 4 |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘0000 1111’ | 8207 | Note 4 |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘0000 1111’ | 8182 | Note 4 |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘1111 0000’ | 8218 | Note 4 |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘0000 1111’ | 8182 | Note 4 |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘1111 0000’ | 8192 | Note 4 |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘0000 1111’ | 8162 | Note 4 |
| Note 1: Set according to sub-clause 4.7.1 and Table 9.3.x.4.1-1 in TS 37.571-1 [6] Note 2: Data for cell 2 is used at a random position in the first 7 instances of the sequence Note 3: Data for cell 3 is used at a random position in the final 8 instances of the sequence Note 4: Data for this cell is used at any position in the 15 instances of the sequence |  |  |  |  |  |  |

OTDOA REFERENCE CELL INFO:

Table 7.5.2-17: OTDOA-ReferenceCellInfo for eMTC inter-frequency RSTD reporting delay test cases 9.4.13 to 9.4.15

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.3.x.4.1-1 in TS 37.571-1 [6], where x represents part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 232 TDD tests: 212 |  |
| add-numDL-Frames | sf-12 | UE with additional-prs-config capability |
|  | sf-20 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po16-r9 | ‘11111111 00000000’ |  |

Table 7.5.2-18: OTDOA-ReferenceCellInfo for eMTC inter-frequency RSTD reporting delay test cases 9.4.16 to 9.4.18

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-ReferenceCellInfo |  | Cell 1 |
| physCellId | 0 | Set according to sub-clause 4.7.1 and Table 9.3.x.4.1-1 in TS 37.571-1 [6], where x represents part of the test case number |
| cellGlobalId | cellidentity (E-UTRAN Cell Identity): eNB ID: '0000 0000 0000 0000 0001'B Cell Identity: '0000 0000'B |  |
| earfcnRef | Not present | Same as the serving cell |
| antennaPortConfig | Not present | Same as the serving cell |
| cpLength | Normal |  |
| prsInfo SEQUENCE |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD and HD-FDD tests: 232 TDD tests: 532 |  |
| add-numDL-Frames | Test 1: sf-30 Test 2: sf-8 | UE with additional-prs-config capability |
|  | Test 1: sf-40 Test 2: sf-10 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po16-r9 | ‘11111111 00000000’ |  |

OTDOA NEIGHBOUR CELL INFO LIST:

Table 7.5.2-19: OTDOA-NeighbourCellInfoList for eMTC inter-frequency RSTD reporting delay test cases 9.4.13 to 9.4.15

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See tables of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see tables of Sequence data values below |  |
| earfcn | 2 |  |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | 252 |  |
| add-numDL-Frames | sf-12 | UE with additional-prs-config capability |
|  | sf-20 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po16-r9 | See tables of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | Not present | Same as for reference cell |
| prs-SubframeOffset | Not present |  |
| expectedRSTD | See tables of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-20: OTDOA-NeighbourCellInfoList for eMTC inter-frequency RSTD reporting delay test cases 9.4.16 to 9.4.18

| Information Element | Value/remark | Comment |
| --- | --- | --- |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE |  |  |
| SEQUENCE (SIZE(15)) OF SEQUENCE | Sequence contains 15 instances of the following data. |  |
| physCellId | See tables of Sequence data values below |  |
| cellGlobalId | For values of cellidentity see tables of Sequence data values below |  |
| earfcn | 2 |  |
| cpLength | Not present | Same as for the reference cell |
| prsInfo |  |  |
| prs-Bandwidth | n50 |  |
| prs-ConfigurationIndex | FDD, HD-FDD: 312 TDD: 612 |  |
| add-numDL-Frames | Test 1: sf-30 Test 2: sf-8 | UE with additional-prs-config capability |
|  | Test 1: sf-40 Test 2: sf-10 | UE with densePrsConfig capability |
| prs-MutingInfo-r9 CHOICE |  |  |
| po8-r9 | See tables of Sequence data values below |  |
| antennaPortConfig | Not present | Same as for the reference cell |
| slotNumberOffset | Not present | Same as for reference cell |
| prs-SubframeOffset | Not present |  |
| expectedRSTD | See tables of Sequence data values below |  |
| expectedRSTD-Uncertainty | 51 | About 5 s |

Table 7.5.2-21: Sequence data values for 15 instances of sequence for eMTC inter-frequency RSTD reporting delay test cases 9.4.13 to 9.4.18

| Cell | Value physCellId | Value cellidentity (E-UTRAN Cell Identity) |  | Value po16-r9 | Value expectedRSTD | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value eNB ID | Value Cell Identity |  |  |  |
| Cell 2 | 6 (Note 1) | '0000 0000 0000 0000 0100'B | ‘0000 0110’B | ‘00000000 11111111’ | 8212 | Note 2 |
| Cell 3 | 12 (Note 1) | '0000 0000 0000 0000 0010'B | ‘0000 1100’B | ‘11111111 00000000’ | 8172 | Note 3 |
| Dummy cell | 1 | '0000 0000 0000 0000 0001'B | '0000 0001'B | ‘00000000 11111111’ | 8162 | Note 4 |
| Dummy cell | 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | ‘11111111 00000000’ | 8218 | Note 4 |
| Dummy cell | 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | ‘00000000 11111111’ | 8211 | Note 4 |
| Dummy cell | 8 | '0000 0000 0000 0000 0010'B | '0000 1000'B | ‘11111111 00000000’ | 8175 | Note 4 |
| Dummy cell | 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | ‘11111111 00000000’ | 8190 | Note 4 |
| Dummy cell | 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | ‘00000000 11111111’ | 8200 | Note 4 |
| Dummy cell | 16 | '0000 0000 0000 0000 0010'B | '0001 0000'B | ‘11111111 00000000’ | 8182 | Note 4 |
| Dummy cell | 111 | '0000 0000 0000 0000 1100'B | '0110 1111'B | ‘00000000 11111111’ | 8207 | Note 4 |
| Dummy cell | 118 | '0000 0000 0000 0000 1111'B | ‘0111 0110’B | ‘00000000 11111111’ | 8182 | Note 4 |
| Dummy cell | 119 | '0000 0000 0000 0000 1110'B | ‘0111 0111’B | ‘11111111 00000000’ | 8218 | Note 4 |
| Dummy cell | 120 | '0000 0000 0000 0000 1111'B | ‘0111 1000’B | ‘00000000 11111111’ | 8182 | Note 4 |
| Dummy cell | 122 | '0000 0000 0000 0000 1010'B | ‘0111 1010’B | ‘11111111 00000000’ | 8192 | Note 4 |
| Dummy cell | 125 | '0000 0000 0000 0000 1011'B | ‘0111 1101’B | ‘00000000 11111111’ | 8162 | Note 4 |
| Note 1: Set according to sub-clause 4.7.1 and Table 9.4.x.4.1-1 in TS 37.571-1 [6] Note 2: Data for cell 2 is used at a random position in the first 7 instances of the sequence Note 3: Data for cell 3 is used at a random position in the final 8 instances of the sequence Note 4: Data for this cell is used at any position in the 15 instances of the sequence |  |  |  |  |  |  |

< End of changes >