# Title:	Test tolerance methodology for SA NR - UTRAN FDD handover tests

# Source:	Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for SA NR - UTRAN FDD handover tests.

The test cases in question, from TS 38.533, are:

- 6.3.1.6 SA NR - UTRAN FDD handover

This document describes the process to derive the test tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extract from TS 38.133 v16.9.0.

#### A.6.3.1.6	 SA NR - UTRAN FDD handover

##### A.6.3.1.6.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT UTRAN FDD handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to UTRAN FDD handover requirements as specified in clause 6.1.2.2.1.

The test comprises of one NR carrier and one UTRA FDD carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.6-1. General test parameters are provided in Table A.6.3.1.6-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.6-3 and A.6.3.1.6-4 respectively.

Table A.6.3.1.6-1: Supported test configurations for SA inter-RAT UTRAN FDD handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.6-2: General test parameters for SA inter-RAT UTRAN FDD handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  |  | 2 | 1 UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| Inter-RAT (UTRAN FDD) measurement quantity |  |  | CPICH Ec/N0 |  |
| b2-Threshold1 |  | dBm | As specified in Table A.6.3.1.6-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2-UTRA |  | dB | -18 | Absolute UTRAN CPICH Ec/Io threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.6-3: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3 | 1 |  |  |
| Duplex mode |  |  | 1 | FDD |  |  |
|  |  |  | 2, 3 | TDD |  |  |
| TDD Configuration |  |  | 2 | TDDConf.1.1 |  |  |
|  |  |  | 3 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1 | SR.1.1 FDD |  |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1 | CR.1.1 FDD |  |  |
|  |  |  | 2 | CR.1.1 TDD |  |  |
|  |  |  | 3 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  |  |
|  |  |  | 2 | TRS.1.1 TDD |  |  |
|  |  |  | 3 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |  |
|  |  |  | 3 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -96 |  |  |
|  |  |  | 3 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3 | -100 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, | -100 |  |  |
|  |  |  | 3 | -97 |  |  |
| Ês/Noc |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -88 | -104 | -104 |
|  |  |  | 3 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -59.78 | -70.59 | -70.59 |
|  |  | dBm/38.16 MHz | 3 | -53.68 | -64.49 | -64.49 |
| Propagation condition |  |  | 1, 2, 3 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.6-4: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| DCH_Ec/Ior | dB | N/A | N/A | Note 1 |
| OCNS_Ec/Ior | dB | -0.941 | 0.941 | Note 2 |
|  | dB | -infinity | -1.8 | -1.8 |
|  | dBm/3,84 MHz | -70 | -70 | -70 |
| CPICH_Ec/Io | dB | -infinity | -14 | -14 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior . |  |  |  |  |

##### A.6.3.1.6.2	Test Requirements

The UE shall start to transmit the UL DPCCH to Cell 2 less than 190 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms, which is specified in clause 5.3.1.1.1.

Tinterrupt = 140 ms in the test; Tinterrupt is defined in clause 5.3.1.1.2. 	This gives a total of 190 ms.

# 3. 	Discussion

The test case consists of 3 test configurations. Each test configuration has 3 time periods: T1, T2 and T3. The distribution of power is identical for all 3 test configurations.

During T1 only NR Cell 1 is detectable. the nominal Ês/Iot value for NR Cell 1 will be 12 dB and the nominal SSB_RP value shall be -88 dBm/15kHz (-85 dBm/30kHz).

During T2 both NR Cell 1 and UTRA Cell 2 are present. UE is configured an measurement event B2 on UTRA RF channel and UE starts to perform measurement on UTRA Cell 2. The nominal Ês/Iot and SSB_RP value for NR Cell 1 will be -4 dB and -100 dBm/15kHz (-107 dBm/30kHz) respectively. The nominal CPICH_Ec/Io, SCH_Ec/Io, CPICH RSCP and Io value for UTRA Cell 2 will be -14 dB, -16 dB, -81.80 dBm/3.84MHz, and -67.80 dBm/3.84MHz respectively. So the side conditions for NR intra-frequency measurement and UTRA inter-RAT measurement need to be satisfied.

Signal levels in T3 are same with T2. So the nominal Ês/Iot and SSB_RP value for NR Cell 1, the nominal CPICH_Ec/Io, SCH_Ec/Io, CPICH RSCP and Io value for UTRA Cell 2 are also the same. During T3 UE is sent an RRC message which triggers UTRA inter-RAT handover. So the side conditions for UTRA inter-RAT handover need to be satisfied.

With nominal signal levels the side conditions for measurements and handover are easily met, but the effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

In the accompanying spreadsheet only configurations 1 and 3 are considered. For other configurations, critical test parameters are identical with test configuration 1 and 3, thus the same level uncertainties and method of deciding test tolerances can be used.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.3.1.6.1-1, Table A.6.3.1.6.1-2, Table A.6.3.1.6.1-3 and Table A.6.3.1.6.1-4 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

The key parameters are:

- Noc (in dBm/15kHz): The noise EPRE on NR RF channel;

- Es/Noc (in dB): The ratio of signal EPRE of NR over noise EPRE on NR RF channel;

- Ioc (in dBm/3.84MHz): PSD of the BW-limited Noise on UTRA RF channel;

- Ior/Ioc (in dB): The received PSD of the downlink signal on UTRA RF channel;

- CPICH Ec/Ior (in dB): The ratio of CPICH chip energy to Ior;

- SCH_Ec/Ior (in dB): The ratio of SCH chip energy to Ior;

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case at the start of T2, UTRA Cell 2 becomes detectable and the UE is expected to detect UTRA Cell 2 and to send a MeasurementReport message triggered by event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2). So the UE shall perform SS-RSRP and CPICH Ec/Io measurements on NR Cell 1 and UTRA Cell 2 respectively. Then it will compare the results to b2-Threshold1 and b2-Threshold2UTRA-FDD respectively. A further step is done to calculate SS-RSRP difference and CPICH Ec/Io difference:

- (SS-RSRP – b2-Threshold1)

- (CPICH Ec/Io – b2-Threshold2UTRA-FDD)

Furthermore, SSB Es/Iot, SSB_RP, Io of NR Cell 1, CPICH_Ec/Io, SCH_Ec/Io, CPICH_RSCP, (Io/Ior – CPICH_Ec/Ior) and Io of UTRA Cell 2 are calculated, as these are side conditions for the measurement accuracy.

c) Uncertainties

The SS provides AWGN, 1 NR cell and 1 UTRA cell to the UE. We propose to control the following parameters:

- AWGN absolute power on NR RF channel, Noc ±1.5 dB;

- AWGN absolute power of UTRA RF channel, Ioc ±0.7 dB;

- Ratio of NR Cell 1 signal / AWGN for NR Cell 1, Ês/Noc ±0.3 dB

- Ratio of UTRA Cell 2 signal / AWGN, Îor / Ioc ± 0.3dB

- Ratio of UTRA Cell 2 code level / Ior , Ec / Ior ± 0.1dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

For this test case the signals on UTRA RF channel is not faded, and the values for Ês / Noc and Îor / Ioc are chosen to be similar to equivalent parameters in UTRA. The absolute level uncertainty of Noc for cell 1 is chosen to ensure the analysis and Test Requirements are valid for frequency bands up to 4.2GHz. The absolute levels of Ioc is specified as ± 0.7dB, similar to the uncertainty for other absolute power values such as RefSens, and UTRA bands above 3GHz are not within scope. The value for Ec / Ior is specified as ± 0.1dB, similar to the uncertainty in UTRA FDD test specification TS 34.121-1.

The Test system uncertainties appear in section c) of the accompanying spreadsheet.

In addition, the UE SS-RSRP and CPICH_Ec/Io absolute measurement accuracy need to be considered, because when the UE makes the Cell 1 SS-RSRP and Cell 2 CPICH Ec/Io comparison with the b2-Threshold1 and b2-Threshold2UTRA-FDD respectively, it has some inaccuracy which could affect the test verdict. The UE measurement accuracy is defined in the following extract from TS 38.133:

#### 10.1.2.1	Intra-frequency SS-RSRP accuracy requirements

##### 10.1.2.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 is configured.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

## B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### 10.3.2	UTRAN FDD CPICH Ec/No

NOTE:	This measurement is for handover between E-UTRAN and UTRAN FDD.

The requirements in this clause are valid for terminals supporting this capability.

The measurement period for RRC_CONNECTED state is specified in clause 9.4.6.

In RRC_CONNECTED state the accuracy requirements shall be the same as the inter-frequency measurement accuracy requirements for FDD CPICH Ec/No in TS 25.133 [29].

If the UE, in RRC_CONNECTED state, needs measurement gaps to perform UTRAN FDD measurements, the UTRAN FDD measurement procedure and measurement gap pattern stated in clause 9.4.6 shall apply.

The reporting range and mapping specified for FDD CPICH Ec/No in TS 25.133 [29] shall apply.

#### 9.1.2.2	Inter frequency measurement accuracy

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.3. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.3.

##### 9.1.2.2.1	Absolute accuracy requirement

The accuracy requirements in table 9.7 are valid under the following conditions:

# CPICH_RSCP1|dBm according to Annex B.3.6 for a corresponding Band

# Table 9.7: CPICH_Ec/Io Inter frequency absolute accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| CPICH_Ec/Io | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum Io is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

## B.3.6. Conditions for inter frequency CPICH Ec/Io measurements accuracy

This section defines the inter frequency CPICH RSCP applicable for a corresponding operating band.

The CPICH RSCP conditions for measurements accuracy of inter-frequency CPICH Ec/Io are defined in Table B.3.1-1.

## B.3.1. Conditions for intra frequency CPICH RSCP measurements accuracy

This section defines the intra frequency CPICH RSCP applicable for a corresponding operating band.

The CPICH RSCP conditions for measurements accuracy of intra-frequency CPICH RSCP are defined in Table B.3.1-1

# Table B.3.1-1: Conditions for measurements of intra-frequency CPICH RSCP

| Parameter | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bands | Bands | Bands | Bands | Bands |
|  | I, IV, VI, X, XI, XIX, XXI, XXXII | IX | II, V, VII | III, VIII, XII, XIII, XIV, XX, XXII | XXV, XXVI |
| CPICH RSCP1¦dBm | -114 dBm | -113 dBm | -112 dBm | -111 dBm | -110.5dBm (Note 1) |
| Note 1: The condition is -112 dBm when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

In addition, NR Cell 1 and UTRA Cell 2 shall be detectable in T2 to ensure NR intra-frequency measurement and UTRA inter-RAT measurement requirements defined in TS 38.133 cl.9.2.5.1 and 9.4.6.2 apply.

### 9.2.2	Requirements applicability

The requirements in clause 9.2 apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

#### 9.4.6.2	Requirements when no DRX is used

9.4.6.2.1	Identification of a new UTRA FDD cell

When explicit neighbour list is provided and no DRX is used, either measurement gaps are scheduled or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable cell belonging to the monitored set within

A cell shall be considered detectable when

-	CPICH Ec/Io > -20 dB,

-	SCH_Ec/Io > -17 dB for at least one channel tap and SCH_Ec/Ior is equally divided between primary synchronisation code and secondary synchronisation code. When L3 filtering is used an additional delay can be expected.

In summary,

- Es/Iot: Es/Iot shall be no less than -6dB;

- SSB_RP: BG NR_FDD_FR1_H has the tightest requirement. SSB_RP shall be no less than -123.5 dBm/15kHz (120.5 dBm/30kHz);

- Io: BG NR_FDD_FR1_H has the tightest requirement. Io shall be no less than -70dBm/Ch BW and no higher than -50dBm/Ch BW.

- (Io/Ior - CPICH Ec/Ior): (Io/Ior - CPICH Ec/Ior) shall be no higher than 20dB;

- CPICH Ec/Io: CPICH Ec/Io is no less than -16dB;

- Io: Io requirements for Band XXV is the tightest. Io shall be no less than -90.5 dBm/3.84MHz and no higher than -50 dBm/3.84MHz;

- CPICH RSCP: CPICH RSCP shall be no less than -110.5dBm;

- SCH_Ec/Io: SCH_Ec/Io shall be no less than -17 dB.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in downlink signal: The uncertainties in downlink signal should be within Es/Iot range, SSB_RP range and Io range for NR Cell 1, and should be within Io range, CPICH Ec/Io range, CPICH RSCP range and SCH_Ec/Io range for UTRA Cell 2 to ensure that NR intra-frequency measurement and UTRA inter-RAT measurement requirements apply.

- The uncertainties in UE NR intra-frequency measurement and UTRA CPICH Ec/Io measurement: The uncertainties in UE measurement and reporting should be constrained to ensure B2 event can be correctly triggered.

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

In general, all the values are listed for each time interval T1 and T2 but as Cell 2 is not detectable during T1 those entries are greyed out. Analysis is performed for configuration 1 (BW 10MHz, SCS 15kHz, 52 RBs) and configuration 3 (BW 40MHz, SCS 30kHz, 106 RBs), the shaded cells, in the spreadsheet, show what changes with configuration.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Denoting critical parameter y as a function of test equipment uncertainties x1, x2, ..., i.e. y=fx1,x2,⋯. Then sensitivity factors of xi on y can be expressed as:

sensitivity factors of xi on y=∂y∂xi

Then we have:

| controlled parameters | Key parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Noc | Ês/Noc | Ioc | Ior/Ioc | CPICH Ec/Ior | SCH Ec/Ior |
| CPICH Ec/Io, Cell 2 | 0 | 0 | 0 | 0.602 | 1 | 0 |
| SCH Ec/Io, Cell 2 | 0 | 0 | 0 | 0.602 | 0 | 1 |
| CPICH RSCP, Cell 2 | 0 | 0 | 1 | 1 | 1 | 0 |
| Io/Ior - CPICH Ec/Ior, Cell 2 | 0 | 0 | 0 | 0.602 | 1 | 0 |
| Io, UTRA RF channel | 0 | 0 | 1 | 0.398 | 0 | 0 |
| Es/Iot, Cell 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| SSB_RP, Cell 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| Io, NR RF channel | 1 | 0.941 for T1 0.285 for T2/T3 | 0 | 0 | 0 | 0 |

Table 1. sensitivity factors of key parameters on controlled parameters

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the CPICH Ec/Io measurement absolute accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the SS-RSRP/CPICH Ec/Io measurement absolute accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the SS-RSRP/CPICH Ec/Io measurement absolute accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T2, UTRA Cell 2 becomes detectable. UE is expected to perform NR SS-RSRP and UTRA CPICH Ec/Io measurement on Cell 1 and Cell 2 respectively. The Event B2 measurement report trigger requirement in TS 38.331 clause 5.5.4.9 is “Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2), inequality B2-1 (Entering condition 1):

Mp + Hys < Thresh1

and inequality B2-2 (Entering condition 2):

Mn + Ofn + Ocn – Hys > Thresh2

The values in these equations are derived as follows:

- Mn is the CPICH Ec/Io measurement result of UTRA Cell 2 at T2. It's nominally -14 dB;

- Mp is the SS-RSRP measurement result of NR Cell 1 at T2. It's nominally -100 dBm/15kHz (-97 dBm/30kHz);

- Ofn is given by utra-FDD-Q-OffsetRange as defined within MO corresponding to UTRA Cell 2. It's 0dB according to TS 38.508-1 Table 4.6.3-77A;

- Ocn is 0dB since no cell-specific offset is applicable for UTRA Cell according to 38.331 cl.6.3.2.

- Hys is the hysteresis parameter for this event. It's specified as 0dB in TS 38.133 Table A.6.3.1.6.1-2;

- Thresh1 is event threshold for NR Cell 1. It's specified as -96 dBm for 15kHz (-93 dBm for 30kHz) in TS 38.133 Table A.6.3.1.6.1-2.

- Thresh2 is event threshold for UTRA Cell 2. It's specified as -18 dB in TS 38.133 Table A.6.3.1.6-2.

Then the equations therefore can be reduced and rearranged to:

Mp (-100 dBm) – Thresh1 (-96 dBm) < 0

and

Mn (-14 dB) – Thresh2 (-18 dB) > 0

So entering condition of Event B2 is satisfied under nominal conditions.

After taking into account of both TE uncertainties and UE measurement absolute accuracy the overall uncertainty for UE measured (Cell 1 SS-RSRP - Thresh1) and (Cell 2 CPICH Ec/Io - Thresh2) is ±9.530dB and ±2.206dB as calculated in step d) of the accompanying spreadsheet. One can observe that:

- During T2, the (Ior/Ioc - CPICH Ec/Ior), CPICH Ec/Io, Io, CPICH RSCP and SCH Ec/Io of UTRA Cell 2 are fulfilling the required side conditions taking into account the uncertainty of ±2.206dB. Furthermore, Entering condition 2 for event B2 is satisfied taking into account the uncertainty. So no offset is needed for UTRA Cell related key parameters.

- During T2, the Es/Iot, SSB_RP and Io of NR Cell 1 are fulfilling the required side conditions taking into account the uncertainty of ±9.530dB. However, entering condition 1 for event B2 can not always be satisfied taking into account the uncertainty. So Noc needs to be decreased to ensure SS-RSRP - Thresh1 < 0.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

For NR Cell 1, Noc needs to be decreased to ensure SS-RSRP - Thresh1 < 0. The actual offset required is 1.54 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements. It is not necessary to calculate all parameters during each time interval, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and test tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.3.1.6 also contains configurations with different subcarrier spacing and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- b2-Threshold1, section a)

- RBs in Channel bandwidth, section a) and associated comment

- SSB_RP minimum requirement, section g) and associated Source/Reference

# 6. Treatment of other test cases

N/A