3GPP TSG-RAN5 Meeting #93-e	R5-218392

# Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5392 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MPR, ACLR, SEM CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2021-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Tables 6.2.3A.1.4.1-1, 6.6.2.1A.1.4.1-1, and 6.6.2.3A.1.4.1-1 an invalid allocation is specified since it is larger than NRB of the SCC. Minimum conformance requirements for SEM for CA are not aligned to core specification Test requirement for several bandwidth sets is missing in SEM for CA test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The allocation of test IDs 10 and 13 has been corrected in Tables 6.6.2.1A.1.4.1-1, and 6.6.2.3A.1.4.1-1. Minimum conformance requirements in Tables 6.6.2.1A.0-1 and 6.6.2.1A.0-2 have been updated The test requirements in clause 6.6.2.1A.1.5 have been updated. Minimum conformance requirement in Annex F has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test IDs 10 and 13 cannot be tested. The test requirement will remain incorrect and several bandwidth sets cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3A.1.4.1, 6.6.2.1A.0, 6.6.2.1A.1.4.1, 6.6.2.1A.1.5, 6.6.2.3A.1.4.1, F3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-217563. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 6.2.3A	Maximum Power Reduction (MPR) for CA

< Unchanged sections omitted >

6.2.3A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 | N/A for this test | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.6.2.1A	Spectrum emission mask for CA

##### 6.6.2.1A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the spectrum emission mask of the UE is defined per component carrier while both component carriers are active and the requirements are specified in subclauses 6.6.2.1.3 and 6.6.2.2.3. If for some frequency spectrum emission masks of component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a component carrier spectrum emission mask overlaps with the channel bandwidth of another component carrier, then the emission mask does not apply for that frequency.

For intra-band contiguous carrier aggregation the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the aggregated channel bandwidth (Table 5.4.2A-1) For intra-band contiguous carrier aggregation the bandwidth class B and C, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.1A.0-1 and Table 6.6.2.1A.0-2 for the specified channel bandwidth.

Table 6.6.2.1A.0-1: General E-UTRA CA spectrum emission mask for Bandwidth Class B

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) |  | 50RB+50RB (19.9 MHz) |  | Measurement bandwidth |
| 0-1 | -18 | -20 | -21 |  | -21 |  | 30 kHz |
| 1-5 | -10 | -10 | -10 |  | -10 |  | 1 MHz |
| 5-9.8 | -13 | -13 | -13 |  | -13 |  | 1 MHz |
| 9.8-14.8 | -25 | -13 | -13 |  | -13 |  | 1 MHz |
| 14.8-14.95 |  | -13 | -13 |  | -13 |  | 1 MHz |
| 14.95-19.80 |  | -25 | -13 |  | -13 |  | 1 MHz |
| 19.80-19.90 |  | -25 | -25 |  | -13 |  | 1 MHz |
| 19.90-19.95 |  | -25 | -25 |  | -25 |  | 1 MHz |
| 19.95-24.80 |  |  | -25 |  | -25 |  | 1 MHz |
| 24.80-24.90 |  |  |  |  | -25 |  | 1 MHz |

Table 6.6.2.1A.0-2: General E-UTRA CA spectrum emission mask for Bandwidth Class C

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 25RB+100RB (24.95MHz) |  | 50RB+75RB (24.75 MHz) |  | 50RB+100RB (29.9 MHz) |  | 75RB+75RB (30 MHz) |  | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) |  |  | Measurement bandwidth |
| 0-1 |  | -22 |  | -22 |  | -22.5 |  | -22.5 |  | -23.5 | -24 |  |  | 30 kHz |
| 1-5 |  | -10 |  | -10 |  | -10 |  | -10 |  | -10 | -10 |  |  | 1 MHz |
| 5-24.75 |  | -13 |  | -13 |  | -13 |  | -13 |  | -13 | -13 |  |  | 1 MHz |
| 24.75-24.95 |  | -13 |  | -25 |  | -13 |  | -13 |  | -13 | -13 |  |  | 1 MHz |
| 24.95-29.75 |  | -25 |  | -25 |  | -13 |  | -13 |  | -13 | -13 |  |  | 1 MHz |
| 29.75-29.9 |  | -25 |  |  |  | -13 |  | -13 |  | -13 | -13 |  |  | 1 MHz |
| 29.9-29.95 |  | -25 |  |  |  | -25 |  | -13 |  | -13 | -13 |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  | -25 |  | -13 |  | -13 | -13 |  |  | 1 MHz |
| 30-34.85 |  |  |  |  |  | -25 |  | -25 |  | -13 | -13 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  |  | -25 |  | -25 |  | -25 | -13 |  |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  |  | -25 |  | -25 | -13 |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  |  |  |  | -25 | -13 |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  |  |  |  |  | -25 | -25 |  |  | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  |  |  |  |  | -25 |  |  | 1 MHz |

For intra-band non-contiguous carrier aggregation transmission the spectrum emission mask requirement is defined as a composite spectrum emissions mask. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. Composite spectrum emission mask is defined as follows

a)	Composite spectrum emission mask is a combination of individual sub-block spectrum emissions masks

b)	In case the sub-block consist of one component carrier the sub-lock general spectrum emission mask is defined in subclause 6.6.2.1.3

c)	If for some frequency sub-block spectrum emission masks overlap then spectrum emission mask allowing higher power spectral density applies for that frequency

d)	If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the spectrum emission mask of the UE is defined per E-UTRA band while all component carriers are active. For the E-UTRA band supporting one component carrier the requirements in subclauses 6.6.2.1.3 and 6.6.2.2.3 apply. For the E-UTRA band supporting two contiguous component carriers the requirements specified in subclause 6.6.2.1A.0 apply. If for some frequency spectrum emission masks of single component carrier and two contiguous component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency spectrum emission masks of single component carrier or two contiguous component carriers overlap then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.1A.

< Unchanged sections omitted >

##### 6.6.2.1A.1	Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA)

6.6.2.1A.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.6.2.1A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.1.4	Test description

6.6.2.1A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 | N/A for this test | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.1.4.3.

6.6.2.1A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.1.5-1 or 6.6.2.1A.1.5-2, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.1.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.1.5-1 or Table.6.6.2.1A.1.5-2 for Bandwidth Class C and Table.6.6.2.1A.1.5-3 or Table.6.6.2.1A.1.5-4 for Bandwidth Class B, as applicable.

Table 6.6.2.1A.1.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) |  | Measurement bandwidth |
| 0-1 | -20.5 | -20.5 | -21 | -21 | -22 | -22.5 |  | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 |  | 1 MHz |
| 5-24.75 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 |  | 1 MHz |
| 24.75-24.95 |  | -23.5 |  |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.5 |  |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.5 |  |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.5 |  |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.5 |  |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.5 |  | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 25RB+100RB (24.95MHz) |  | 50RB+75RB (24.75 MHz) |  | 50RB+100RB (29.9 MHz) |  | 75RB+75RB (30 MHz) |  | 75RB+100RB (34.85 MHz) |  | 100RB+100RB (39.8 MHz) |  | Measurement bandwidth |  |
| 0-1 |  | -20.2 |  | -20.2 |  | -20.7 |  | -20.7 |  | -21.7 |  | -22.2 |  | 30 kHz |  |
| 1-5 |  | -8.2 |  | -8.2 |  | -8.2 |  | -8.2 |  | -8.2 |  | -8.2 |  | 1 MHz |  |
| 5-24.75 |  | -11.2 |  | -11.2 |  | -11.2 |  | -11.2 |  | -11.2 |  | -11.2 |  | 1 MHz |  |
| 24.75-24.95 |  |  |  | -23.2 |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| 24.95-29.75 |  | -23.2 |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| 29.75-29.9 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| 29.9-29.95 |  |  |  |  |  | -23.2 |  |  |  |  |  |  |  | 1 MHz |  |
| 29.95-30 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| 30-34.85 |  |  |  |  |  |  |  | -23.2 |  |  |  |  |  | 1 MHz |  |
| 34.85-34.9 |  |  |  |  |  |  |  |  |  | -23.2 |  |  |  | 1 MHz |  |
| 34.9-35 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| 35-39.8 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| 39.8-39.85 |  |  |  |  |  |  |  |  |  |  |  | -23.2 |  | 1 MHz |  |
| 39.85-44.8 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 MHz |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-3: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands ≤ 3GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) |  | 50RB+50RB (19.9 MHz) |  | Measurement bandwidth |
| 0-1 | -16.5 | -18.5 | -19.5 |  | -19.5 |  | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 |  | -8.5 |  | 1 MHz |
| 5-9.8 | -11.5 | -11.5 | -11.5 |  | -11.5 |  | 1 MHz |
| 9.8-14.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 14.8-14.95 |  |  |  |  |  |  | 1 MHz |
| 14.95-19.80 |  | -23.5 |  |  |  |  | 1 MHz |
| 19.80-19.90 |  |  | -23.5 |  |  |  | 1 MHz |
| 19.90-19.95 |  |  |  |  | -23.5 |  | 1 MHz |
| 19.95-24.80 |  |  |  |  |  |  | 1 MHz |
| 24.80-24.90 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-4: General E-UTRA spectrum emission mask for CA Bandwidth Class B, 3GHz < E-UTRA bands ≤ 4.2GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 25RB+25RB (9.8MHz) |  | 25RB+50RB (14.95 MHz) |  |  | 25RB+75RB (19.8MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 |  | -16.2 |  | -18.2 |  |  | -19.2 | -19.2 | 30 kHz |
| 1-5 |  | -8.2 |  | -8.2 |  |  | -8.2 | -8.2 | 1 MHz |
| 5-9.8 |  | -11.2 |  | -11.2 |  |  | -11.2 | -11.2 | 1 MHz |
| 9.8-14.8 |  | -23.2 |  |  |  |  |  |  | 1 MHz |
| 14.8-14.95 |  |  |  |  |  |  |  |  | 1 MHz |
| 14.95-19.80 |  |  |  | -23.2 |  |  |  |  | 1 MHz |
| 19.80-19.90 |  |  |  |  |  |  | -23.2 |  | 1 MHz |
| 19.90-19.95 |  |  |  |  |  |  |  | -23.2 | 1 MHz |
| 19.95-24.80 |  |  |  |  |  |  |  |  | 1 MHz |
| 24.80-24.90 |  |  |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

< Unchanged sections omitted >

##### 6.6.2.3A.1	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA)

6.6.2.3A.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra band contiguous DL CA and UL CA.

6.6.2.3A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1.4	Test description

6.6.2.3A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test | Minimum Requirement in TS 36.101 |  | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- | --- |
| < Unchanged rows omitted > |  |  |  |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.1 | Same as 6.6.1 |  | Same as 6.6.1 |
| 6.6.2.1 Spectrum Emission Mask | For 1.4 MHz BW: -10 dBm / 30kHz -25dBm to -10dBm / 1MHz For 3 MHz BW: -13 dBm / 30kHz -25dBm to -10dBm / 1MHz For 5 MHz BW: -15dBm / 30kHz -25dBm to -10dBm / 1MHz For 10 MHz BW: -18dBm / 30kHz -25dBm to -10dBm / 1MHz For 15 MHz BW: -20dBm / 30kHz -25dBm to -10dBm / 1MHz For 20 MHz BW: -21dBm / 30kHz -25dBm to -10dBm / 1MHz | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 | Same as 6.6.2.1 |  | Same as 6.6.2.1 |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | For 9.8MHz BW: -18 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 14.95MHz BW: -20 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 19.8MHz BW: -21 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 19.9MHz: -21 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 24.75MHz BW: -22 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 24.95 MHz BW: -22 dBm / 30kHz -25 dBm to - 10 dBm / 1 MHz For 29.9 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 30 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 34.85 MHz BW: -23.5dBm / 30kHz -25dBm to -10dBm / 1MHz For 39.8 MHz BW: -24dBm / 30kHz -25dBm to -10dBm / 1MHz | Same as 6.6.2.1 |  | Same as 6.6.2.1 |
| 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 per CC | Same as 6.6.2.1 per CC |  | Same as 6.6.2.1 per CC |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 for each CC | Same as 6.6.2.1 |  | Same as 6.6.2.1 for each CC |
| < Unchanged rows omitted > |  |  |  |  |

< End of changes >