3GPP TSG-RAN5 Meeting #93-e	R5-218466

# Electronic Meeting, 8 – 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0417 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 5.2.3.2.1_1, 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case 5.2.3.2.1_1 is missing test index 1-10 and 1-11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing test index 1-10 and 1-11 to test case 5.2.3.2.1_1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-217109 and the result of 1 revision |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

##### 5.2.3.2.1	4Rx TDD FR1 PDSCH mapping Type A performance

5.2.3.2.1.0	Minimum conformance requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.3.2.1.0-3, Table 5.2.3.2.1.0-4, Table 5.2.3.2.1.0-5 and Table 5.2.3.2.1.0-6, with the test parameters defined in Table 5.2.3.2.1.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.3.2.1.0-1.

Table 5.2.3.2.1.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 1-7, 1-8, 1-9, 1-10, 1-11, 2-1, 2-2, 3-1, 4-1 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 4 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A performance requirements for Enhanced Receiver Type 1 under 4 receive antenna conditions. | 5-1 |

Table 5.2.3.2.1.0-2: Test Parameters for Testing

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration |  | Mapping type |  | Type A |
|  |  | k0 |  | 0 |
|  |  | Starting symbol (S) |  | 2 |
|  |  | Length (L) |  | Specific to each Reference channel |
|  |  | PDSCH aggregation factor |  | 1 |
|  |  | PRB bundling type |  | Static |
|  |  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 WB for Test 3-1 2 for other tests |
|  |  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 50, LRBs = 6 Other tests: Type 0 |
|  |  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  |  | VRB-to-PRB mapping type |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration |  | DMRS Type |  | Type 1 |
|  |  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9, 1-10, 1-11 1 for other tests |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking |  | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  |  | CSI-RS periodicity | Slots | Test 1-7, 1-10, 1-11: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  |  | CSI-RS offset | Slots | Test 1-7, 1-10, 1-11: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  |  | Frequency Occupation |  | Test 1-7, 1-10, 1-11: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  |  | 16 for Test 1-4 10 for Test 1-9 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.3.2.1.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-1.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1A | TDLB100-400 | 2x4, ULA Low | 70 | -4.1 |
| 1-2 | R.PDSCH.2-1.2 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 70 | -2.7 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 21.6 |
| 1-4 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 30 | -1.2 |
| 1-5 | R.PDSCH.2-5.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x4, ULA Low | 70 | -3.8 |
| 1-6 | R.PDSCH.2-6.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x4, ULA Low | 70 | -3.6 |
| 1-7 | R.PDSCH.2-10.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-1000 | 1x4 | 70 | 3.4 |
| 1-8 | R.PDSCH.2-11.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-5 | TDLB100-400 | 2x4, ULA Low | 70 | -4.0 |
| 1-9 | R.PDSCH.2-12.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-6 | TDLB100-400 | 2x4, ULA Low | 70 | -4.0 |
| 1-10 | R.PDSCH.2-10.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-1200 | 2x4 | 70 | 5.8 |
| 1-11 | R.PDSCH.2-10.3 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | HST-1667 | 1x4 | 70 | 6.8 |

Table 5.2.3.2.1.0-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 13.6 |
| 2-2 | R.PDSCH.2-9.1 TDD | 20 / 30 | 64QAM, 0.50 | FR1.30-4 | TDLA30-10 | 2x4, ULA Low | 70 | 13.7 |

Table 5.2.3.2.1.0-5: Minimum performance for Rank 3

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.2-2.3 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 11.1 |

Table 5.2.3.2.1.0-6: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.2-2.4 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 15.4 |

Table 5.2.3.2.1.0-7: Minimum performance for Rank 3 and EnhancedReceiver Type 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.2-2.3 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Medium A | 70 | 22.9 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.2.3.2.1.

###### 5.2.3.2.1_1	4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.2.1_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 1 and Rank 2 scenarios.

5.2.3.2.1_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward, supporting 4Rx antenna ports.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and 4Rx antenna ports.

5.2.3.2.1_1.3	Test description

5.2.3.2.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.4 for TE diagram and clause A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.3.2.1.0-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA or (EN-DC, DC bearer MCG and SCG, Connected without release On) for NSA according to TS 38.508-1 [] clause 4.5. Message contents are defined in clause 5.2.3.2.1_1.4.3.

5.2.3.2.1_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.3.2.1.0-3 and Table 5.2.3.2.1.0-4. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.3.2.1_1.3.4-1 and 5.2.3.2.1_1.3.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.3.2.1_1.3.4-1 and 5.2.3.2.1_1.3.4-2 as appropriate.

5.2.3.2.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.3.2.1_1.3.3_1	Message exceptions for SA

Table 5.2.3.2.1_1.3.3_1-1: BWP

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | 13750 | For Test 2-2 (20MHz BW, SCS 30kHz) |  |
|  | 28875 | For other tests (40MHz BW, SCS 30kHz) |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-2: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4 | n4 for tests 1-1, 1-8, 1-9 |  |
|  | wideband | wideband for test 3-1 |  |
|  | Not present | n2 for other tests |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-3: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos1 | pos1 for all tests except tests 1-1, 1-7, 1-8, 1-9 |  |
|  | Not present | pos2 for tests 1-1, 1-7, 1-8, 1-9, 1-10, 1-11 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-4: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Not present | n8 for other tests |  |
|  | n16 | n16 for test 1-4 |  |
|  | n10 | n10 for test 1-9 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-5: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| Slots20 | 1 (for CSI-RS resources 1 and 2) 2 (for CSI-RS resources 3 and 4) | Periodicity 20 slots and offset 1/2 for test 1-7, 1-10, 1-11 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-5A: CSI-RS-ResourceMapping for TRS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 | For Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 | TRS |
|  | 8 | For Tests 1-8, 1-9: l0 = 8 for CSI-RS resource 2 and 4 | TRS |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-6: CSI-FrequencyOccupation for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| nrofRBs | 52 | 52 for tests 1-7, 1-10, 1-11, 2-2 | TRS |
|  | 108 | 108 for other tests | TRS |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-7: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 163 | Only for test 2-2 |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-8: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 7 | For test 1-9 |  |
| sl20 | 5 | For test 2-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.2.1_1.3.3_1-9: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[1] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘110’B | Slot S1 for test 1-9 |

Table 5.2.3.2.1_1.3.3_1-10: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 3 entry |  | Test 1-5, Test 1-6 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 44 | Start symbol(S)=2, Length(L)=4 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 5 entry |  | Test 1-9 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 100 | Start symbol(S)=2, Length(L)=8 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 81 | Start symbol(S)=2, Length(L)=10 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[5] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

5.2.3.2.1_1.3.3_2	Message exceptions for NSA

Same as 5.2.3.2.1_1.3.3_1

5.2.3.2.1_1.3.4	Test requirement

Table 5.2.3.2.1.0-2 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.3.2.1_1.3.4-1 and Table 5.2.3.2.1_1.3.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.3.2.1_1.3.4-1: Test Requirements for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-1.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1A | TDLB100-400 | 2x4, ULA Low | 70 | -3.1 |
| 1-2 | R.PDSCH.2-1.2 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 70 | -1.7 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 22.5 |
| 1-4 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 30 | -0.3 |
| 1-5 | R.PDSCH.2-5.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x4, ULA Low | 70 | -2.8 |
| 1-6 | R.PDSCH.2-6.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x4, ULA Low | 70 | -2.6 |
| 1-7 | R.PDSCH.2-10.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-1000 | 1x4 | 70 | 4.3 |
| 1-8 | R.PDSCH.2-11.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-5 | TDLB100-400 | 2x4, ULA Low | 70 | -3.1 |
| 1-9 | R.PDSCH.2-12.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-6 | TDLB100-400 | 2x4, ULA Low | 70 | -3.1 |
| 1-10 | R.PDSCH.2-10.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLC300-1200 | 2x4 | 70 | 6.7 |
| 1-11 | R.PDSCH.2-10.3 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | HST-1667 | 1x4 | 70 | 7.7 |

Table 5.2.3.2.1_1.3.4-2: Test Requirements for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 14.6 |
| 2-2 | R.PDSCH.2-9.1 TDD | 20 / 30 | 64QAM, 0.50 | FR1.30-4 | TDLA30-10 | 2x4, ULA Low | 70 | 14.7 |

<< End of changes>>