3GPP TSG-RAN WG5 Meeting #93-e	R5-216610

Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-3 | CR | 0145 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test applicabilities for NR UE Rx-Tx time difference measurement test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN#90-e, WI of UE Conformance Test Aspects - NR Positioning Support has been approved, related contents should be introduced into 37.571-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding applicabilities of tests and additional information for NR RSTD measurements - Adding applicabilities of tests conditions for NR RSTD test cases in Table 4-12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test applicabilities for NR RSTD test cases are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4-11, Table 4-12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-1 CR 0343 TS 37.571-1 CR 0344 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table 4-11: Applicability of tests and additional information for testing for test cases in TS 37.571-1 [5] for NR

| Clause | TC Title | Release of LPP | Applicability |  | Additional Information |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions |  |  | Release RAT |  |
| 11 | NR MBS measurement requirements |  |  |  |  |  |  |  |  |  |  |
| 11.1B | MBS Measurement Reporting Delay (NR) | Rel-13 | C45nr | All NR UEs supporting UE-Assisted MBS |  |  |  |  |  | Rel-15 |  |
| 11.2B | MBS Sensitivity Measurement Accuracy (NR) | Rel-13 | C45nr | All NR UEs supporting UE-Assisted MBS |  |  |  |  |  | Rel-15 |  |
| 11.3B | MBS Nominal Measurement Accuracy (NR) | Rel-13 | C45nr | All NR UEs supporting UE-Assisted MBS |  |  |  |  |  | Rel-15 |  |
| 11.4B | MBS Dynamic Range Measurement Accuracy (NR) | Rel-13 | C45nr | All NR UEs supporting UE-Assisted MBS |  |  |  |  |  | Rel-15 |  |
| 11.5B | MBS Measurement Accuracy in Multipath (NR) | Rel-13 | C45nr | All NR UEs supporting UE-Assisted MBS |  |  |  |  |  | Rel-15 |  |
| 13 | A-GNSS minimum performance requirements |  |  |  |  |  |  |  |  |  |  |
| 13.2.1-1 | Sensitivity Coarse Time Assistance: Sub-Test 1 | Rel-9 | C01nr | All FR1 NR UEs. The UEs shall support A-GPS L1C/A only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-2 | Sensitivity Coarse Time Assistance: Sub-Test 2 | Rel-9 | C02nr | All FR1 NR UEs. The UEs shall support A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-3 | Sensitivity Coarse Time Assistance: Sub-Test 3 | Rel-12 | C03nr | All FR1 NR UEs. The UEs shall support A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-4 | Sensitivity Coarse Time Assistance: Sub-Test 4 | Rel-9 | C04nr | All FR1 NR UEs. The UEs shall support A-GPS and Modernized GPS only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-5 | Sensitivity Coarse Time Assistance: Sub-Test 5 | Rel-9 | C05nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-8 | Sensitivity Coarse Time Assistance: Sub-Test 8 | Rel-12 | C15nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-9 | Sensitivity Coarse Time Assistance: Sub-Test 9 | Rel-12 | C11nr | All FR1 NR UEs. The UEs shall support A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.1-10 | Sensitivity Coarse Time Assistance: Sub-Test 10 | Rel-12 | C12nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.1-11 | Sensitivity Coarse Time Assistance: Sub-Test 11 | Rel-12 | C17nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.1-12 | Sensitivity Coarse Time Assistance: Sub-Test 12 | Rel-12 | C27nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.2.1-13 | Sensitivity Coarse Time Assistance: Sub-Test 13 | Rel-12 | C28nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.2-1 | Sensitivity Fine Time Assistance: Sub-Test 1 | Rel-9 | C06nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS L1C/A only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C34nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS L1C/A only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-2 | Sensitivity Fine Time Assistance: Sub-Test 2 | Rel-9 | C07nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GLONASS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C35nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GLONASS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-3 | Sensitivity Fine Time Assistance: Sub-Test 3 | Rel-12 | C08nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-Galileo only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C36nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-Galileo only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-4 | Sensitivity Fine Time Assistance: Sub-Test 4 | Rel-9 | C09nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS and Modernized GPS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C37nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS and Modernized GPS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-5 | Sensitivity Fine Time Assistance: Sub-Test 5 | Rel-9 | C10nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C38nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-8 | Sensitivity Fine Time Assistance: Sub-Test 8 | Rel-12 | C16nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS/Modernized GPS and A-Galileo only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C39nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS/Modernized GPS and A-Galileo only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-9 | Sensitivity Fine Time Assistance: Sub-Test 9 | Rel-12 | C13nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C40nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.2-10 | Sensitivity Fine Time Assistance: Sub-Test 10 | Rel-12 | C14nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS/Modernized GPS and A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C41nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS/Modernized GPS and A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.2-11 | Sensitivity Fine Time Assistance: Sub-Test 11 | Rel-12 | C18nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C42nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.2.2-12 | Sensitivity Fine Time Assistance: Sub-Test 12 | Rel-12 | C29nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C43nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only, and Fine Time Assistance |  |  |  |  |  | Rel-15 |  |
| 13.2.2-13 | Sensitivity Fine Time Assistance: Sub-Test 13 | Rel-12 | C30nr | All FR1 NR UEs supporting EN-DC. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
|  |  | Rel-15 | C44nr | All FR1 NR UEs supporting NG-RAN NR. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only, and Fine Time Assistance (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.3-1 | Nominal Accuracy: Sub-Test 1 | Rel-9 | C01nr | All FR1 NR UEs. The UEs shall support A-GPS L1C/A only |  |  |  |  |  | Rel-15 |  |
| 13.3-2 | Nominal Accuracy: Sub-Test 2 | Rel-9 | C02nr | All FR1 NR UEs. The UEs shall support A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.3-3 | Nominal Accuracy: Sub-Test 3 | Rel-12 | C03nr | All FR1 NR UEs. The UEs shall support A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.3-4 | Nominal Accuracy: Sub-Test 4 | Rel-9 | C04nr | All FR1 NR UEs. The UEs shall support A-GPS and Modernized GPS only |  |  |  |  |  | Rel-15 |  |
| 13.3-5 | Nominal Accuracy: Sub-Test 5 | Rel-9 | C05nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.3-8 | Nominal Accuracy: Sub-Test 8 | Rel-12 | C15nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.3-9 | Nominal Accuracy: Sub-Test 9 | Rel-12 | C11nr | All FR1 NR UEs. The UEs shall support A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.3-10 | Nominal Accuracy: Sub-Test 10 | Rel-12 | C12nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.3-11 | Nominal Accuracy: Sub-Test 11 | Rel-12 | C17nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.3-12 | Nominal Accuracy: Sub-Test 12 | Rel-12 | C27nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.3-13 | Nominal Accuracy: Sub-Test 13 | Rel-12 | C28nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.4-1 | Dynamic Range: Sub-Test 1 | Rel-9 | C01nr | All FR1 NR UEs. The UEs shall support A-GPS L1C/A only |  |  |  |  |  | Rel-15 |  |
| 13.4-2 | Dynamic Range: Sub-Test 2 | Rel-9 | C02nr | All FR1 NR UEs. The UEs shall support A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.4-3 | Dynamic Range: Sub-Test 3 | Rel-12 | C03nr | All FR1 NR UEs. The UEs shall support A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.4-4 | Dynamic Range: Sub-Test 4 | Rel-9 | C04nr | All FR1 NR UEs. The UEs shall support A-GPS and Modernized GPS only |  |  |  |  |  | Rel-15 |  |
| 13.4-5 | Dynamic Range: Sub-Test 5 | Rel-9 | C05nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.4-8 | Dynamic Range: Sub-Test 8 | Rel-12 | C15nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.4-9 | Dynamic Range: Sub-Test 9 | Rel-12 | C11nr | All FR1 NR UEs. The UEs shall support A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.4-10 | Dynamic Range: Sub-Test 10 | Rel-12 | C12nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.4-11 | Dynamic Range: Sub-Test 11 | Rel-12 | C17nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.4-12 | Dynamic Range: Sub-Test 12 | Rel-12 | C27nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.4-13 | Dynamic Range: Sub-Test 13 | Rel-12 | C28nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.5-1 | Multi-path scenario: Sub-Test 1 | Rel-9 | C01nr | All FR1 NR UEs. The UEs shall support A-GPS L1C/A only |  |  |  |  |  | Rel-15 |  |
| 13.5-2 | Multi-path scenario: Sub-Test 2 | Rel-9 | C02nr | All FR1 NR UEs. The UEs shall support A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.5-3 | Multi-path scenario: Sub-Test 3 | Rel-12 | C03nr | All FR1 NR UEs. The UEs shall support A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.5-4 | Multi-path scenario: Sub-Test 4 | Rel-9 | C04nr | All FR1 NR UEs. The UEs shall support A-GPS and Modernized GPS only |  |  |  |  |  | Rel-15 |  |
| 13.5-5 | Multi-path scenario: Sub-Test 5 | Rel-9 | C05nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.5-8 | Multi-path scenario: Sub-Test 8 | Rel-12 | C15nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.5-9 | Multi-path scenario: Sub-Test 9 | Rel-12 | C11nr | All FR1 NR UEs. The UEs shall support A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.5-10 | Multi-path scenario: Sub-Test 10 | Rel-12 | C12nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.5-11 | Multi-path scenario: Sub-Test 11 | Rel-12 | C17nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.5-12 | Multi-path scenario: Sub-Test 12 | Rel-12 | C27nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.5-13 | Multi-path scenario: Sub-Test 13 | Rel-12 | C28nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.6-1 | Moving scenario and periodic update: Sub-Test 1 (Rel-9 to Rel-13) | Rel-9, Rel-10, Rel-11, Rel-12, Rel-13 | C01nr | All FR1 NR UEs. The UEs shall support A-GPS L1C/A only |  |  |  |  |  | Rel-15 |  |
| 13.6-2 | Moving scenario and periodic update: Sub-Test 2 (Rel-9 to Rel-13) | Rel-9, Rel-10, Rel-11, Rel-12, Rel-13 | C02nr | All FR1 NR UEs. The UEs shall support A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.6-3 | Moving scenario and periodic update: Sub-Test 3 (Rel-9 to Rel-13) | Rel-12, Rel-13 | C03nr | All FR1 NR UEs. The UEs shall support A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.6-4 | Moving scenario and periodic update: Sub-Test 4 (Rel-9 to Rel-13) | Rel-9, Rel-10, Rel-11, Rel-12, Rel-13 | C04nr | All FR1 NR UEs. The UEs shall support A-GPS and Modernized GPS only |  |  |  |  |  | Rel-15 |  |
| 13.6-5 | Moving scenario and periodic update: Sub-Test 5 (Rel-9 to Rel-13) | Rel-9, Rel-10, Rel-11, Rel-12, Rel-13 | C05nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.6-8 | Moving scenario and periodic update: Sub-Test 8 (Rel-9 to Rel-13) | Rel-12, Rel-13 | C15nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo only |  |  |  |  |  | Rel-15 |  |
| 13.6-9 | Moving scenario and periodic update: Sub-Test 9 (Rel-9 to Rel-13) | Rel-12, Rel-13 | C11nr | All FR1 NR UEs. The UEs shall support A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.6-10 | Moving scenario and periodic update: Sub-Test 10 (Rel-9 to Rel-13) | Rel-12, Rel-13 | C12nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.6-11 | Moving scenario and periodic update: Sub-Test 11 (Rel-9 to Rel-13) | Rel-12,Rel-13 | C17nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.6-12 | Moving scenario and periodic update: Sub-Test 12 (Rel-9 to Rel-13) | Rel-12, Rel-13 | C27nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only |  |  |  |  |  | Rel-15 |  |
| 13.6-13 | Moving scenario and periodic update: Sub-Test 13 (Rel-9 to Rel-13) | Rel-12, Rel-13 | C28nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only (Note 1) |  |  |  |  |  | Rel-15 |  |
| 13.7-1 | Moving scenario and periodic update: Sub-Test 1 (Rel-14 onwards) | Rel-14 | C19nr | All FR1 NR UEs. The UEs shall support A-GPS L1C/A only and periodical reporting |  |  |  |  |  |  | Rel-15 |
| 13.7-2 | Moving scenario and periodic update: Sub-Test 2 (Rel-14 onwards) | Rel-14 | C20nr | All FR1 NR UEs. The UEs shall support A-GLONASS only and periodical reporting |  |  |  |  |  |  | Rel-15 |
| 13.7-3 | Moving scenario and periodic update: Sub-Test 3 (Rel-14 onwards) | Rel-14 | C21nr | All FR1 NR UEs. The UEs shall support A-Galileo only and periodical reporting |  |  |  |  |  |  | Rel-15 |
| 13.7-4 | Moving scenario and periodic update: Sub-Test 4 (Rel-14 onwards) | Rel-14 | C22nr | All FR1 NR UEs. The UEs shall support A-GPS and Modernized GPS only and periodical reporting |  |  |  |  |  |  | Rel-15 |
| 13.7-5 | Moving scenario and periodic update: Sub-Test 5 (Rel-14 onwards) | Rel-14 | C23nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS only and periodical reporting |  |  |  |  |  |  | Rel-15 |
| 13.7-8 | Moving scenario and periodic update: Sub-Test 8 (Rel-14 onwards) | Rel-14 | C24nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo only and periodical reporting |  |  |  |  |  |  | Rel-15 |
| 13.7-9 | Moving scenario and periodic update: Sub-Test 9 (Rel-14 onwards) | Rel-14 | C25nr | All FR1 NR UEs. The UEs shall support A-BDS only and periodical reporting (Note 1) |  |  |  |  |  |  | Rel-15 |
| 13.7.10 | Moving scenario and periodic update: Sub-Test 10 (Rel-14 onwards) | Rel-14 | C26nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-BDS only and periodical reporting (Note 1) |  |  |  |  |  |  | Rel-15 |
| 13.7.11 | Moving scenario and periodic update: Sub-Test 11 (Rel-14 onwards) | Rel-14 | C33nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-GLONASS and A-BDS only and periodical reporting (Note 1) |  |  |  |  |  |  | Rel-15 |
| 13.7-12 | Moving scenario and periodic update: Sub-Test 12 (Rel-14 onwards) | Rel-14 | C31nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-GLONASS only and periodical reporting |  |  |  |  |  | Rel-15 |  |
| 13.7-13 | Moving scenario and periodic update: Sub-Test 13 (Rel-14 onwards) | Rel-14 | C32nr | All FR1 NR UEs. The UEs shall support A-GPS/Modernized GPS and A-Galileo and A-BDS only and periodical reporting (Note 1) |  |  |  |  |  | Rel-15 |  |
| 15 | NR RSTD measurement requirements |  |  |  |  |  |  |  |  |  |  |
| 15.2.1 | UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Rel-16 | Cxxnr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  |  | Rel-16 |  |
| Note 1: If the signal type for BDS supported by the UE includes B1C then Rel-16 of LPP is required. |  |  |  |  |  |  |  |  |  |  |  |

Table 4-12: Applicability of tests Conditions for RAT-independent test cases in TS 37.571-1 [5] for NR

C01nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND NOT (A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C02nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND NOT (A.4.3-2/6 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C03nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/18) THEN R ELSE N/A

C04nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND NOT (A.4.3-2/7 OR A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C05nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND NOT (A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C06nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND NOT (A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C07nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND NOT (A.4.3-2/6 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C08nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C09nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND NOT (A.4.3-2/7 OR A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C10nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND NOT (A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C11nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9) THEN R ELSE N/A

C12nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/18 AND NOT (A.4.3-2/7 OR A.4.3-2/9) THEN R ELSE N/A

C13nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9) AND A.4.3-2/3 THEN R ELSE N/A

C14nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/18 AND NOT (A.4.3-2/7 OR A.4.3-2/9) AND A.4.3-2/3 THEN R ELSE N/A

C15nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND NOT (A.4.3-2/7 OR A.4.3-2/18) THEN R ELSE N/A

C16nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND NOT (A.4.3-2/7 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C17nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/18 AND NOT A.4.3-2/9 THEN R ELSE N/A

C18nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/18 AND NOT A.4.3-2/9 AND A.4.3-2/3 THEN R ELSE N/A

C19nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/6 AND NOT (A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C20nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/7 AND NOT (A.4.3-2/6 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C21nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/9 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/18) THEN R ELSE N/A

C22nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/8 AND NOT (A.4.3-2/7 OR A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C23nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND NOT (A.4.3-2/9 OR A.4.3-2/18) THEN R ELSE N/A

C24nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND NOT (A.4.3-2/7 OR A.4.3-2/18) THEN R ELSE N/A

C25nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/18 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9) THEN R ELSE N/A

C26nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/18 AND NOT (A.4.3-2/7 OR A.4.3-2/9) THEN R ELSE N/A

C27nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/9 AND NOT A.4.3-2/18 THEN R ELSE N/A

C28nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND A.4.3-2/18 AND NOT A.4.3-2/7 THEN R ELSE N/A

C29nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/9 AND NOT A.4.3-2/18 AND A.4.3-2/3 THEN R ELSE N/A

C30nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND A.4.3-2/18 AND NOT A.4.3-2/7 AND A.4.3-2/3 THEN R ELSE N/A

C31nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/9 AND NOT A.4.3-2/18 THEN R ELSE N/A

C32nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND A.4.3-2/18 AND NOT A.4.3-2/7 THEN R ELSE N/A

C33nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/18 AND NOT A.4.3-2/9 THEN R ELSE N/A

C34nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND NOT (A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C35nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND NOT (A.4.3-2/6 OR A.4.3-2/8 OR A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C36nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C37nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND NOT (A.4.3-2/7 OR A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C38nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND NOT (A.4.3-2/9 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C39nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND NOT (A.4.3-2/7 OR A.4.3-2/18) AND A.4.3-2/3 THEN R ELSE N/A

C40nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND NOT (A.4.3-2/6 OR A.4.3-2/7 OR A.4.3-2/8 OR A.4.3-2/9) AND A.4.3-2/3 THEN R ELSE N/A

C41nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/18 AND NOT (A.4.3-2/7 OR A.4.3-2/9) AND A.4.3-2/3 THEN R ELSE N/A

C42nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/18 AND NOT A.4.3-2/9 AND A.4.3-2/3 THEN R ELSE N/A

C43nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/7 AND A.4.3-2/9 AND NOT A.4.3-2/18 AND A.4.3-2/3 THEN R ELSE N/A

C44nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND (A.4.3-2/6 OR A.4.3-2/8) AND A.4.3-2/9 AND A.4.3-2/18 AND NOT A.4.3-2/7 AND A.4.3-2/3 THEN R ELSE N/A

C45nr IF A.4.1-1/6 AND A.4.3-2/20 THEN R ELSE N/A

Cxxnr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 THEN R ELSE N/A