3GPP TSG-RAN WG5 Meeting #93-e	R5-216670

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0194 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.5 - SIP INVITE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For MCData SDS and FD need to be distiguished. - For MCPTT px_MCPTT_PublicServiceId_A has been replaced by tsc_MCPTT_PublicServiceId_A; similar changes are required for MCVideo and MCData - Replaces header field is used only in very specific case and therefore can be specified in test cases where needed - MCPTT condition missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Conditions MCDATA_SDS, MCDATA_FD to distinguish SDS and FD for P-Preferred-Service, Contact and Accept-Contact - px_MCVideo_PublicServiceId_A replaced by tsc_MCVideo_PublicServiceId_A; px_MCData_PublicServiceId_A replaced by tsc_MCData_PublicServiceId_A - Replaces header field removed from Table 5.5.2.5.2-1 - MCPTT condition added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.5	SIP INVITE

##### 5.5.2.5.1	SIP INVITE from the UE

Table 5.5.2.5.1-1: SIP INVITE from the UE

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7, A.2.2.4.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "INVITE" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the participating MCPTT function serving the MCPTT user |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | The public service identity identifying the participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The public service identity identifying the participating MCData function serving the MCData user |  | MCDATA |
| Request-URI | same URI as the SS has sent earlier in the Contact header of a message within the same dialog | Contact URI of the recipient of the BYE |  | re_INVITE |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" | UE accesses the server via UDP |  | UDP |
|  | "SIP/2.0/TCP" | UE accesses the server via TCP |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | "scscf.3gpp.org" | same value as in the Service-Route header field of the 200 OK response to REGISTER |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Route |  |  | RFC 3261 [22] | re_INVITE |
| route-param list | URIs of the Record-Route header sent to the UE in the response which has established the dialog, in reverse order |  |  | MO_CALL |
|  | URIs of the Record-Route header sent to the UE in the INVITE |  |  | MT_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| From |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog ID (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | Same URI as Request-URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| To |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the SS as used earlier in the dialogURI | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog ID (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
| callid | same value as in INVITE creating the dialog |  |  | re_INVITE |
| CSeq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  | re_INVITE |
| method | "INVITE" |  |  |  |
| Supported |  |  | RFC 3261 [22] |  |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  | RFC 4028 [30] |  |
| delta-seconds | any allowed value |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports mission critical data (MCData) service.communication. |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports mission critical data (MCData) service.communication. |  | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA_FD |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-specs | Access network technology and, if applicable, the cell ID | AUTO |  |  |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp” |  |  |  |
| media-range[2] | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | application/vnd.3gpp.mcvideo-info+xml |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| P-Preferred-Identity | if present |  | RFC 3325 [32] |  |
| PPreferredID-value | same URI as in From-header |  |  |  |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Priv-Answer-Mode | not present |  |  |  |
| Answer-Mode | not present |  | RFC 5373 [34] | re_INVITE |
| Answer-Mode |  |  | RFC 5373 [34] |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Resource-Priority |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] | EMERGENCY-CALL or IMMPERIL-CALL |
| r-value |  |  |  | EMERGENCY-CALL |
| namespace | value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 |  |  |
| r-priority | value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 |  |  |
| r-value |  |  |  | IMMPERIL-CALL |
| namespace | value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 |  |  |
| r-priority | value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/sdp" |  | RFC 4566 [27] |  |
| MIME-part-body | SDP Message as described in Table 5.5.3.1.1-1 |  |  | MCPTT |
|  | SDP Message as described in Table 5.5.3.1.1-2 |  |  | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.1-3 |  |  | MCDATA |
| MIME body part |  | MCPTT Info/MCVideo/MCData |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Resource list | RFC 5366 [35] | PRIVATE-CALL OR MCD_1to1 |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | As described in Table 5.5.3.3.1-1 |  |  | MCPTT |
|  | As described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | As described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | (EMERGENCY-CALL AND ALERT_IND) OR LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | This MIME part shall be included if the MCPTT-Info 'alert-ind' element sent in the MCPTT-Info is set to true. |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" | This MIME part shall be included if the MCVideo-Info 'alert-ind' element sent in the MCVideo-Info is set to true. |  | MCVIDEO |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.1-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | MIKEY message |  | MCD_1to1 |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in Table 5.5.9.1-2A | MIKEY message, containing the PSK | TS 33.180 [30] TS 24.282 [87] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| MCD_1to1 | A one-to-one MCData call |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| re_INVITE | INVITE within a dialog |
| ALERT_IND | MCPTT emergency alert is required as specified for the test case or automatically initiated by the client for an emergency call (in case of condition EMERGENCY-CALL when pc_MCX_EmergencyIndWithAlertInd=true); <alert-ind> is set to true in the mcptt-info. |
| NOTE: For further conditions see table 5.5.1-1 |  |

##### 5.5.2.5.2	SIP INVITE from the SS

Table 5.5.2.5.2-1: SIP INVITE from the SS

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7, A.2.2.4.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "INVITE" |  |  |  |
| Request-URI | SIP URI of the UE’s contact address as provided in the Contact-header of the REGISTER message |  |  |  |
| Request-URI | same URI as the UE has sent earlier in the Contact header of a response within the same dialog | Contact URI of the UE ("callee") |  | re_INVITE |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol[1] | "SIP/2.0/TCP" |  |  |  |
| sent-by[1] |  | Address of the P-CSCF that communicates with the called party |  |  |
| host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| via-branch[1] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[2] | "SIP/2.0/UDP" |  |  |  |
| sent-by[2] |  | Address of the other endpoint (the caller) |  |  |
| host | Host name of the SIP URI being used in the From header |  |  |  |
| port | Same port number as in Contact-header | Caller’s port number |  |  |
| via-branch[2] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| Record-Route |  | Record-Route corresponding to the Via header | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI | SIP URI corresponding to first entry of Via header |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | “term@scscf1.3gpp.org” |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[3] | SIP URI |  |  |  |
| user-info and host | “orig@scscf2.3gpp.org” |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[4] | SIP URI |  |  |  |
| user-info and host | “pcscf2.3gpp.org” |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Record-Route | same as in the 180, 183 or 200 response sent to the UE during MO call establishment in reverse order |  | RFC 3261 [22] | re_INVITE AND MO_CALL |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A | SIP URI of the calling UE |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A | SIP URI of the calling UE |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | SIP URI of the calling UE |  | MCDATA |
| port | not present |  |  |  |
| tag | Value assigned by the SS |  |  |  |
| From |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the SS as used earlier in the dialog | Remote URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the SS as used earlier in the dialog | Remote tag of the dialog (from the UE's point of view) |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | px_MCX_SIP_PublicUserId_A_1 | Default public user ID (IMPU) as stored in the UICC |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| To |  |  | RFC 3261 [22] | re_INVITE |
| addr-spec | Same URI of the UE as used earlier in the dialog | Local URI of the dialog (from the UE's point of view) |  |  |
| tag | Same tag of the UE as used earlier in the dialog | Local tag of the dialog (from the UE's point of view) |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Value assigned by the SS |  |  |  |
| Call-ID |  |  | RFC 3261 [22] | re_INVITE |
| callid | same value as in INVITE creating the dialog |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | Value assigned by the SS |  |  |  |
| value | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  | re_INVITE |
| method | "INVITE" |  |  |  |
| Supported |  |  | RFC 3261 [22] |  |
| option-tag | "100rel" | This option tag indicates that the UA can send or receive reliable provisional responses. |  |  |
| option-tag | "timer" |  |  |  |
| option-tag | "tdialog" |  |  |  |
| option-tag | "norefersub" |  |  |  |
| P-Called-Party-ID |  |  | RFC 7315 [52] |  |
| called-pty-id-spec | Same public user ID as used in the To-header |  |  |  |
| Session-Expires |  |  | RFC 4028 [30] |  |
| generic-param | "1800" | The recommended initial value is 1800 in RFC 4028 [30]. |  |  |
| P-Early-Media |  |  | RFC 5009 [60] |  |
| em-parm | "inactive" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as in From-header |  |  |  |
| port | not present |  |  |  |
| Contact |  |  | RFC 3261 [22] RFC 3840 [33] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
| port | Value assigned by the SS |  |  |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. | RFC 3840 [33] clause 9 | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. | RFC 3840 [33] clause 9 | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. | RFC 3840 [33] clause 9 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. | RFC 3840 [33] clause 9 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. | RFC 3840 [33] clause 9 | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. | RFC 3840 [33] clause 9 | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) SDS service. | RFC 3840 [33] clause 9 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) FD service. | RFC 3840 [33] clause 9 | MCDATA_FD |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. | RFC 3840 [33] clause 10.1 | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| feature-param | "isfocus" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "68" | The recommended initial value is 70 in RFC 3261 [22]. Assuming 2 hops as according to the Via header this results in a value of 68 in the message sent to the UE |  |  |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp " |  |  |  |
| media-range[2] | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Answer-Mode | not present |  | RFC 5373 [34] TS 24.379 [9] clause 6.3.2.2.6.3 | re-INVITE OR FIRST-TO-ANSWER |
| Answer-Mode |  |  | RFC 5373 [34] |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Priv-Answer-Mode |  |  |  | FIRST-TO-ANSWER |
| answer-mode-value | "Manual" |  |  |  |
| Resource-Priority |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] | EMERGENCY-CALL or IMMPERIL-CALL |
| r-value |  |  |  | EMERGENCY-CALL |
| namespace | value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 |  |  |
| r-priority | value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 |  |  |
| r-value |  |  |  | IMMPERIL-CALL |
| Namespace | value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 |  |  |
| r-priority | value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| Value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/sdp" |  |  |  |
| MIME-part-body | SDP Message as described in Table 5.5.3.1.2-1 |  | RFC 4566 [27] | MCPTT |
|  | SDP Message as described in Table 5.5.3.1.2-2 |  | RFC 4566 [27] | MCVIDEO |
|  | SDP Message as described in Table 5.5.3.1.2-3 |  | RFC 4566 [27] | MCDATA |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.2-1 |  |  | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.2-2 |  |  | MCVIDEO |
|  | As described in Table 5.5.3.2.2-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  | MCVIDEO |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.2-1 |  | TS 24.379 [9] clause F.3 | MCPTT |
|  | Location-info as described in Table 5.5.3.4.2-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | MIKEY message |  | MCD_1to1 |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in Table 5.5.9.1-2 | MIKEY message, containing the PSK | TS 33.180 [30] TS 24.282 [87] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-2 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| re_INVITE | INVITE within a dialog |
| MCD_1to1 | A one-to-one MCData call |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| For further conditions see table 5.5.1-1 |  |