3GPP TSG-RAN5 Meeting #93-e	R5-216772

# Electronic Meeting, 8th Nov 2021 – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2058 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE ServingCellConfigCommonSIB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To update IE ServingCellConfigCommonSIB for NR-based access to unlicensed spectrum. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IE ServingCellConfigCommonSIB has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will not be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	ServingCellConfigCommonSIB

Table 4.6.3-169: ServingCellConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB |  |  |
| supplementaryUplink | Not present |  |  |
|  | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid | SSB#1 |
|  | ’1000 0000’B |  | SSB#0 |
| groupPresence | Not present |  | FR1 |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| dynamic | NULL |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| highSpeedConfig-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access |

<End of modified section>