3GPP TSG-RAN WG5 Meeting #93-e	R5-217343

# Electronic meeting, 8th - 19th Nov, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0656 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 6.2.4 for configured transmitted power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some errors occur in the section of configured transmitted power. The term of PPowerClass should be referred to UE minimum peak EIRP due to FR2 power classs consists of minimum peak EIRP, maximum TRP, maximum EIRP and EIRP spherical coverage. In addition, some other editorial corrections should be made. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the definition of PPowerClass in 6.2.4.3. - Other editorial corrections in 6.2.4.1 and 6.2.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configured transmitted power will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4.1, 6.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

6.2.4	Configured transmitted power

6.2.4.1	Test purpose

To verify the UE configured transmitted power PUMAX,f,c is within the range defined prescribed by the specified nominal maximum output power and tolerance.

6.2.4.2	Test applicability

The requirements of this test are covered in test cases 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements to all types of NR UE release 15 and forward.

6.2.4.3	Minimum conformance requirements

The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [24].

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PPowerclass + PIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

with PPowerclass the UE minimum peak EIRP as specified in sub-clause 6.2.1.1.3, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1.1.3, MPRf,c as specified in sub-clause 6.2.2.3, A-MPRf,c as specified in sub-clause 6.2.3.3, ΔMBP,n the peak EIRP relaxation as specified in section 6.2.1.1.3 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1.1.3. PIBE is 1.0 dB if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK, MPRf,c = 0 and when NS_200 applies and the network configures the UE to operate with mpr-PowerBoost-FR2-r16, otherwisePIBE is 0.0 dB. The requirement is verified in beam peak direction.

maxUplinkDutyCycle-FR2 as defined in TS 38.306 [26] is a UEcapability to facilitate electromagnetic power density exposure requirements. This UE capability is applicable to all FR2 power classes.

If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c.

If the field of UE capability maxUplinkDutyCycle-FR2 is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means.

P-MPRf,c is the power management maximum output power reduction. The UE shall apply P-MPRf,c for carrier f of serving cell c only for the cases described below. For UE conformance testing P-MPRf,c shall be 0 dB.

a)	ensuring compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic power density exposure requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

NOTE 1:	P-MPRf,c was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRf,c and maxUplinkDutyCycle-FR2 may impact the maximum uplink performance for the selected UL transmission path.

NOTE 3:	MPE P-MPR Reporting, as defined in TS 38.306 [26], is an optional UE capability to report P-MPRf,c when the reporting conditions configured by gNB are met. This UE capability is applicable to all FR2 power classes.

The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4.3-1.

Table 6.2.4.3-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n259, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3.1. |  |  |

6.2.4.4	Test description

This test is covered by clause 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements.

6.2.4.5	Test requirements

This test is covered by clause 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements.

==============================================================

<< End of changes >>