3GPP TSG-RAN5 Meeting #93-e	R5-217412

# Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1547 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of cell configuration MU and TT for Enhancement on MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Enhancement MIMO Test cases have been introduced in in RAN5 TS 38.533, so the related Cell, TT and MU need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Cell configuration, TT and MU for Enhancement MIMO Test Cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancement MIMO Test Case will be still incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.2, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4

Table E.2-1 defines the cell configuration mapping for EN-DC FR1 test cases in chapter 4 of this test specification.

Table E.2-1: Cell configuration mapping for EN-DC FR1 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.5 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.6 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |

## << Unchanged sections omitted >>

F.1.1.2	Measurement of RRM requirements

Many contents are skipped

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  | ± 0.6dB (AWGN conditions) |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB |  | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  | Same as 4.5.5.1 |  |  | Same as 4.5.5.1 |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  | Same as 4.5.5.3 |  |  | Same as 4.5.5.3 |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  |  | Same as 4.5.5.5 |  |  | Same as 4.5.5.5 |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  | ±0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |

## << Unchanged sections omitted >>

F.1.3.2	Measurement of RRM requirements

Many contents are skipped

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test |  |  | Minimum requirement in TS 38.133 [6] |  |  | Test tolerance (TT) |  |  | Test requirement in TS 38.533 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 |  |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  |  | 2.1dB 0.7dB 112Tc 112Tc |  |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |  |
| 4.3.2.2.2 |  |  | Same as 4.3.2.2.1 |  |  | Same as 4.3.2.2.1 |  |  | Same as 4.3.2.2.1 |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy |  |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB |  |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section D.4.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.2 |  |  | Same as 4.5.1.2 |  |  | Same as 4.5.1.2 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.2 |  |  | Same as 4.5.1.2 |  |  | Same as 4.5.1.2 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.2 |  |  | Same as 4.5.1.2 |  |  | Same as 4.5.1.2 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | SNRs as specified |  |  | 0.6dB |  |  | Formula: SNR + TT |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  |  | Same as 4.5.2.1 |  |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  | During T1: 0dB 0dB 0dB 0dB |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  |  | Same as 4.5.2.3 |  |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  | During T1: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  |  | Same as 4.5.2.5 |  |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  |  | Same as 4.5.3.1 |  |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  |  | Same as 4.5.3.1 |  |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  | Same as 4.5.5.1 |  |  | Same as 4.5.5.1 |  |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  | Same as 4.5.5.3 |  |  | Same as 4.5.5.3 |  |  | Same as 4.5.5.3 |  |  |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  |  | Same as 4.5.5.5 |  |  | Same as 4.5.5.5 |  |  | Same as 4.5.5.5 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |

## << End of changes >>