3GPP TSG-RAN5 Meeting #93-e	R5-217792

# Electronic Meeting, 8th Nov 2021 – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2043 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update chapter 4.5.4 RRC_CONNECTED |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add connectivity NE-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NE-DC has been added for RRC_CONNECTED. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The connectivity NE-DC will not be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-216442 and the outcome of 2 revisions to this document. |  |  |  |  |  |  |  |  |

<Start of first modified section>

### 4.5.4	RRC_CONNECTED

#### 4.5.4.1	Initiation

The SS shall:

1>	perform according to clause 4.5.2 RRC_IDLE;

1>	if connectivity is EN-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	if connected without release is On:

3>	perform according to the table 4.5.4.2-2: RF E-UTRA RRC_CONNECTED;

2>	else:

3>	perform according to the table 4.5.4.2-1: E-UTRA RRC_CONNECTED;

1>	if connectivity is E-UTRA/EPC:

2>	use 1 E-UTRA cell, default parameters;

2>	perform according to the table 4.5.4.2-1: E-UTRA RRC_CONNECTED;

1>	if connectivity is NR:

2>	use 1 NR cell, default parameters;

2> if sidelink is On

3>	use 1 simulated NR SL UE;

3> if GNSS Sync is On

4> use 1 GNSS simulator.

2>	if connected without release is not present:

3>	perform according to the table 4.5.4.2-3: NR RRC_CONNECTED;

1>	if connectivity is WLAN:

2>	use 1 WLAN cell, default parameters;

2>	if connected without release is not present:

3>	perform according to the table 4.5.4.2-4: WLAN IPsec_SA_Established;

2>	else:

3>	Not defined;

1>	if connectivity is NR-DC:

2>	use 2 NR cells, default parameters;

2>	perform according to the table 4.5.4.2-5: NR-DC RRC_CONNECTED;

1>	if connectivity is NE-DC:

2>	use 1 E-UTRA cell and 1 NR cell, default parameters;

2>	perform according to the table 4.5.4.2-6: NE-DC RRC_CONNECTED;

#### 4.5.4.2	Procedures

Table 4.5.4.2-1: E-UTRA RRC_CONNECTED

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-6 | Same as TS 36.508 [2] table 4.5.3.3-1, steps 2-7. | - | - | - | - |
| 7 | Same as TS 36.508 [2] table 4.5.3.3-1, step 8. The RRCConnectionReconfiguration is using condition EN-DC_SRB2-DRB for connectivity EN-DC and bearers MCG(s) and SCG or connectivity E-UTRA/EPC. The RRCConnectionReconfiguration is using an associated condition MCG_and_SCG for bearers MCG(s) and SCG or condition MCG_and_split for bearers MCG(s) and split. For connectivity E-UTRA/EPC there’s no associated condition. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF MCG(s) and SCG or MCG(s) and split. In parallel to steps 8-9 the UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the NR cell. | - | - | - | - |
| 8-9 | Same as TS 36.508 [2] table 4.5.3.3-1, steps 9-10a1 | - | - | - | - |
| 10a1-10a2 | IF Test Loop Function=On, same as TS 36.508 [2] table 4.5.4.3-1, steps 1-2. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | - | - | - | - |

Table 4.5.4.2-2: RF E-UTRA RRC_CONNECTED

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-9 | Same as table 4.5.2.2-1, steps 1-9. | - | - | - | - |
| 10a1-10a2 | IF Test Mode = On OR Test Loop Function = On THEN same as TS 36.508 [2] table 4.5.2A.3-1, steps 10-11. The ACTIVATE TEST MODE is using the associated condition for the test loop. | - | - | - | - |
| - | EXCEPTION: Steps 11a1 to 11b8 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 11a1-11a8 | IF Test Mode = On OR Test Loop Function = On THEN same as TS 36.508 [2] table 4.5.2A.3-1, steps 12-18. | - | - | - | - |
| 11b1-11b8 | ELSE, same as TS 36.508 [2] table 4.5.2.3-1, steps 10-16. | - | - | - | - |
| 12-15 | Same as table 4.5.4.2-1, steps 7-10. | - | - | - | - |

Table 4.5.4.2-3: NR RRC_CONNECTED

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-DRB(n, m) where n and m are the number of DRB(s) configured with RLC-AM and RLC-UM respectively. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 9a1 | IF Test Loop Function=On, the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 9a2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |

Table 4.5.4.2-4: WLAN IPsec_SA_Established

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to initiate IPSec SA. | - | - | - | - |
| 2 | The generic procedure for UE-requested IPsec Secure tunnel establishment, specified in subclause 4.5A.64, takes place performing establishment of security association and one child security association. | - | - | - | - |

Table 4.5.4.2-5: NR-DC RRC_CONNECTED

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Same as table 4.5.4.2-3, steps 1-8. | - | - | - | - |
| 8A | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add a new SCG DRB or a new split DRB | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: In parallel to steps 8B and 8C the UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the PSCell. | - | - | - | - |
| 8B | The UE transmits an RRCReconfigurationComplete message including nr-SCG-Response. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8C | The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 9a1 | IF Test Loop Function=On, the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 9a2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |

Table 4.5.4.2-6: NE-DC RRC_CONNECTED

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Same as table 4.5.4.2-3, steps 1-8. | - | - | - | - |
| 8A | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add a new SCG DRB or a new split DRB | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: In parallel to steps 8B and 8C the UE performs a C-RNTI based Contention Free Random Access (CFRA) procedure on the PSCell. | - | - | - | - |
| 8B | The UE transmits an RRCReconfigurationComplete message including eutra-SCG-Response. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8C | The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour which depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 9a1 | IF Test Loop Function=On, the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 9a2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |

#### 4.5.4.3	Specific message contents

All specific message contents shall be according clause 4.6 and 4.7 and TS 36.508 [2] clause 4.6 and 4.7 with the exceptions below.

Table 4.5.4.3-0: RRCConnectionReconfiguration (step 7, Table 4.5.4.2-1)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoNASList |  | Not present | no NAS message | MCG(s) only |
| dedicatedInfoNASList SEQUENCE (SIZE(1..maxDRB)) OF |  | 1 entry |  | MCG_and_SCG OR MCG_and_split |
| dedicatedInfoNAS [1] |  | OCTET STRING including ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST | according to table 4.5.4.3-1 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.4.3-1: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 7, Table 4.5.4.2-1)

| Derivation path: TS 36.508 [2] Table 4.7.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | 12 |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |
| TFT | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |
| Negotiated QoS | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |
| Negotiated LLC SAPI | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |
| Radio priority | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |
| Protocol configuration options | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |
| Extended protocol configuration options | According to reference dedicated EPS bearer context #6 - in TS 36.508 [2] table 6.6.2-1A |  |  |

Table 4.5.4.3-2: RRCReconfiguration (step 7, Table 4.5.4.2-3)

Derivation Path: TS 38.508-1, table 4.8.1-1B

Table 4.5.4.3-3: Void

Table 4.5.4.3-4: PDU SESSION MODIFICATION COMMAND (step 8A, Table 4.5.4.2-5 and Table 4.5.4.2-6)

| Derivation Path: Table 4.7.2-9. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST associated with the Internet PDU session if available or with the first PDU session |  |  |
| Authorized QoS rules |  | One entry |  |  |
| QoS rule [1] |  | Reference QoS rule #5 as defined in Table 4.8.2.1-5. | QFI=5 |  |
| Authorized QoS flow descriptions |  | One entry |  |  |
| QoS flow [1] |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. | QFI=5 |  |

Table 4.5.4.3-5: RRCReconfiguration (step 8A, Table 4.5.4.2-5)

Derivation Path: TS 38.508-1, table 4.8.1-1CA

Table 4.5.4.3-6: RRCReconfigurationComplete (step 8B, Table 4.5.4.2-5)

Derivation Path: TS 38.508-1, table 4.6.1-14 with condition NR-DC

Table 4.5.4.3-7: RRCReconfiguration (step 8A, Table 4.5.4.2-6)

Derivation Path: TS 38.508-1, table 4.8.1-1CB

Table 4.5.4.3-8: RRCReconfigurationComplete (step 8B, Table 4.5.4.2-6)

Derivation Path: TS 38.508-1, table 4.6.1-14 with condition NE-DC

<End of modified section>