3GPP TSG RAN Meeting #93-e	                    R5-218202

Electronic Meeting, 08th Nov – 16th Nov 2021

Agenda item:	5.3.1.17

Source:   Apple, AT&T, China Telecom, Dish Network, NTT Docomo, Keysight Technologies, Nokia, Oppo, Orange, Qualcomm, Rohde & Schwarz, Samsung, Verizon, ZTE

Title:       Incorporating TR 38.884 outcome in RAN5 Test Specifications

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

3GPP RAN4 has in Rel-17 worked on a Study Item focusing on Enhanced Test Methods for FR2 UEs. The study has identified improvements to current FR2 test methodologies as well as new enhanced FR2 test methods across a range of topics. This includes some issues reported by RAN5 such as:

- Test requirement relaxation due to High DL/Low UL signal power limitations

- Mitigation of polarization basis mismatch

- Extreme Temperature Condition testing methodology.

In addition, several test time reduction topics are included in the RAN4 SID. It should be noted that the completion level of 3GPP RAN4 study on enhanced FR2 test methods is at 70% at RP#93-e (September-2021).

# 2	Discussion

## 2.1     Summary of TR 38.884

The scope of the proposed RAN5 effort broadly covers:

- Improvements in current methods or new test methodologies to resolve multiple testability concerns raised by RAN5 which have regulatory impact (RP-191603: large relaxation in core requirements due to testability issues)

- Enhanced Test Methods updates overcome previous testing limitations

- Alternate Methodology enhancements for Test Time Reduction

A detailed view of the test methodology enhancements identified within the RAN4 study item, (which must be reviewed and analysed for incorporating into RAN5 test specifications) are within the high-level initial scope listed in items 1 through 8 in section 2.1.1 below.  The test time reduction techniques recommended are understood to be optional within the RAN5 test specifications and depending on UE vendor declaration.

### 2.1.1     High-level initial SCOPE of intended RAN5 Work Plan

A RAN5 Work Plan to incorporate TR 38.884 outcomes into RAN5 test specifications will intend to cover the following initial scope:

- High DL power and low UL power test cases: improvement of current permitted methods (e.g. IFF) has been identified in the relaxation of requirements for the specific tests reported by RAN5.

- High DL power and low UL power test cases: new methodologies based on near-field (i.e. Direct Near Field (DNF), Combined Far-Field/Direct Near Field (CFFDNF), and Combined Far-Field/Near Field (CFFNF)) have been analyzed and the study outcome is included in TR 38.884.

- Enhanced test method for EIRP measurements: adopt the TPMI method defined in TR 38.884 to activate the dual polarization transmission on the UE, removing the need for a specific test function and define the applicability of the method to the EIRP based test procedures and FR2 UE types.

- Enhanced test methods for UL Demodulation Measurements: adopt zero-forcing MIMO receiver architecture based on Method 1 as described in TR 38.884 so that dual-polarization transmissions by the UE can be demodulated by the test equipment receiver.

- Analyze and incorporate testability enhancements to support the verification of RF requirements for inter-band (FR2+FR2) CA

  - An analysis of the impact of AoA offsets in the test setup for inter-band CA with Common Beam Management (CBM) is needed after the scope of CBM requirements and associated agreements are better understood.

- Incorporate any new recommendations in TR 38.884 for testing under extreme temperature conditions for all applicable FR2 UE RF test cases.

- Analyze and incorporate the testability enhancements defined in TR 38.884 to reduce test time. All the recommended test methodology enhancements are understood to be optional.

  - New Measurement grids based on 4x2 antenna pattern assumption, dependent on UE declaration

  - RSRPB-based RX beam peak search

  - Fast Spherical Coverage

  - Single Link Polarization Measurement, dependent on UE declaration

  - Non-uniform TRP measurement grids

- Adopt the recommendations in TR 38.884 and perform further analysis of testability aspects for the introduction of the new band n262

  - Considering the extension of frequency applicability of the permitted methods in TR38.810 from 43.5 GHz up to at least 48.2 GHz

  - Considering the extension of frequency applicability of the test methodology enhancements in Objectives 1 through 7 above

NOTE 1:  It should be noted that topics related to ’NR Enhancements for 52-71 GHz’ has just commenced in RAN4 post RAN#93 and is currently not planned to be included in initial scope. Study of item 1 and item 2 (improvements of relaxation) can consider the possibility that test system supports both FR2-1 and FR2-2.

NOTE 2: The work plan initial scope may include MU assessments required in RAN5 specific to the topics listed above.

NOTE 3:

- Significant deviations from TR 38.884 outcomes and/or

- Mapping to known RAN5 testability issues

are intended to be tracked as part of the progress of the proposed RAN5 umbrella work plan

### 2.1.2	RAN5 Impact

This section lists the anticipated list of test specifications that would need to be updated due to the FR2 Enhanced Test Methods Study

| TS/TR No. | Description of anticipated changes | Related RAN5 Work Item(s) |
| --- | --- | --- |
| TS 38.521-2 | Introduction of enhanced test methods aspects for SA FR2 |  |
| TS 38.521-3 | Introduction of enhanced test methods aspects for NSA FR2 |  |
| TS 38.522 | Updated applicability for tests impacted by enhanced FR2 test methods |  |
| TS 38.508-1 | Update to FR2 RF test environments |  |
| TS 38.508-2 | Introduction of UE declarations for alternate enhanced test methods |  |
| TS 38.509 | Introduction of any conformance test functions for enhanced test methods. |  |
| TS 38.521-4 | Introduction of n262 MU and enhanced test methods aspects for 5G Demod |  |
| TS 38.533 | Introduction of n262 MU and enhanced test methods aspects for 5G RRM |  |
| TR 38.903 | Documentation of MU assessment for enhanced test methods |  |

Observation 1: There is significant impact to multiple FR2 RF tests due to the enhanced test methods defined in TR 38.884. The RAN4 SID outcome needs to be evaluated and accordingly incorporated in RAN5 specifications to resolve multiple FR2 RF testability issues, several of which were reported by RAN5.

Observation 2: A new RAN5 WID based on the RAN4 SID is not feasible as per 3GPP procedures

## 2.2	Incorporating FR2 Test Methods into RAN5 specifications

### 2.2.1 Background

During the initial development of FR2 RF TEST cases in TS 38.521-2 as part of the Rel.15, the test methods were incorporated into the RAN5 test specifications via the 5GS_NR_LTE-UEConTest Rel15 WID.

These test methods were defined by the RAN4 FS_NR_Test_methods SI with outcomes captured in TR 38.810. In case of the initial development of tests the process of adopting TR 38.810 recommended test methods was automatic as the same was needed to define initial FR2 OTA test procedures for the tests in TS 38.521-2.

Observation 3: Existing process to adopt TR 38.810 defined test methods into TS 38.521-2 was clear and easy to track as those methods was needed to define all initial FR2 RF tests in Release 15.

However, in case of the enhanced test methods SID outcomes in TR 38.884, the process is neither automatic nor straightforward due to multiple dependencies.

  - Many of the tests that would need to be updated are marked complete in the TS 38.521-2 Rel.15 Work plan with known testability issues recorded.

  - Several impacted tests are also not part of the Rel.16 FR2 Enhancements Work Item (the only other FR2 RF focused RAN5 Work Item that is currently active).

Observation 4: Adoption of TR 38.884 enhanced test methods into RAN5 test specifications (primarily FR2 RF) is more complicated to track considering the impacted tests and their dependency on TR38.884 is not tracked in any RAN5 Work Plan document (for example several impacted tests are marked complete with known issues in Rel.15 WID and/or not included in Rel.16 FR2 RF enhancements WID

The below table provides an example for Observation 4.

| Example | 5GS_NR_LTE-UEConTest Rel15 WID | NR_RF_FR2_req_enh-UEConTest – Rel16 FR2 RF WID | RAN4 SID (FR2 Enhanced Test Methods) – TR 38.884 |
| --- | --- | --- | --- |
| FR2 RF TC 6.3.1 | Marked 100% complete with note that Core Requirements are not testable | Not in scope of RAN4/RAN5 WID | New Test Methodology to overcome testability issues |
| Multiple tests involving TRP measurements | Marked 100% complete with test function to disable Tx Diversity on UE | Several impacted tests not in scope of RAN4/RAN5 WID scope | Enhancement to overcome current test limitation (disable Tx diversity on UE) |

### 2.2.3 Discussion on possible ways forward for RAN5 to incorporate TR 38.884

#### 2.2.3.1	For Release 15 and forward applicable enhanced FR2 test methods

For NR enhanced FR2 test methods that are release-independent and should apply to Release15 and onwards, a separate sub work-plan can be created within the existing 5GS_NR_LTE-UEConTest RAN5 Work Item.

Proposal 1: Create a sub-area work plan within 5GS_NR_LTE-UEConTest Rel.15 WID to track adoption of FR2 enhanced test methods (applicable to Rel-15 and forward) into RAN5 test specifications.

#### 2.2.3.2	For Release 16 and forward applicable enhanced FR2 test methods

Some of the enhanced test methods are applicable to Rel-16 (and higher) UEs

One example is the TPMI side condition method, where TPMI side conditions are applicable certain to Rel-16 (and higher) UEs as described in the TR 38.884 snippet below

Proposal 2: Create a sub-area work plan within NR_RF_FR2_req_enh-UEConTest Rel.16 WID to track adoption of FR2 enhanced test methods (applicable to Rel-16 and forward) into RAN5 test specifications.

#### 2.2.3.3 Alternate Option (Preferred) - Umbrella Work Plan

Considering that the task of incorporating FR2 enhanced test methods in TS 38.884 into RAN5 specifications may fall under different active/closed/maintenance WIDs over some period, another option is listed.

Proposal 3: Create an umbrella RAN5 Work Plan (following the above-outlined initial scope) to track adoption of TR 38.884 outcomes into RAN5 test specifications.

A dedicated agenda item may track the Work Plan related TDocs for this task.

# 3	Summary

In this contribution, the following observations were made

Observation 1: There is significant impact to multiple FR2 RF tests due to the enhanced test methods defined in TR 38.884. The RAN4 SID outcome needs to be evaluated and accordingly incorporated in RAN5 specifications to resolve multiple FR2 RF testability issues, several of which were reported by RAN5.

Observation 2: A new RAN5 WID based on the RAN4 SID is not feasible as per 3GPP procedures

Observation 3: Existing process to adopt TR 38.810 defined test methods into TS 38.521-2 was easy to track due to clear dependency as those methods were needed to define all initial FR2 RF tests in Release 15 via 5GS_NR_LTE-UEConTest Rel.15 WID.

Observation 4: Adoption of TR 38.884 enhanced test methods into RAN5 test specifications (primarily FR2 RF) is more complicated to track considering the impacted tests and their dependency on TR38.884 is not tracked in any RAN5 Work Plan document (for example several impacted tests are marked complete with known issues in Rel.15 WID and not included in Rel.16 FR2 RF enhancements WID)

OPTION A

Proposal 1: Create a sub-area work plan within 5GS_NR_LTE-UEConTest Rel.15 WID to track adoption of FR2 enhanced test methods (applicable to Rel-15 and forward) into RAN5 test specifications.

Proposal 2 : Create a sub-area work plan within NR_RF_FR2_req_enh-UEConTest Rel.16 WID to track adoption of FR2 enhanced test methods (applicable to Rel-16 and forward) into RAN5 test specifications.

OPTION B (Preferred)

Proposal 3: Create an umbrella RAN5 Work Plan (following the above-outlined initial scope) to track adoption of TR 38.884 outcomes into RAN5 test specifications.

# 4	References

TR 38.810 - NR: Study on Test Methods

TR 38.884 - FR2 Enhanced Test Methods for NR UEs

R5-217727 New WID on UE Conformance – Enhanced Test Methods for FR2, Apple, 3GPP TSG RAN WG5 Meeting #93-e, November 2021

TS 38.521-2 – FR2 RF Test Specification