3GPP TSG-RAN5 Meeting #94-e	R5-220180

# Online, ,February 21 –March 04, 2022

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.918 | CR | n/a | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Test Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SD value of IE S-NSSAI in REGISTRATION ACCEPT is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the value of SD in Table A.2.1.1.4.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.918 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Table A.2.1.1.4.2-1: REGISTRATION ACCEPT (step 3)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | ‘001’B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000001 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000001’B | 1 |  |
| SD |  | 0x000001 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000002 |  |  |
| Mapped configured SST |  | Not Present |  |  |
| Mapped configured SD |  | Not Present |  |  |

<<< END OF CHANGES >>>