3GPP TSG-RAN5 Meeting #94-e	R5-220199

# Electronic Meeting, Feb 21 - Mar 4, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0289 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Physical Layer Baseline Implementation Capabilities for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NE-DC needs to be added into Physical Layer Baseline Implementation Capabilities in TS 38.508-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NE-DC has been added into Physical Layer Baseline Implementation Capabilities in TS 38.508-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for NE-DC can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

A.4.3.2B	NR-DC, EN-DC and NE-DC Physical Layer Baseline Implementation Capabilities

A.4.3.2B.1	NR-DC Physical Layer Baseline Implementation Capabilities

A.4.3.2B.1.0	General NR-DC capabilities

Table A.4.3.2B.1.0-1: Downlink NR-DC capabilities (for one or more of the supported NR-DC configurations)

| Item | Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR-DC with 2 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_2CC |  |
| 2 | DL NR-DC with 3 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_3CC |  |
| 3 | DL NR-DC with 4 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_4CC |  |
| 4 | DL NR-DC with 5 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_5CC |  |
| 5 | DL NR-DC with 6 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_6CC |  |
| 6 | DL NR-DC with 7 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_7CC |  |
| 7 | DL NR-DC with 8 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_8CC |  |
| 8 | DL NR-DC with 9 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_9CC |  |
| 9 | DL NR-DC with 10 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_10CC |  |

Table A.4.3.2B.1.0-2: Uplink NR-DC capabilities (for one or more of the supported NR-DC configurations)

| Item | Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR-DC with 2 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_2CC |  |
| 2 | UL NR-DC with 3 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_3CC |  |
| 3 | UL NR-DC with 4 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_4CC |  |
| 4 | UL NR-DC with 5 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_5CC |  |
| 5 | UL NR-DC with 6 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_6CC |  |
| 6 | UL NR-DC with 7 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_7CC |  |
| 7 | UL NR-DC with 8 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_8CC |  |
| 8 | UL NR-DC with 9 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_9CC |  |
| 9 | UL NR-DC with 10 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_10CC |  |

<< UNCHANGED PARTS SKIPPED >>

A.4.3.2B.2	EN-DC Physical Layer Baseline Implementation Capabilities

A.4.3.2B.2.0	General EN-DC capabilities

Table A.4.3.2B.2.0-1: Downlink EN-DC capabilities (for one or more of the supported EN-DC configurations)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL EN-DC with 2 carriers | 38.101-3, 5.5B |  |
| 2 | DL EN-DC with 3 carriers | 38.101-3, 5.5B |  |
| 3 | DL EN-DC with 4 carriers | 38.101-3, 5.5B |  |
| 4 | DL EN-DC with 5 carriers | 38.101-3, 5.5B |  |
| 5 | DL EN-DC with 6 carriers | 38.101-3, 5.5B |  |
| 6 | DL EN-DC with 7 carriers | 38.101-3, 5.5B |  |
| 7 | DL EN-DC with 8 carriers | 38.101-3, 5.5B |  |

Table A.4.3.2B.2.0-1A: Downlink EN-DC capabilities (number of NR DL carriers)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL EN-DC with 1 NR DL carriers | 38.101-3, 5.5B |  |
| 2 | DL EN-DC with 2 NR DL carriers | 38.101-3, 5.5B |  |
| 3 | DL EN-DC with 3 NR DL carriers | 38.101-3, 5.5B |  |
| 4 | DL EN-DC with 4 NR DL carriers | 38.101-3, 5.5B |  |
| 5 | DL EN-DC with 5 NR DL carriers | 38.101-3, 5.5B |  |
| 6 | DL EN-DC with 6 NR DL carriers | 38.101-3, 5.5B |  |
| 7 | DL EN-DC with 7 NR DL carriers | 38.101-3, 5.5B |  |

Table A.4.3.2B.2.0-2: Uplink EN-DC capabilities (for one or more of the supported EN-DCconfigurations)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL EN-DC with 2 carriers | 38.101-3, 5.5B |  |
| 2 | UL EN-DC with 3 carriers | 38.101-3, 5.5B |  |
| 3 | UL EN-DC with 4 carriers | 38.101-3, 5.5B |  |
| 4 | UL EN-DC with 5 carriers | 38.101-3, 5.5B |  |
| 5 | UL EN-DC with 6 carriers | 38.101-3, 5.5B |  |
| 6 | UL EN-DC with 7 carriers | 38.101-3, 5.5B |  |
| 7 | UL EN-DC with 8 carriers | 38.101-3, 5.5B |  |

Table A.4.3.2B.2.0-2A: Uplink EN-DC capabilities (number of NR UL carriers)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL EN-DC with 1 NR UL carriers | 38.101-3, 5.5B |  |
| 2 | UL EN-DC with 2 NR UL carriers | 38.101-3, 5.5B |  |
| 3 | UL EN-DC with 3 NR UL carriers | 38.101-3, 5.5B |  |
| 4 | UL EN-DC with 4 NR UL carriers | 38.101-3, 5.5B |  |
| 5 | UL EN-DC with 5 NR UL carriers | 38.101-3, 5.5B |  |
| 6 | UL EN-DC with 6 NR UL carriers | 38.101-3, 5.5B |  |
| 7 | UL EN-DC with 7 NR UL carriers | 38.101-3, 5.5B |  |

<< UNCHANGED PARTS SKIPPED >>

A.4.3.2B.3	NE-DC Physical Layer Baseline Implementation Capabilities

A.4.3.2B.3.0	General NE-DC capabilities

Table A.4.3.2B.3.0-1: Downlink NE-DC capabilities (for one or more of the supported NE-DC configurations)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NE-DC with 2 carriers | 38.101-3, 5.5B |  |
| 2 | DL NE-DC with 3 carriers | 38.101-3, 5.5B |  |
| 3 | DL NE-DC with 4 carriers | 38.101-3, 5.5B |  |
| 4 | DL NE-DC with 5 carriers | 38.101-3, 5.5B |  |
| 5 | DL NE-DC with 6 carriers | 38.101-3, 5.5B |  |

Table A.4.3.2B.3.0-1A: Downlink NE-DC capabilities (number of NR DL carriers)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL NE-DC with 1 NR DL carriers | 38.101-3, 5.5B |  |

Table A.4.3.2B.3.0-2: Uplink NE-DC capabilities (for one or more of the supported NE-DCconfigurations)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NE-DC with 2 carriers | 38.101-3, 5.5B |  |

Table A.4.3.2B.3.0-2A: Uplink NE-DC capabilities (number of NR UL carriers)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NE-DC with 1 NR UL carriers | 38.101-3, 5.5B |  |

A.4.3.2B.3.1	Inter-band NE-DC within FR1

A.4.3.2B.3.1.1	Inter-band NE-DC within FR1 (two bands)

Table A.4.3.2B.3.1.1-1: Downlink Bandwidth Class Combination capabilities for Inter-band NE-DC within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.3.1.1-2)

| Item | DL inter-band NE-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | Inter-band NE-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4a.1 | pc_DL_inter_band_NE_DC_FR1_2B_Class_A_A |  |
| 2 | Inter-band NE-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4a.1 | pc_DL_inter_band_NE_DC_FR1_2B_Class_(2A)_A |  |
| 3 | Inter-band NE-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4a.1 | pc_DL_inter_band_NE_DC_FR1_2B_Class_A_C |  |
| 4 | Inter-band NE-DC within FR1 BW Class Combination A_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4a.1 | pc_DL_inter_band_NE_DC_FR1_2B_Class_A_(2A) |  |

Table A.4.3.2B.3.1.1-1a: Uplink Bandwidth Class Combination capabilities for Inter-band NE-DC within FR1 and two bands (for one or more of the supported configurations in Table A.4.3.2B.3.1.1-2)

| Item | UL inter-band NE-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band NE-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4a.1 | pc_UL_inter_band_NE_DC_FR1_2B_Class_A_A |  |

Table A.4.3.2B.3.1.1-2: Supported Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration / Item (Note 1) | Release | Supported | Supported NE-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) |
| --- | --- | --- | --- | --- |
| DC_n28A_3A | Rel-17 |  |  |  |
| DC_n28A_3C | Rel-17 |  |  |  |
| DC_n28A_39A | Rel-17 |  |  |  |
| DC_n28A_39C | Rel-17 |  |  |  |
| NOTE 1: Notation used for inter-band NE-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4a.1-1, e.g. ‘DC_n28A_3A’ indicates NE-DC operation on NR band n28 with NR DL Bandwidth Class A and E-UTRA band 3 with E-UTRA DL CA Bandwidth Class A. |  |  |  |  |

Table A.4.3.2B.3.1.1-3: Inter-band NE-DC within FR1 (two bands) PC3 UE RF Baseline Implementation Capabilities

| Item | NE-DC configuration | Inter-band NE-DC within FR1 (two bands) PC3 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_n28A_3A DC_n28A_3C | NR Frequency band: 703–748 MHz (UL),758 MHz–803 MHz (DL) LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-3, 6.2B.1.3a | Rel-17 | pc_nrBand28_Band3_ PC3_Supp |  |
| 2 | DC_n28A_39A DC_n28A_39C | NR Frequency band: 703–748 MHz (UL),758 MHz–803 MHz (DL) LTE Frequency band: 1880-1920 MHz | 38.101-3, 6.2B.1.3a | Rel-17 | pc_nrBand28_Band39_ PC3_Supp |  |

Table A.4.3.2B.3.1.1-3a: Inter-band NE-DC within FR1 (two bands) NR part power class UE RF Baseline Implementation Capabilities

| Item | EN-DC configuration | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Supported NR part power class |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DC_n28A_3A DC_n28A_3C | DC_n28A_3A NR part power class DC_n28A_3C NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_nrBand28_Band3_powerClassNRPart_r16 |  |
| 2 | DC_n28A_39A DC_n28A_39C | DC_n28A_39A NR part power class DC_n28A_39C NR part power class | 38.306, 4.2.7.1 | Rel-16 | pc_nrBand28_Band39_powerClassNRPart_r16 |  |

Table A.4.3.2B.3.1.1-4: UE Power Class implementation Capabilities for inter-band NE-DC within FR1 (two bands)

| Item | UE Power Class implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UE Power Class 3 for Inter-band NE-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3a | Applicable to the bands in Table A.4.3.2B.3.1.1-3 |

<< END OF CHANGES >>