3GPP TSG-RAN WG5 Meeting #94-e 	R5-220375

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1272 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Tx TC for 4 Rel-17 combos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test requirement for 4 Verizon required ENDC combos are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 6.2B.1.3 requirement for ENDC combos DC_2A_n77A, DC_5A_n77A, DC_13A_n77A, DC_66A_n77A Updated Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1.3, 6.2B.4.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

### <Start of Changes>

#### 6.2B.1.3	UE Maximum Output Power for Inter-Band EN-DC within FR1

6.2B.1.3.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC operating on FR1.

6.2B.1.3.3	Minimum conformance requirements

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  | 23 | +2/-3 |
| DC_1A_n77A |  |  | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A DC_1A_n84A_ULSUP-FDM_n78A |  |  | 23 | +2/-3 |
| DC_1A_n79A |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-31 |
| DC_2A_n41A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-31 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n7A |  |  | 23 | +2/-31 |
| DC_3A_n28A |  |  | 23 | +2/-31 |
| DC_3A_n40A |  |  | 23 | +2/-31 |
| DC_3A_n41A, DC_3C_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n51A |  |  | 23 | +2/-31 |
| DC_3A_n77A |  |  | 23 | +2/-31 |
| DC_3A_n78A | 266 | +2/-31 | 23 | +2/-31 |
| DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A |  |  | 23 | +2/-31 |
| DC_3A_n82A |  |  | 23 | +2/-31 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-31 |
| DC_5A_n66A |  |  | 23 | +2/-31 |
| DC_5A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-31 |
| DC_7A_n51A |  |  | 23 | +2/-31 |
| DC_7A_n66A |  |  | 23 | +2/-31 |
| DC_7A_n78A DC_7C_n78A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  | 23 | +2/-31 |
| DC_8A_n41A, DC_8A_n81A_ULSUP-TDM, DC_8A_n81A_ULSUP-FDM |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A |  |  | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n79A |  |  | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n28A DC_20A_n83A |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  | 23 | +2/-3 |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 23 | +2/-3 |
| DC_21A_n78A |  |  | 23 | +2/-3 |
| DC_21A_n79A |  |  | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A n51A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A |  |  | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_39A_n41A | 26 | +2/-31 | 23 | +2/-2 |
| DC_39A_n78A |  |  | 23 | +2/-31 |
| DC_39A_n79A | 26 | +2/-3 | 23 | +2/-31 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A |  |  | 23 | +2/-3 |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2/-31 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2/-31 |
| DC_41A_n79A DC_41C_n79A | 26 | +2/-31 | 23 | +2/-31 |
| DC_42A_n51A |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-31 |
| DC_66A_n41A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A |  |  | 23 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

### <Skipped unhanged sections>

6.2B.1.3.5	Test requirements

For test ID 1~6 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in Table 6.2B.1.3.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA carier and NR carrier respectively for Power class 3, and in Table 6.2B.1.3.5-2a for Power class 2.

For test ID 1~6 the maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed by Power Class 3 and tolerance in Table 6.2B.1.3.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed by Power Class 3 and tolerance in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA carier and NR carrier respectively.

Table 6.2B.1.3.5-1: Maximum output power for inter-band EN-DC (two bands), for overlapping UL transmission

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n40A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A DC_1A_n84A_ULSUP-FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n5A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_2A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n66A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_2A_n71A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT3 |
| DC_2A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n7A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n28A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n40A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n41A, DC_3C_n41A | 268 | +2+TT/-3-TT3 | 23 | +2+TT/-3-TT3 |
| DC_3A_n51A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n77A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n78A | 268 | +2+TT/-3-TT3 | 23 | +2 +TT/-3-TT3 |
| DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n82A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_5A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_5A_n40A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_5A_n66A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_5A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_5A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n28A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_7A_n51A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_7A_n66A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_7A_n78A DC_7C_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n1A |  |  |  | +2 +TT/-3-TT |
| DC_8A_n3A |  |  |  | +2 +TT/-3-TT |
| DC_8A_n40A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_8A_n41A, DC_8A_n81A_ULSUP-TDM, DC_8A_n81A_ULSUP-FDM |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_14A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_14A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n8A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n28A DC_20A_n83A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_25A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_39A_n41A | 26 | +2/-31 | 23 | +2 +TT/-3-TT3 |
| DC_39A_n78A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_39A_n79A | 26 | +2/-3 | 23 | +2 +TT/-3-TT3 |
| DC_40A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_41A_n79A DC_41C_n79A | 26 | +2/-31 | 23 | +2 +TT/-3-TT3 |
| DC_42A_n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_48A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n5A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_66A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n71A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP -FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3.5-2. NOTE 2: TT applies to overall output power with E-UTRA UL transmission overlapping with NR UL transmission in time, and its value is the maximum TT among all E-UTRA and NR UL carriers. For detailed values refer to Table 6.2B.1.3.5-3. NOTE 3: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 5: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 6: Power Class 3 is the default power class unless otherwise stated. NOTE 7: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

Table 6.2B.1.3.5-2: Void

Table 6.2B.1.3.5-2a: Maximum output power for inter-band EN-DC (two bands), for non-overlapping UL transmission for power class 2 (Rel-16 and forward)

| EN-DC configuration | Carrier | Power class 2 (dBm) | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_2A_n77A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n41A | NR carrier | 26 | +2+TT/-32-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-22-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n78A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_5A_n77A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_13A_n77A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_39A_n41A | NR carrier | 26 | +2+TT/-32-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-22-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_39A_n79A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_41A_n79A DC_41C_n79A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-22-TT | UE indicates PC2 on E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_66A_n77A | NR carrier | 26 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3.5-3. NOTE 2: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |

Table 6.2B.1.3.5-3: Test Tolerance for UE maximum output power (Overlapping UL transmission)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

### <Skipped unhanged sections>

##### 6.2B.4.2.3	Inter-Band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n3 | 1 | 0.3 |
|  | n3 | 0.3 |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n40 | 1 | 0.5 |
|  | n40 | 0.5 |
| DC_1_n51 | 1 | 0.6 |
|  | n51 | 0.6 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
|  | n5 | 0.3 |
| DC_2_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_2_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n71 | 2 | 0.3 |
|  | n71 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 2 | 0.6 |
|  | n7 | 0.8 |
| DC_2_n78 | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n1 | 3 | 0.3 |
|  | n1 | 0.3 |
| DC_3_n7 | 3 | 0.5 |
|  | n7 | 0.5 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n40 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3_n41 | 3 | 0.5 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3_n51 | 3 | 0.3 |
|  | n51 | 0.3 |
| DC_3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_5_n2, DC_5-5_n2 | 5 | 0.3 |
|  | n2 | 0.3 |
| DC_5_n40 | 5 | 0.3 |
|  | n40 | 0.3 |
| DC_5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n77 | 5 | 0.6 |
|  | n7 | 0.8 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n1 | 7 | 0.6 |
|  | n1 | 0.5 |
| DC_7_n3 | 7 | 0.5 |
|  | n3 | 0.5 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n51 | 7 | 0.3 |
|  | n51 | 0.3 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n78, DC_7-7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_n1 | 8 | 0.3 |
|  | n1 | 0.3 |
| DC_8_n3 | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_8_n40 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n41 | 8 | 0.3 |
|  | n41 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12_n5 | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n66 | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_12_n78 | 12 | 0.5 |
|  | n78 | 0.8 |
| DC_13_n2 | 13 | 0.3 |
|  | n2 | 0.3 |
| DC_13_n66 | 13 | 0.3 |
|  | n66 | 0.3 |
| DC_13_n77 | 13 | 0.5 |
|  | n7 | 0.8 |
| DC_14_n2 | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_14_n66 | 14 | 0.3 |
|  | n66 | 0.3 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n1 | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_20_n3 | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n51 | 20 | 0.5 |
|  | n51 | 0.5 |
| DC_20_n77 | 20 | 0.6 |
|  | n77 | 0.8 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26_n77 | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n51 | 28 | 0.5 |
|  | n51 | 0.5 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30_n5 | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_38_n78 | n78 | 0.5 |
| DC_39_n41 | 39 | 0.5 |
|  | n41 | 0.5 |
| DC_39_n78 | 39 | 0.3 |
|  | n78 | 0.8 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n1 | n1 | 0.5 |
|  | 40 | 0.5 |
| DC_40_n415 | 40 | 0.5 |
|  | n41 | 0.5 |
| DC_40_n77 | n77 | 0.5 |
| DC_40_n78 | n78 | 0.56 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_42_n51 | 42 | 0.6 |
|  | n51 | 0.8 |
| DC_48_n5 | 48 | 0.3 |
|  | n5 | 0.3 |
| DC_48_n66 | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.6 |
|  | n7 | 0.8 |
| DC_66_n78 | 66 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 - 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### <End of Changes>