3GPP TSG-RAN WG5 Meeting #94-e	R5-220463

# Electronic Meeting, , 21th February  – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0233 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.11 - Default MCVideo Transmission Control Messages and other Information Elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Message structure and content incorrect in transmission control messages -see clause 9 in TS 24.581 - Specific message content has no conditions yet - Acc. to C1-192530 for TS 24.581 Rel 14.5 and reflected in C1-193658 for Rel 16 transmission cancel request and response are void as they are not used in specification - Default message content as specified in TS 24.581 is not necessarily to be listed, media multiplexing is not used (see also C1-194684) - ON-NETWORK is default condition - Incorrect references |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Message structure and content corrected - Conditions introduced in Specific message content for consistent usage similar to clause 5.5.6 - Table 5.5.11.2.10 transmission cancel request and response voided - Default message content as specified in TS 24.581 removed similar to e.g. clause 5.5.6 media plane control messages for MCPTT - Condition ON-NETWORK removed in messages - References corrected - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid, inconsistent or incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.11	Default MCVideo Transmission Control Messages and other Information Elements

Considerations in regard to describing specific values:

-	SSRC

-	Synchronization SouRCe (SSRC) values are used in most of the messages specified in clause 5.5.6. The SSRC value is randomly chosen by the participant in, and globally unique within, an RTP session as specified in IETF RFC 3550 [76]. Because the value chosen by the UE (MCVideo client) cannot be controlled, specifying a "hard coded" value to be used by the SS (MCVideo Server) or the SS-UE (MCVideo Client) is prone to triggering a collision by choosing a value which may be the same as the one chosen by the UE. How to resolve SSRC collisions is described in IETF RFC 3550 [76] however, resolving them as part of the MCVideo test case definitions e.g. in TS 36.579-6 [84] is not foreseen and is left to the test implementation.

-	For the purposes of default and specific messages definition throughout the present specification, as well as, throughout the rest of the MCPTT conformance test specifications e.g. the TS 36.579-6 [84] no explicit SSRC values are defined and instead the following notation is used to clarify the messages origin/destination:

-	When there is no danger for misunderstanding the notation 'The SSRC of the message sender' and the 'The SSRC of the intended recipient of the message' are used whereas the "sender" and the "recipient" are to be understood in the context of the test i.e. the test entities being involved to exchange messages.

#### 5.5.11.1	Transmission Control Specific Messages Sent by the Transmission Participant

##### 5.5.11.1.1	Transmission Request

Table: 5.5.11.1.1-1 Transmission Request

| Derivation Path: TS 24.581 [88] Table 9.2.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP-header |  |  |  |  |
| Subtype | “00000” | Transmission Request | TS 24.581 [88] clause 9.2.4 and Table 9.2.2.1-1 |  |
|  | “10000” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant sending the Transmission Request message. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV0 |  |  |  |
| Transmission Priority |  |  | TS 24.581 [88] clause 9.2.3.2 |  |
| Transmission Priority Value | Not present or any allowed value | If present, a value between '0' and '255' where '0' is the lowest priority and '255' is the highest priority. If the Transmission Priority field is not included in the message the default priority is used as the Transmission Priority value. The value of the default priority is '0'. The default priority is sometimes referred to as normal priority. |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo ID of the transmission participant sending the Transmission Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88] clause 9.2.3.21 | FA |

##### 5.5.11.1.2	Transmission Release

Table: 5.5.11.1.2-1 Transmission Release

| Derivation Path: TS 24.581 [88] Table 9.2.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP-header |  |  |  |  |
| Subtype | “00010” | Transmission Release | TS 24.581 [88] clause 9.2.7 and Table 9.2.2.1-1 |  |
|  | “10010” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant with permission to send media. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo ID of the transmission participant sending the Transmission Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Transmission Indicator |  |  |  |  |
| Transmission Indicator | "1000000000000000" | Normal call | TS 24.581 [88] clause 9.2.3.11 |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.1.3	Queue Position Request

Table: 5.5.11.1.3-1 Queue Position Request

| Derivation Path: TS 24.581 [88] Table 9.2.11-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Queue Position Request | TS 24.581 [88] clause 9.2.11 and 9.2.2.1-1 |  |
|  | “10011” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant requesting information about its position in the transmission request queue. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76], |  |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |

##### 5.5.11.1.4	Receive Media Request

Table: 5.5.11.1.4-1 Receive Media Request

| Derivation Path: TS 24.581 [88] Table 9.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Receive Media Request | TS 24.581 [88] clause 9.2.14 and 9.2.2.1-1 |  |
|  | “10100” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant requesting the reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV0 |  |  |  |
| User ID |  | The User ID field is used to carry the identity of the user who is requesting the reception of the media Note: If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| User ID | px_MCVideo_ID_User_A |  | TS 24.581 [88] Table 9.2.3.8-2 |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Reception Priority |  | Describes the level of reception priority requested in a Reception Request message or granted in a Reception Granted message. The max reception priority that can be requested in a Reception Request message is negotiated between the transmission control participant and the transmission control server | TS 24.581 [88] clause 9.2.3.19 and 6.2.5.3.3 |  |
| Reception Priority value | not present or any allowed value | The reception priority ('0' to '255') where '0' is the lowest reception priority and '255' is the highest reception priority. If the Reception Priority field is not included in the message the default reception priority is used as the Reception Priority value. The value of the default reception priority is '0'. The default reception priority is sometimes referred to as normal reception priority. |  |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88]clause 9.2.3.21 | FA |

##### 5.5.11.1.5	Void

##### 5.5.11.1.6	Remote Transmission Request

Table: 5.5.11.1.6-1 Remote Transmission Request

| Derivation Path: TS 24.581 [88] Table 9.2.22-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00111” | Remote Transmission Request | TS 24.581 [88] clause 9.2.22 and Table 9.2.2.1-1 |  |
|  | “10111” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant requesting the reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV0 |  |  |  |
| Remote ID |  | Carries the identity of the user whose media transmission is requested. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |

##### 5.5.11.1.7	Remote Transmission Cancel Request

Table: 5.5.11.1.7-1 Remote Transmission Cancel Request

| Derivation Path: TS 24.581 [88] Table 9.2.24-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01000” | Remote transmission cancel request | TS 24.581 [88] clause 9.2.24 and Table 9.2.2.1-1 |  |
|  | “11000” |  |  | ACK |
| SSRC | The SSRC of the message sender. | The SSRC field carries the SSRC of the transmission participant requesting the reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV0 |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the identity of the user whose media transmission is requested for cancellation. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |

#### 5.5.11.2	Transmission Control Specific Messages Sent by the Transmission Control Server

##### 5.5.11.2.1	Transmission Granted

Table: 5.5.11.2.1-1 Transmission Granted

| Derivation Path: TS 24.581 [88] Table 9.2.5-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| RTCP |  |  |  |  |  |
| Subtype | “00000” | Transmission granted | TS 24.581 [88] clause 9.2.5 and 9.2.2.1-2 |  |  |
|  | “10000” |  |  | ACK |  |
| SSRC | The SSRC of the message sender | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV1 | Transmission Control messages sent by the transmission control server and transmission control participant |  |  |
| Duration |  |  | TS 24.581 [88] clause 9.2.3.3 |  |
| Duration | "00000000 10000000" | 128 sec (an arbitrary value) |  |  |
| SSRC of granted transmission participant | The SSRC of the intended recipient of the message | Notation in accordance with clause 5.5.6.1. . | IETF RFC 3550 [76] |  |
| Transmission priority | Not present | If the Transmission Priority field is not included in the message the default priority (='0') is used as the Floor Priority value |  |  |
| User ID | Not present |  |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Queue Size | Not present |  |  |  |
| Queue Size | "0" | the number of queued MCVideo clients in the MCVideo call | TS 24.581 [88] clause 9.2.3.15 | OFF-NETWORK |
| SSRC of queued floor participant | Not present |  |  |  |
|  | The SSRC of queued transmission participant | Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] | OFF-NETWORK |
| Queued User ID | Not present |  |  |  |
|  | px_MCVideo_ID_User_C | MCVideo ID of the transmission participant in the queue | TS 24.581 [88] clause 9.2.3.14 | OFF-NETWORK |
| Queue Info | Not present |  |  |  |
| Queue Info |  | queue position and granted transmission priority in the queue |  | OFF-NETWORK |
| queue position info | "00000001" |  | TS 24.581 [88] clause 9.2.3.5 |  |
| queue priority level | "00000000" |  | TS 24.581 [88] clause 9.2.3.2 |  |
| Transmission Indicator |  |  | TS 24.581 [88] Table 9.2.3.11-2 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.2	Transmission Rejected

Table: 5.5.11.2.2-1 Transmission Rejected

| Derivation Path: TS 24.581 [88] Table 9.2.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00001” | Transmission rejected | TS 24.581 [88] clause 9.2.6 and 9.2.2.1-2 |  |
|  | “10001” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV1 |  |  |  |
| Reject Cause |  | Includes the reason for the rejecting the transmission request and can be followed by a text-string explaining why the transmission request was rejected. Therefore the length of the packet will vary depending on the size of the application dependent field. | TS 24.581 [88] clause 9.2.3.4 |  |
| Reject Cause | "255" | Th<Reject cause> value set to '255' indicates that the transmission control server does not grant the transmission request due to the transmission control server local policy. | TS 24.581 [88] clause 9.2.6.2 |  |
| Reject Cause Phrase | “Other reason” | A text string encoded the text string in the SDES item CNAME. | IETF RFC 3550 [76] |  |
| User ID | Not present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the requesting transmission participant to which the Transmission Rejected message is sent. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.3	Transmission Arbitration Taken

Table: 5.5.11.2.3-1 Transmission Arbitration Taken

| Derivation Path: TS 24.581 [88] Table 9.2.8-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00010” | Transmission Arbitration Taken | TS 24.581 [88] clause 9.2.8 and 9.2.2.1-2 |  |
|  | “10010” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV1 |  |  |  |
| Granted Party's Identity |  | Identifies the MCVideo user that is granted to send media. | TS 24.581 [88] clause 9.2.3.6 |  |
| Granted Party's Identity | px_MCVideo_ID_User_A | If the length of the <Granted Party's> value is not (2 + multiple of 4) bytes, the Granted Party's Identity field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is set to zero. The padding bytes are ignored by the receiver. |  |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause9.2.3.7 |  |
| Permission to Request the Transmission | "1" | Coded as follows: 0 The receiver is not permitted to request transmission. 1 The receiver is permitted to request transmission |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the transmission participant sending the Transmission Arbitration Taken message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Message Sequence Number |  | Used to bind a number of Transmission Arbitration Taken or bind a number of Transmission Idle messages together | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | "1" | The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call . |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| SSRC of Granted Transmission Participant | SSRC of granted transmission participant: | Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |

##### 5.5.11.2.4	Transmission Arbitration Release

Table: 5.5.11.2.4-1 Transmission Arbitration Release

| Derivation Path: TS 24.581 [88] Table 9.2.9-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Subtype | “00011” | Transmission Arbitration Release | TS 24.581 [88] clause 9.2.9 and 9.2.2.1-2 |  |
|  | “10011” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV1 |  |  |  |
| Granted Party's Identity |  | Identifies the MCVideo user that is granted to send media. | TS 24.581 [88] clause 9.2.3.6 |  |
| Granted Party's Identity | px_MCVideo_ID_User_A | If the length of the <Granted Party's> value is not (2 + multiple of 4) bytes, the Granted Party's Identity field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is set to zero. The padding bytes are ignored by the receiver. |  |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause 9.2.3.7 |  |
| Permission to Request the Transmission | "1" | Coded as follows: 0 The receiver is not permitted to request transmission. 1 The receiver is permitted to request transmission |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the transmission participant sending the Transmission Arbitration Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Message Sequence Number |  | Used to bind a number of Transmission Arbitration Taken or bind a number of Transmission Idle messages together | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | "1" | The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| SSRC of Granted Transmission Participant | The SSRC of the intended recipient of the message | Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |

##### 5.5.11.2.5	Transmission Revoked

Table: 5.5.11.2.5-1 Transmission Revoked

| Derivation Path: TS 24.581 [88] Table 9.2.10-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Transmission Revoked | TS 24.581 [88] clause 9.2.10 and 9.2.2.1-2 |  |
|  | “10100” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV1 |  |  |  |
| Reject Cause |  | Message includes <Reject Cause> cause value in the Reject Cause field explaining why the transmission control server wants the transmission participant to stop sending media and can be followed by additional information. Therefore the length of the packet can vary depending on the value of the rejection cause. | TS 24.581 [88] clause 9.2.3.4 |  |
| Reject Cause Value | "7" | The <Reject Cause> value set to '7' indicates that the MCVideo client's permission to send a media is being queued. No additional information is included. | TS 24.581 [88] clause 9.2.10.2 |  |
| Reject Cause Phrase | "Queue the transmission" | A text string encoded the text string in the SDES item CNAME. | TS 24.581 [88] clause 9.2.10.2 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.6	Queue Position Info

Table: 5.5.11.2.6-1 Queue Position Info

| Derivation Path: TS 24.581 [88] Table 9.2.12-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| RTCP |  |  |  |  |  |  |  |  |  |
| Subtype |  | “00101” |  | Queue Position Info |  |  | TS 24.581 [88] clause 9.2.12 and 9.2.2.1-2 |  |  |  |
|  |  | “10101” |  |  |  |  |  |  | ACK |  |
| SSRC |  | The SSRC of the message sender |  | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. |  | IETF RFC 3550 [76] |  |  |  |
| name |  | MCV1 |  |  |  |  |  |  |  |
| User ID |  | Not present |  |  |  |  |  |  |  |
| User ID |  |  |  | The User ID field is used in off-network only. The User ID field carries the MCVideo user ID of the transmission participant sending the Queue Position Info message. |  | TS 24.581 [88] clause 9.2.3.8 |  | OFF-NETWORK |  |
| User ID |  | px_MCVideo_ID_User_A |  | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |  |  |  |
| SSRC of Queued Transmission Participant |  | Not present |  |  |  |  |  |  |  |
| SSRC of Queued Transmission Participant |  | The SSRC of the queued transmission participant |  | Applicable only in off-network and shall carry the SSRC of the queued transmission participant. Notation in accordance with clause 5.5.6.1. |  | IETF RFC 3550 [76]. |  | OFF-NETWORK |  |
| Queued User ID |  | Not present |  |  |  |  |  |  |  |
| Queued User ID |  | px_MCVIDEO_ID_User_B |  | Used in off-network only. The Queued User ID field carries the MCVideo ID of the queued transmission control participant. |  | TS 24.581 [88] clause 9.2.3.8 |  | OFF-NETWORK |  |
| Queue Info |  |  |  | Defines the queue position and granted transmission control priority in the queue. |  | TS 24.581 [88] clause 9.2.3.5 |  |  |  |
| Queue Position Info |  | "1" |  | value is a binary value |  |  |  |  |  |
| Queue Priority Level |  | "0" |  | value consists of 8 bit parameter giving the transmission priority. The value of the default priority is ‘0’. The default priority is sometimes referred to as normal priority. |  | TS 24.581 [88] clause 9.2.3.2 |  |  |  |
| Track Info |  | Not present |  | The MCVideo call does not involve a non-controlling MCVideo function |  | TS 24.581 [88] clause 9.2.3.13 |  |  |  |
| Transmission Control Indicator |  |  |  |  |  | TS 24.581 [88] clause 9.2.3.11 |  |  |  |
| Transmission Indicator |  | "1000000000000000" |  | Normal call |  |  |  |  |  |  |
|  |  | "0001000000000000" |  | Emergency call |  |  |  |  | EMERGENCY-CALL |  |
|  |  | "0000100000000000" |  | Imminent peril call |  |  |  |  | IMMPERIL-CALL |  |

##### 5.5.11.2.7	Media Transmission Notification

Table: 5.5.11.2.7-1 Media Transmission Notification

| Derivation Path: TS 24.581 [88] Table 9.2.13-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00110” | Media Transmission Notification | TS 24.581 [88] clause 9.2.13 and 9.2.2.1-2 |  |
|  | “10110” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV1 |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the requesting transmission participant to which the Transmission Rejected message is sent. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media |  |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause 9.2.3.7 |  |
| Permission to Request the Transmission | "1" | Coded as follows: 0 The receiver is not permitted to request transmission. 1 The receiver is permitted to request transmission |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal Call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88]clause 9.2.3.21 | FA |

##### 5.5.11.2.8	Receive Media Response

Table: 5.5.11.2.8-1 Receive Media Response

| Derivation Path: TS 24.581 [88] Table 9.2.15-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00111” | Receive Media Response | TS 24.581 [88] clause 9.2.15 and 9.2.2.1-2 |  |
|  | “10111” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant requesting the reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76], |  |
| name | MCV1 |  |  |  |
| Result |  | Indicates whether media reception is possible as per the request | TS 24.581 [88] clause 9.2.3.17 |  |
| Result | "1" | 0 - The receiver is not permitted (rejected) to receive the media transmission. 1 - The receiver is permitted (granted) to receive the media transmission. |  |  |
| Reject Cause | Not present | Includes the reason for the rejecting the media receive request and can be followed by a text-string explaining why the media receive request was rejected. Therefore the length of the packet will vary depending on the size of the application dependent field |  |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.9	Media Reception Notification

Table: 5.5.11.2.9-1 Media Reception Notification

| Derivation Path: TS 24.581 [88] Table 9.2.16-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01000” | Media Reception Notification | TS 24.581 [88] clause 9.2.16 and 9.2.2.1-2 |  |
|  | “11000” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV1 |  |  |  |
| User ID |  | The User ID field carries the MCVideo ID of the user transmitting the media. Note: If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88] clause 9.2.3.21 | FA |

##### 5.5.11.2.10	Void

##### 5.5.11.2.11	Transmission Cancel Request Notify

Table: 5.5.11.2.11-1 Transmission Cancel Request Notify

| Derivation Path: TS 24.581 [88] Table 9.2.19-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01010” | Transmission Cancel Request Notify | TS 24.581 [88] clause 9.2.19 and 9.2.2.1-2 |  |
|  | “11010” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |

##### 5.5.11.2.12	Remote Transmission Response

Table: 5.5.11.2.12-1 Remote Transmission Response

| Derivation Path: TS 24.581 [88] Table 9.2.23-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01011” | Remote Transmission Response | TS 24.581 [88] clause 9.2.23 and 9.2.2.1-2 |  |
|  | “11011” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |

##### 5.5.11.2.13	Remote Transmission Cancel Response

Table: 5.5.11.2.13-1 Remote Transmission Cancel Response

| Derivation Path: TS 24.581 [88] Table 9.2.25-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01100” | Remote Transmission Cancel Response | TS 24.581 [88] clause 9.2.25 and 9.2.2.1-2 |  |
|  | “11100” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant requesting the reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |

##### 5.5.11.2.14	Media Reception Override Notification

Table: 5.5.11.2.14-1 Media Reception Override Notification

| Derivation Path: TS 24.581 [88] Table 9.2.28-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01101” | Media Reception Override Notification | TS 24.581 [88] clause 9.2.28 and 9.2.2.1-2 |  |
|  | “11101” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission participant requesting the reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |
| User ID | 16-bit binary value | Carries the identity of the user who is requesting the reception of the media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| Overriding ID | 16-bit binary value | Carries the identity of the user of the overriding media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Overridden ID | 16-bit binary value | Carries the identity of the user of the overridden media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |

##### 5.5.11.2.15	Transmission End Notify

Table: 5.5.11.2.15-1 Transmission End Notify

| Derivation Path: TS 24.581 [88] Table 9.2.29-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01110” | Transmission End Notify | TS 24.581 [88] clause 9.2.29 and 9.2.2.1-2 |  |
|  | “11110” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |
| User ID |  | Carries the identity of the user whose media transmission has been released | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.6.1 | IETF RFC 3550 [76]. |  |

##### 5.5.11.2.16	Transmission Idle

Table: 5.5.11.2.16-1 Transmission Idle

| Derivation Path: TS 24.581 [88] Table 9.2.30-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01111” |  | TS 24.581 [88] clause 9.2.2.1-2 |  |
| SSRC | The SSRC of the message sender | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | “MCV1” | Transmission Control messages sent by the Transmission Control Server and the Transmission Control Participant. |  |  |
| Message Sequence Number |  |  |  |  |
| Message Sequence Number | "1" | value is a binary value. The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

#### 5.5.11.3	Transmission control specific messages sent by both the transmission control server and transmission control participant

##### 5.5.11.3.1	Transmission End Request

Table: 5.5.11.3.1-1 Transmission End Request

| Derivation Path: TS 24.581 [88] Table 9.2.20-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| RTCP |  |  |  |  |
| Subtype | “00000” | Transmission End Request | TS 24.581 [88] clause 9.2.20 and 9.2.2.1-3 |  |
|  | “10000” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV2 |  |  |  |
| User ID |  | The User ID field is used to carry the identity of the user whose media transmission is requested to be terminated. |  |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |
| Reject Cause | not present | Includes the reason explaining why the transmission control server wants the transmission participant to stop sending media | TS 24.581 [88] clause 9.2.3.4 |  |

##### 5.5.11.3.2	Transmission End Response

Table: 5.5.11.3.2-1 Transmission End Response

| Derivation Path: TS 24.581 [88] Table 9.2.21-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| RTCP |  |  |  |  |
| Subtype | “00001” | Transmission End Response | TS 24.581 [88] clause 9.2.21 and 9.2.2.1-3 |  |
|  | “10001” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76]. |  |
| name | MCV2 |  |  |  |
| User ID |  | The User ID field is used to carry the identity of the user whose media transmission is requested to be terminated. |  |  |
| User ID | px_MCVideo_ID_User_A | If the length of the <User ID> value is not (2 + multiple of 4) bytes User ID field shall be padded to (2 + multiple of 4) bytes. The value of the padding bytes is to zero. The padding bytes are ignored by the receiver. |  |  |

##### 5.5.11.3.3	Media Reception End Request

Table: 5.5.11.3.3-1 Media Reception End Request

| Derivation Path: TS 24.581 [88] Table 9.2.26-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00010” | ##### Media Reception End Request | TS 24.581 [88] clause 9.2.26 and 9.2.2.1-3 |  |
|  | “10010” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server or the transmission control participant requesting the end of reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV2 |  |  |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.3.4	Media Reception End Response

Table: 5.5.11.3.4-1 Media Reception End Response

| Derivation Path: TS 24.581 [88] Table 9.2.27-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Media Reception End Response | TS 24.581 [88] clause 9.2.27 and 9.2.2.1-3 |  |
|  | “10011” |  |  | ACK |
| SSRC | The SSRC of the message sender | The SSRC field carries the SSRC of the transmission control server or the transmission control participant requesting the end of reception of the media from another user. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV2 |  |  |  |
| SSRC of transmitter | The SSRC of the user transmitting the media | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |

##### 5.5.11.3.5	Transmission Control Ack

Table: 5.5.11.3.5-1: Transmission Control Ack

| Derivation Path: TS 24.581 [88] Table 9.2.31-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Transmission Control Ack | TS 24.581 [88] clause 9.2.31 and 9.2.2.1-3 |  |
| SSRC | The SSRC of the message sender | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. Notation in accordance with clause 5.5.6.1. | IETF RFC 3550 [76] |  |
| name | MCV2 |  |  |  |
| Source |  |  | TS 24.581 [88] clause 9.2.3.12 |  |
| Source | "2" | the controlling MCVideo function is the sender of the message |  | DOWNLINK |
|  | “0” | the transmission participant is the sender of the message |  | UPLINK |
| Message name |  |  | TS 24.581 [88] clause 9.2.3.18 |  |
| Message Name | the message name of the received message | value is as coded as an ascii name field |  |  |
| Message type |  |  | TS 24.581 [88] clause 9.2.3.10 |  |
| Message Type | "0001xxxx" | Message Type of the transmission control messages which reuqested the acknowledgement |  |  |

| Condition | Explanation |
| --- | --- |
| UPLINK | The message is sent from the UE |
| DOWNLINK | The message is sent from the SS |
| For further conditions see table 5.5.6.1-1 |  |