3GPP TSG-RAN WG5 Meeting #94-e	R5-220491

# Electronic Meeting, , 21th February  – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0038 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCVideo Test Case 5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As discussed at the TTCN workshop #55 (R5w210310 clause 2.3) MCVideo Client Configuration Test cases shall re-use MCPTT test cases as far as possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pre-test conditions, test procedure sequence and specific message contents replaced by references to MCPTT test case 5.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid or incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3	Configuration / Group Affiliation / Remote change / De-affiliation / Home MCVideo system

5.3.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information, that the UE (MCVideo Client) is allowed to be affiliated }

ensure that {
  when { MCVideo User requests for current affiliation status and to subscribe to affiliation status changes for the MCVideo User }

then { UE (MCVideo Client) requests to subscribe to affiliation status changes for the MCVideo User by sending the SS (MCVideo Server) a SIP SUBSCRIBE message and starts informing the MCVideo User of any affiliation status changes for the MCVideo User after the subscription is accepted }

}

(2)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information, that the UE (MCVideo Client) is allowed to be affiliated }

ensure that {
  when { MCVideo User requests to affiliate to an MCVideo group }

then { UE (MCVideo Client) requests to affiliate to a MCVideo group by sending the SS (MCVideo Server) a SIP PUBLISH message }

}

(3)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information, that the UE (MCVideo Client) is allowed to be affiliated }

ensure that {
  when { MCVideo User requests for current affiliation status and to subscribe to affiliation status changes for a target user }

then { UE (MCVideo Client) requests to subscribe to affiliation status changes for the target user by sending the SS (MCVideo Server) a SIP SUBSCRIBE message and starts informing the MCVideo User of any affiliation status changes for the target user after the subscription is accepted }

}

(4)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information that the UE (MCVideo Client) is allowed to make affiliation changes for another user }

ensure that {
  when { MCVideo User requests that a target user be affiliated to an MCVideo group via mandatory mode }

then { UE (MCVideo Client) requests that a target user be affiliated to an MCVideo group via mandatory mode by sending the SS (MCVideo Server) a SIP PUBLISH message }

}

(5)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information that the UE (MCVideo Client) is allowed to make affiliation changes for another user }

ensure that {
  when { MCVideo User requests that a target user be de-affiliated to an MCVideo group via mandatory mode }

then { UE (MCVideo Client) requests that a target user be de-affiliated to an MCVideo group via mandatory mode by sending the SS (MCVideo Server) a SIP PUBLISH message }

}

(6)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information that the UE (MCVideo Client) is allowed to make affiliation changes for another user }

ensure that {
  when { MCVideo User requests that a target user be affiliated to an MCVideo group via negotiated mode }

then { UE (MCVideo Client) requests that a target user be affiliated to an MCVideo group via negotiated mode by sending the SS (MCVideo Server) a SIP MESSAGE }

}

(7)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information, that the UE (MCVideo Client) is allowed to be affiliated }

ensure that {
  when { MCVideo User requests to de-subscribe to affiliation status changes for a target user }

then { UE (MCVideo Client) requests to de-subscribe to affiliation status changes for a target user by sending the SS (MCVideo Server) a SIP SUBSCRIBE message }

}

(8)

with { UE (MCVideo Client) already affiliated with a MCVideo group }

ensure that {
  when { MCVideo User requests to de-affiliate from an MCVideo group }

then { UE (MCVideo Client) requests to de-affiliate from an MCVideo group by sending the SS (MCVideo Server) a SIP PUBLISH message }

}

(9)

with { UE (MCVideo Client) already provisioned with the group information or a pointer to the group information, that the UE (MCVideo Client) is allowed to be affiliated }

ensure that {
  when { MCVideo Server requests that the MCVideo User choose to affiliate to an MCVideo group via negotiated mode by sending a SIP MESSAGE }

then { UE (MCVideo Client) accepts to affiliate to a MCVideo group by sending the SS (MCVideo Server) a SIP PUBLISH message }

}

5.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.281, clauses 8.2.1.2, 8.2.1.3, 8.2.1.4, and 8.2.1.5. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.281, clause 8.2.1.2]

In order:

-	to indicate that an MCVideo user is interested in one or more MCVideo group(s) at an MCVideo client;

-	to indicate that the MCVideo user is no longer interested in one or more MCVideo group(s) at the MCVideo client;

-	to refresh indication of an MCVideo user interest in one or more MCVideo group(s) at an MCVideo client due to near expiration of the expiration time of an MCVideo group with the affiliation status set to the "affiliated" state received in a SIP NOTIFY request in subclause 8.2.1.3;

-	to send an affiliation status change request in mandatory mode to another MCVideo user; or

-	any combination of the above;

the MCVideo client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [11], IETF RFC 3903 [12], and IETF RFC 3856 [13].

In the SIP PUBLISH request, the MCVideo client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCVideo function serving the MCVideo user;

2)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body. In the application/vnd.3gpp.mcvideo-info+xml MIME body, the MCVideo client shall include the <mcvideo-request-uri> element set to the MCVideo ID of the MCVideo user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14];

4)	if the targeted MCVideo user is interested in at least one MCVideo group at the targeted MCVideo client, shall set the Expires header field according to IETF RFC 3903 [12], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [15].

5)	if the targeted MCVideo user is no longer interested in any MCVideo group at the targeted MCVideo client, shall set the Expires header field according to IETF RFC 3903 [12], to zero; and

6)	shall include an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 8.3.1. In the MIME body, the MCVideo client:

a)	shall include all MCVideo groups where the targeted MCVideo user indicates its interest at the targeted MCVideo client;

b)	shall include the MCVideo client ID of the targeted MCVideo client;

c)	shall not include the "status" attribute and the "expires" attribute in the <affiliation> element; and

d)	shall set the <p-id> child element of the <presence> root element to a globally unique value.

The MCVideo client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [11].

[TS 24.281, clause 8.2.1.3]

NOTE 1:	The MCVideo UE also uses this procedure to determine which MCVideo groups the MCVideo user successfully affiliated to.

In order to discover MCVideo groups:

1)	which the MCVideo user at an MCVideo client is affiliated to; or

2)	which another MCVideo user is affiliated to;

the MCVideo client shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [11], IETF RFC 3856 [13], and IETF RFC 6665 [16].

In the SIP SUBSCRIBE request, the MCVideo client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCVideo function serving the MCVideo user;

2)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body. In the application/vnd.3gpp.mcvideo-info+xml MIME body, the MCVideo client shall include the <mcvideo-request-uri> element set to the MCVideo ID of the targeted MCVideo user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14];

4)	if the MCVideo client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [16], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [15].

5)	if the MCVideo client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [16], to zero; and

6)	shall include an Accept header field containing the application/pidf+xml MIME type; and

7)	if requesting MCVideo groups where the MCVideo user is affiliated to at the MCVideo client, shall include an application/simple-filter+xml MIME body indicating per client restrictions of presence event package notification information according to subclause 8.3.2.

In order to re-subscribe or de-subscribe, the MCVideo client shall generate an in-dialog SIP SUBSCRIBE request according to 3GPP TS 24.229 [11], IETF RFC 3856 [13], and IETF RFC 6665 [16]. In the SIP SUBSCRIBE request, the MCVideo client:

1)	if the MCVideo client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [16], to 4294967295;

NOTE 3:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [15].

2)	if the MCVideo client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [16], to zero; and

3)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request according to 3GPP TS 24.229 [11], IETF RFC 3856 [13], and IETF RFC 6665 [16], if SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-user affiliation information constructed according to subclause 8.3.1, then the MCVideo client shall determine affiliation status of the MCVideo user for each MCVideo group at the MCVideo client(s) in the MIME body. If the <p-id> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP NOTIFY request is included, the <p-id> element value indicates the SIP PUBLISH request which triggered sending of the SIP NOTIFY request.

[TS 24.281, clause 8.2.1.4]

NOTE:	Procedure for sending affiliation status change request in negotiated mode to several target MCVideo users is not supported in this version of the specification.

Upon receiving a request from the MCVideo user to send an affiliation status change request in negotiated mode to a target MCVideo user, the MCVideo client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [11] and IETF RFC 3428 [17]. In the SIP MESSAGE request, the MCVideo client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCVideo function serving the MCVideo user;

2)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body. In the application/vnd.3gpp.mcvideo-info+xml MIME body, the MCVideo client shall include the <mcvideo-request-uri> element set to the MCVideo ID of the target MCVideo user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14] in the SIP MESSAGE request;

4)	shall include an application/vnd.3gpp.mcvideo-affiliation-command+xml MIME body as specified in Annex F.4; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [11].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCVideo client shall indicate to the user that the request has been delivered to an MCVideo client of the target MCVideo user.

[TS 24.281, clause 8.2.1.5]

Upon receiving a SIP MESSAGE request containing:

1)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Asserted-Service header field according to IETF RFC 6050 [14]; and

2)	an application/vnd.3gpp.mcvideo-affiliation-command+xml MIME body with a list of MCVideo groups for affiliation under the <affiliate> element and a list of MCVideo groups for de-affiliation under the <de-affiliate> element;

then the MCVideo client:

1)	shall send a 200 (OK) response to the SIP MESSAGE request;

2)	shall seek confirmation of the list of MCVideo groups for affiliation and the list of MCVideo groups for de-affiliation, resulting in an accepted list of MCVideo groups for affiliation and an accepted list of MCVideo groups for de-affiliation; and

3)	if the user accepts the request:

a)	shall perform affiliation for each entry in the accepted list of MCVideo groups for affiliation for which the MCVideo client is not affiliated, as specified in subclause 8.2.1.2; and

b)	shall perform de-affiliation for each entry in the accepted list of MCVideo groups for de-affiliation for which the MCVideo client is affiliated, as specified in subclause 8.2.1.2.

5.3.3	Test description

5.3.3.1	Pre-test conditions

Same pre-test conditions as for MCPTT test case 5.3 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCVideo"

5.3.3.2	Test procedure sequence

Same test procedure sequence as for MCPTT test case 5.3 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCVideo"

5.3.3.3	Specific message contents

Same specific message contents as for MCPTT test case 5.3 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCVideo"

-	Condition MCVIDEO is used for all messages.