3GPP TSG-RAN WG5 Meeting #94-e	R5-220501

# Electronic Meeting, , 21th February  – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0020 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCData Test Case 5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As discussed at the TTCN workshop #55 (R5w210310 clause 2.3) MCData Client Configuration Test cases shall re-use MCPTT test cases as far as possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pre-test conditions, test procedure sequence and specific message contents replaced by references to MCPTT test case 5.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid or incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.4	Configuration / Determination of MCData Service Settings / Current Active MCData Settings / De-subscribe

5.4.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {
  when { MCData User requests to verify the currently active MCData service settings or to discover MCData service settings }

then { UE (MCData Client) sends a SIP SUBSCRIBE message to find the MCData service settings and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having already subscribed to find the MCData service settings }

ensure that {
  when { MCData User requests to re-subscribe for MCData service settings }

then { UE (MCData Client) sends a SIP SUBSCRIBE message to re-subscribe for the MCData service settings and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(3)

with { UE (MCDATA Client) having already subscribed to find the MCData service settings }

ensure that {
  when { MCData User requests to de-subscribe for MCData service settings }

then { UE (MCData Client) sends a SIP SUBSCRIBE message to de-subscribe for the MCData service settings and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

5.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.282 clause 7.2.4. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated these are Rel-14 requirements.

[TS 24.282, clause 7.2.4]

In order to discover MCData service settings of another MCData client of the same MCData user or to verify the currently active MCData service settings of this MCData client, the MCData client shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [5], IETF RFC 6665 [36], and IETF RFC 4354 [35].

In the SIP SUBSCRIBE request, the MCData client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCData function serving the MCData user;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData client shall include the <mcdata-request-uri> element set to the MCData ID of the MCData user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7];

4)	shall set the Event header field to the 'poc-settings' value;

5)	shall include an Accept header field containing the "application/poc-settings+xml" MIME type;

6)	if the MCData client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [36], to 4294967295; and

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

7)	if the MCData client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [36], to zero.

In order to re-subscribe or de-subscribe, the MCData client shall generate an in-dialog SIP SUBSCRIBE request according to 3GPP TS 24.229 [5], IETF RFC 6665 [36], IETF RFC 4354 [35]. In the SIP SUBSCRIBE request, the MCData client:

1)	shall set the Event header field to the 'poc-settings' value;

2)	shall include an Accept header field containing the "application/poc-settings+xml" MIME type;

3)	if the MCData client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [36], to 4294967295; and

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

4)	if the MCData client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [36], to zero.

Upon receiving a SIP NOTIFY request according to 3GPP TS 24.229 [5], IETF RFC 6665 [36] and IETF RFC 4354 [35], that contains an application/poc-settings+xml MIME body the MCData client shall cache:

1)	the <am-settings> element of the poc-settings+xml MIME body for each MCData client identified by the "id" attribute according to IETF RFC 4354 [35] as the current Answer-mode indication of that MPCTT client; and

2)	the <selected-user-profile-index> element of the poc-settings+xml MIME body for each MCData client identified by the "id" attribute according to IETF RFC 4354 [35] as the active MCData user profile of that MCData client.

5.4.3	Test description

5.4.3.1	Pre-test conditions

Same pre-test conditions as for MCPTT test case 5.5 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCData"

5.4.3.2	Test procedure sequence

Same test procedure sequence as for MCPTT test case 5.5 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCData"

5.4.3.3	Specific message contents

Same specific message contents as for MCPTT test case 5.5 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCData"

-	Condition MCDATA is used for all messages.