3GPP TSG-RAN5 Meeting #94-e	R5-220770

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1539 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of Tx TCs having impact on ETSI EN 301 908-25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some of the Editorial issues in clause 6 need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial corrections of table numbering issues for TC 6.2F.3.3, TC 6.3A.3.0.3.2, Table 6.3A.3.1.4.3-1 to 6.3A.3.1.4.3-7, TC 6.3C.4.2.5, TC 6.3D.3.4.2, Table 6.4.2.1a.5, TC6.4.2.2.3, TC 6.4A.2.3.1.5, Table 6.5.3.3.5.15, Table 6.5A.1.1.4.3, Table 6.5A.2.2.1.4.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some TCs will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.5 6.2F.3.3 6.2F.3.3.5 6.2F.3.5 6.3.3.2.3 6.3A.3.0.3.2 6.3A.3.1.4.3 6.3C.4.2.5 6.3D.3.4.2 6.4.2.1a.5 6.4.2.2.3 6.4A.2.3.1.5 6.5.3.3.5.15 6.5A.1.1.4.3 6.5A.2.2.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of changes>

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2F.1.5-1.

Table 6.2F.1.5-1: Maximum Output Power test requirement for Power Class 5

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2+TT/-3-TT |
| n96 |  |  |  |  |  |  | 20 | +2+TT/-3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

Table 6.2F.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 5.925GHz | 5.925GHz < f ≤ 7.125GHz |
| --- | --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB | TBD |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB | TBD |

For the UE which supports inter-band NR CA configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

Editor's note [IS]: s.t. missing???

<Unchanged Text Skipped>

6.2F.3.3	Minimum conformance requirements

6.2F.3.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max (MPR, A-MPR) where MPR is defined in clause 6.2F.2.

Table 6.2F.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.3.1-1A.

Table 6.2F.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_28 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.2 |
| NS_29 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.3 |
| NS_30 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.4 |
| NS_31 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.5 |
| NS_53 |  | n96 | 20, 40, 60, 80 |  | 6.2F.3.3.6 |
| NS_54 |  | n96 | 20, 40, 60, 80 |  | 6.2F.3.3.7 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2F.3.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  |  |
| n96 | NS_01 | NS_53 | NS_54 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2F.3.3.5	A-MPR for NS_31

When "NS_31" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.5-1.

Table 6.2F.3.3.5-1: A-MPR for NS_31 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 4.0 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 64 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.5 |
| CP-OFDM | QPSK |  | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centred at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

<Unchanged Text Skipped>

6.2F.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2F.3.5-1.

Table 6.2F.3.5-1: Test requirement for Power Class 5

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBD |  |  |  |  |  |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2F.3.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2F.3.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 5.925GHz | 5.925GHz < f ≤ 7.125GHz |
| --- | --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB | TBD |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB | TBD |

For the UE which supports inter-band NR CA configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

[Editor's note [IS]: s.t. missing???]

<Unchanged Text Skipped>

6.3.3.2.3	Minimum conformance requirements

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS. ON/OFF scenarios include: the beginning or end of DTX, measurement gap, contiguous, and non-contiguous transmission, etc.

6.3.3.3	Transmit power time mask for slot and short or subslot boundaries

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can’t provide enough resolution to identify non-conformant UEs. Therefore, the minimum requirement is not testable.

6.3.3.4	PRACH time mask

6.3.3.4.1	Test purpose

To verify that the PRACH time mask meets the requirements given in 6.3.3.4.5.

The time mask for PRACH time mask defines the transient period(s)allowed between transmit OFF power and transmit ON power when transmitting the PRACH.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

<Unchanged Text Skipped>

6.3A.3.0.3.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power when the capability uplinkTxSwitchingPowerBoosting is present and the IE uplinkTxSwitchingPowerBoosting is enabled, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3A.3.0.3.2-1a and Figure 6.3A.3.0.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3A.3.0.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.0.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

<Unchanged Text Skipped>

6.3A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

Table 6.3A.3.1.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[5], Table 4.6.3-90 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 |  |  |
| } |  |  |  |

Table 6.3A.3.1.4.3-2: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms10 |  | FR1_15kHz |
| nrofDownlinkSlots | 6 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 14 |  | FR1_60kHz |
| nrofDownlinkSymbols | 10 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 12 |  | FR1_60kHz |
| nrofUplinkSlots | 3 |  | FR1_15kHz,FR1_30kHz |
|  | 4 |  | FR1_60kHz |
| nrofUplinkSymbols | 4 |  | FR1_30kHz |
|  | 2 |  | FR1_15kHz, |
|  | 8 |  | FR1_60kHz |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

Table 6.3A.3.1.4.3-3: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k2 | 4 |  | FR1_15kHz,FR1_30kHz |
|  | 6 |  | FR1_60kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_15kHz |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_30kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |

Table 6.3A.3.1.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

Table 6.3A.3.1.4.3-5: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.3A.3.1.4.3-6: P-Max

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  |  |

Table 6.3A.3.1.4.3-7:  FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

<Unchanged Text Skipped>

6.3C.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3.4.3.4.2 should satisfy the test requirements specified in Table 6.3C.4.2.5-1 thru 6.3C.4.2.5-5.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ± (6.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3C.4.2.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 15 RBs | TPC=+1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 15 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3C.1. |  |  |  |  |  |  |  |

Table 6.3C.4.2.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 |  | Sub-frames before RB change | Fixed = 15 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 15 RBs to 1 RB | TPC=-1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3C.1. |  |  |  |  |  |  |  |

Table 6.3C.4.2.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 1RB to 20 RBs | TPC=+1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 20 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 50 RBs | TPC=+1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 50 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3C.1. |  |  |  |  |  |  |  |

Table 6.3C.4.2.5-4: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1RBs | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 20 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 20 RBs to 1 RB | TPC=-1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 50 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 50 RBs to 1 RB | TPC=-1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3C.1. |  |  |  |  |  |  |  |

Table 6.3C.4.2.5-5: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 5 | 15 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 15 | TPC=0dB | 11.76 | 10dB ≤ ΔP < 15dB | 11.76 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 10,15,20, 25,30,40 |  | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 15 | 3 | Alternating 1 and 20 | TPC=0dB | 13.01 | 10dB ≤ ΔP < 15dB | 13.01 +/- (4 + TT) |
|  |  | 4 | Alternating 1 and 50 | TPC=0dB | 16.99 | 15dB ≤ ΔP | 16.99 +/- (5 + TT) |
| Note 1: The starting resource block shall be RB# 0. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3C.1. |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.3D.3.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3D.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slots 8 for15kHz SCS, on slots 8 and 18 for 30kHz SCS and on slots 17 and 37 for 60kHz SCS. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAXlevel.

<Unchanged Text Skipped>

6.4.2.1a.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, shall not exceed the values in Table 6.4.2.1a.5-1.

The PUSCH, derived in Annex E.4.6.2, shall not exceed the values in Table 6.4.2.1a.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4.2.1a.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | [10] + TT |
| 256 QAM | % | [8] + TT |
| Note 1: TT is defined in Table 6.4.2.1.5-2. |  |  |

Table 6.4.2.1a.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 0 |
| 256 QAM | % | 0.3 for 15 dBm < PUL 0.8 for -25 dBm < PUL≤ 15 dBm 1.1 for -40dBm ≤ PUL ≤ -25dBm |

<Unchanged Text Skipped>

6.4.2.2.3	Minimum conformance requirements

Carrier leakage is an additive sinusoid waveform whose frequency is the same as the modulated waveform carrier frequency. The measurement interval is one slot in the time domain.

In the case that uplink sharing, the carrier leakage may have 7.5 kHz shift with the carrier frequency.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.4.2.2.3-1.

Table 6.4.2.2.3-1: Requirements for Carrier Leakage

| Parameter | Relative Limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 |
| -30 dBm ≤ Output power < 0 dBm | -20 |
| -40 dBm ≤ Output power < -30 dBm | -10 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4.2.2.

<Unchanged Text Skipped>

6.4A.2.3.1.5	Test requirement

Each of the [20] In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.4A.2.3.1.5-1.

Table 6.4A.2.3.1.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | + TT |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28+TT | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25+TT | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28+TT | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25+TT | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20+TT | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10+TT | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRBis defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Section 5.3). NOTE 7: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: is the limit specified in Table 6.4.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power normalized by the number of allocated RBs, measured in dBm. |  |  |  |  |

<Unchanged Text Skipped>

6.5.3.3.5.15	Test requirement (network signalling value “NS_47”)

Table 6.5.3.3.5.15-1: Additional requirement for "NS_47"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 30 |  |
| 2530 ≤ f ≤ 2535 | -25 | 1 MHz |
| 2505 ≤ f ≤ 2530 | -30 | 1 MHz |

<Unchanged Text Skipped>

6.5A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.1.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |

Table 6.5A.1.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  | Power class 3 and Bandwidth class B |
|  | 10 |  | Power class 3 and Bandwidth class C |

<Unchanged Text Skipped>

6.5A.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.2.2.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 3, 4, 7 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 5, 6, 13 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 1, 2, 8-12 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 14 |

<End of changes>