3GPP TSG-RAN5 Meeting #94-e	R5-220792

# Electronic Meeting, 21st Feb– 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0694 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 6.2D.2 for ULFPTx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Requirements of ULFPTx mode is sepecified by RAN4. The corresponding test needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test configuration and test requirement of ULFPTx into MPR for UL-MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ULFPTx requirements are not covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2D.2	UE maximum output power reduction for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- OTA test procedure for UL MIMO is still under investigation

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

6.2D.2.1	Test purpose

The number of RB identified in 6.2D.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2D.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting UL MIMO.

6.2D.2.3	Minimum conformance requirements

6.2D.2.3.1	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 1

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.1.3.1-1 is specified in sub-clause 6.2.2.3.1. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

6.2D.2.3.2	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 2

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.1.3.2-1 is specified in sub-clause 6.2.2.3.2. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

6.2D.2.3.3	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 3

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.1.3.3-1 is specified in sub-clause 6.2.2.3.3. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

6.2D.2.3.4	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 4

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.1.3.4-1 is specified in sub-clause 6.2.2.3.4. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

6.2D.2.3.5	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 5

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.1.3.4-1 is specified in sub-clause 6.2.2.3.4. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2D.2.

6.2D.2.4	Test description

6.2D.2.4.1	Initial condition

Same initial condition in clause 6.2.2.4.1, with following exceptions:

-	Instead of Table 6.2.2.4.1-1 use Table 6.2D.2.4.1-1.

-	Instead of Table 6.2.2.4.1-2 use Table 6.2D.2.4.1-2.

-	Instead of Table 6.2.2.4.1-3 use Table 6.2D.2.4.1-3.

-	Instead of Table 6.2.2.4.1-7 use Table 6.2D.2.4.1-4.

-	Instead of Table 6.2.2.4.1-8 use Table 6.2D.2.4.1-5.

-	Instead of Table 6.2.2.4.1-9 use Table 6.2D.2.4.1-6.

Table 6.2D.2.4.1-1: Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 3 | Low |  |  |  | CP-OFDM 64 QAM | 2@0 |
| 4 | High |  |  |  | CP-OFDM 64 QAM | 2@NRB-2 |
| 5 | Low |  |  |  | CP-OFDM 64 QAM | 7@0 |
| 6 | High |  |  |  | CP-OFDM 64 QAM | 7@NRB-7 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2. |  |  |  |  |  |  |

Table 6.2D.2.4.1-2: Test Configuration Table (Power Class 1, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 2 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 3 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 4 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 5 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 6 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 7 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 8 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 10 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 11 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 12 | Mid |  |  |  | CP-OFDM 64 QAM | Inner_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2D.2.4.1-3: Test Configuration Table (Power Class 1, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 2 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 3 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 4 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 5 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 6 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 7 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 8 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 10 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 11 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2D.2.4.1-4: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz t |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-5: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 4 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 5 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 8 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 9 | Mid |  |  |  | CP-OFDM 64 QAM | Inner_Full |
| 10 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-6: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-7: Test Configuration Table for ULFPTx (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 3 | Low |  |  |  | CP-OFDM 64 QAM | 2@0 |
| 4 | High |  |  |  | CP-OFDM 64 QAM | 2@NRB-2 |
| 5 | Low |  |  |  | CP-OFDM 64 QAM | 7@0 |
| 6 | High |  |  |  | CP-OFDM 64 QAM | 7@NRB-7 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2. NOTE 2: Test IDs 1 ~ 6 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-8: Test Configuration Table for ULFPTx (Power Class 1, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range,High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 5 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 6 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 7 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 8 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 9 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 10 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 11 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 12 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 13 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 14 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full_Region2 |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. NOTE 2: Test IDs 16 ~ 26 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-9: Test Configuration Table for ULFPTx (Power Class 1, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full_Region2 |
| 5 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 8 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 12 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. NOTE 2: Test IDs 16 ~ 26 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-10: Test Configuration Table for ULFPTx (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highes supported channel bandwidth that ≤ 200 MHz t |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-11: Test Configuration Table for ULFPTx (Power Class 2, 3 and 4, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 4 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 7 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full |
| 8 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 9 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 10 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 12 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 13 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 14 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 15 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 16 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 17 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 18 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 19 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 20 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: Test IDs 11 ~ 20 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |  |  |

Table 6.2D.2.4.1-12: Test Configuration Table for ULFPTx (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: Test IDs 13 ~ 21 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |  |  |

6.2D.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level. Allow at least BEAM_SELECT_WAIT_TIME (Note 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2.2.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

7.	If UE supports ULFPTx, repeat test steps 1~6 with UL RMC according to Table 6.2D.2.4.1-7 through Table 6.2D.2.4.1-12. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2.2.4.1-1 to Table 6.2.2.4.1-9, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.2D.2.5	Test requirements

The test requirement is the same as in clause 6.2.2.5.

For UE supporting ULFPTx, the test requirement is the same as in clause 6.2.2.5.

<End of Changes >