3GPP TSG RAN Meeting #94-e	                   R5-220811

Electronic Meeting, 21st February – 4th March 2022

Agenda item:	5.3.19.9

Source:	Huawei, HiSilicon

Title: 	Discussion on minimum test time for 1% residual BLER

Document for:	DISCUSSION AND ENDORSEMENT

1	Background

The minimum test time for PDSCH demodulation test cases have been specified in TS 38.521-4 for various fading channel and RMCs [1-3]. Most test cases have the minimum requirement of 70% or 30% throughput.

In Rel-16, RAN4 specified a new type of PDSCH demodulation test cases with minimum requirement of 1% residual BLER. The relevant test cases are:

5.2.2.1.6	Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.14	Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme

Whether existing minimum test time could apply to 1% residual BLER requirement shall be studied.

2	Discussion

The new type of PDSCH demodulation test cases is different from legacy test cases in following two aspects:

- The performance of legacy test cases are counted based on ACK/NACK/DTX, but the new test cases are counted based on successfully received TB block.

- In the two test cases, a TB block is repeated twice in each transmission opportunity. One test case is repeated within a slot, and another is repeated inter-slot.

Based on above, the converging time is expected to be different from legacy test cases even under the same fading condition. The minimum test time needs to be re-calculated based on simulation.

Observation 1: Existing minimum test time for legacy PDSCH demodulation test cases couldn’t apply to 1% residual BLER.

In order to decide a proper minimum test time, the link simulation was run for both test cases. The BLER corridor is selected as 1% +/- 0.1%.  30 random seeds were used, and simulation continues for 70 seconds for each random seed.

The simulation result shows that the residual BLER doesn’t converge to 1% +/-0.1% for all the seeds. In the worst case, the residual BLER converge to 2% in the end of the 70 seconds. Considering the min test time for legacy test cases is 45 seconds for 10Hz scenarios and 20 seconds for 100Hz scenarios, further extending the test time to above 70 seconds would not be a good option.

Proposal 1: The minimum test time for 1% residual BLER is decided as 70 seconds.

According to the result of link simulation, there is a chance that the DUT would be failed due to the limited test time. Some compensation needs to be considered. Similar as the legacy test cases, SNR uncertainty due to finite test time could be a choice. However considering the offset between the worst case BLER and the target BLER, i.e. 2% vs 1%, the SNR uncertainty required might be relatively large. Therefore a combined way forward could be considered, i.e. define uncertainty for both SNR and BLER. Based on observation on existing simulations, a 0.5% increase of BLER requirement and a 0.7dB increase of SNR might be enough to ensure the BLER convergence. Due to the time limitation, no simulation is executed to verify this set of parameters, but we may take this as the starting point for further checking.

Proposal 2: Decide the BLER uncertainty due to limited time as [0.5%] and SNR uncertainty due to limited time as [0.7dB]. Brackets are to be removed in RAN5#95-e based on further checking.

3	Summary

This document discusses how to decide the minimum test time for 1% residual BLER test cases. Following observation and proposals are provided for RAN5 to consider:

Observation 1: Existing minimum test time for legacy PDSCH demodulation test cases couldn’t apply to 1% residual BLER.

Proposal 1: The minimum test time for 1% residual BLER is decided as 70 seconds.

Proposal 2: Decide the BLER uncertainty due to limited time as [0.5%] and SNR uncertainty due to limited time as [0.7dB]. Brackets are to be removed in RAN5#95-e based on further checking.

4	References

[1]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[2]	R5-194978, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	R5-204738	FR2 Demod MU due to finite test time