3GPP TSG-RAN5 Meeting #94-e	R5-220837

# Electronic Meeting, 21 February – 4 March 2022

Source:	Qualcomm Incorporated

Title:	Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE

# Document for:	Agreement

Agenda Item:	6.3.1.20

## 1.	Introduction

Standalone Non-Public networks (SNPN) were introduced in Rel-16 and work is currently commencing in RAN5 for conformance specifications, which are tracking core specifications. There may be a possibility that a UE can be SNPN ONLY UE, i.e. designed to access Standalone Network only [4]. For such UE, we may have to provide coverage for applicable legacy NR (Rel-15) test cases. As per agreed w/f [5], initial scoping was captured in an xls attached with discussion document [6], Chapter 6 [1] test cases were scoped initially.

## 2.	Discussion

Below scoping steps have been completed:

Scoping -1 --> Since SNPN is only applicable for Standalone test cases, all option-2\standalone\5GC core Rel-15 test cases were chosen.

Scoping 2 --> Due to the lack of conformance requirement, below features and corresponding test cases will be marked as "Not Applicable"

Following procedures are not defined in Rel-16 for a UE operating in SNPN, for ex-

- PLMN Selection

- IRAT to LTE

- CAG

- Autonomous Search

- IMS

- Emergency

- HO, Roaming.. etc

Scoping 3-->Each test case to be analyzed to determine what is the minimum test environment that will make it eligible for execution. For example - NR Cell Change to SNPN; Sys Info Combination; SIB1, SIB10 changes etc

Scoping 4 -->Mark the status of each test cases, for ex- a 100% means it is fully scoped out and can be executed with changes identified in #3 above

Open items --> How to apply the test cases towards execution i.e possible update of TS 38.523-2 and handle TTCN level changes. Below proposals are discussed as way forward.

## 3.	Proposal/Way Forward

Since the issue of applying legacy test to a “vertical” UE (SNPN Only, IIoT only, NR-U only or RedCap only, MultiSIM only) applies a broad array of R-16, R-17 WI currently in progress, a more holistic approach shall be considered in case we have to realize addition of such scope in TS 38.523-1 and TS38.523-2.

Proposal#1 – Once the scoping is complete and verified, create a “bis” test case of each impacted test case in TS38.523-1. For example –

6.1.2.1	Cell selection / Qrxlevmin & Cell reselection (Intra NR)

6.1.2.1b	Cell selection / Qrxlevmin & Cell reselection (Intra NR) – SNPN Only

6.1.2.1b1	Test Purpose (TP)

Same as TC 6.1.2.1 but applied to SNPN Only UE

6.1.2.1b.2	Conformance requirements

Same as TC 6.1.2.1 but applied to SNPN Only UE

6.1.2.1b.3	Test description

6.1.2.1b.3.1	Pre-test conditions

Same as TC 6.1.2.1 with the following differences:

-	NR Cell Change to SNPN belonging to the same NID; Sys Info Combination; SIB1, SIB10 changes

6.1.2.1b.3.2	Test procedure sequence

Same as TC 6.1.2.1

Similarly, update the applicability of this separate test case in TS38.523-2. For example –

| 6.1.2.1 | Cell selection / Qrxlevmin & Cell reselection (Intra NR) | Rel-15 | C21 | UEs supporting 5G Core |
| --- | --- | --- | --- | --- |
| 6.1.2.1b | Cell selection / Qrxlevmin & Cell reselection (Intra NR) – SNPN Only | Rel-16 | Cxx | UEs supporting 5G Core and SNPN Only |
| 6.1.2.2 | Cell selection / Qqualmin / Intra NR / Serving cell becomes non-suitable (Srxlev > 0, Squal < 0) | Rel-15 | C21 | UEs supporting 5G Core |

The drawback of this approach is that it will too many copies of the applicable test and may confuse the reader.

Proposal#2 – Create a new cl\chapter in TS38.523-1, for ex-

12 	 	Applicable R-15 tests for verticals

12.1		SNPN Only UE

12.1.1		Idle Mode Tests

12.1.1.1 	Cell selection / Qrxlevmin & Cell reselection (Intra NR) – SNPN Only

12.1.1.1	Cell selection / Qrxlevmin & Cell reselection (Intra NR) – SNPN Only

12.1.1.11	Test Purpose (TP)

Same as TC 6.1.2.1 but applied to SNPN Only UE

12.1.1.1.2	Conformance requirements

Same as TC 6.1.2.1 but applied to SNPN Only UE

12.1.1.1.3	Test description

12.1.1.1.3.1	Pre-test conditions

Same as TC 6.1.2.1 with the following differences:

-	NR Cell Change to SNPN belonging to the same NID; Sys Info Combination; SIB1, SIB10 changes

12.1.1.1.3.2	Test procedure sequence

Same as TC 6.1.2.1

Similarly, update the applicability of this separate test case in TS38.523-2 as well in a new cl. For example –

| 12 | Applicable R-15 tests for verticals |  |  |  |
| --- | --- | --- | --- | --- |
| 12.1 | SNPN Only UE |  |  |  |
| 12.1.1 | Idle Mode Tests |  |  |  |
| 12.1.1.1 | Cell selection / Qrxlevmin & Cell reselection (Intra NR) – SNPN Only | Rel-15 | Cxx | UEs supporting 5G Core and SNPN only |

This approach will allow to put all such verticals (SNPN, NR-U, RedCap etc) into a separate chapter with each vertical having each own sub-clause (For 12.1 – SNPN Only UE). This will result in a cleaner spec and readability for legacy tests will be maintained, we will then have dedicated space and procedure for allowing such test case definition for any newer vertical feature.

Proposal#3 – Not all verticals may need extensive certification, given the lack of industry direction and projection, wait for all such test case addition until advised by certification forums. If a need arises, follow proposal-2, WP owners can complete the scoping of such tests then.

## 4.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[3]	TS 38.508-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception User Equipment (UE) conformance specification;

[4]	TS 38.300: NR; NR and NG-RAN Overall description; Stage-2; Rel-16 (cl 16.6.1)

[5]	R5-213408: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#91e

[6]	R5-214736: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#92e