3GPP TSG-RAN5 Meeting #94-e	R5-220887

# Online, 21st Feb – 4th Mar 2022

Title:							Introduction of DL MU for FR2 absolute power tolerance

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

There is a difference in the derivation of MU for absolute power tolerance between FR1 and FR2. This paper proposes to introduce combined MU of UL and DL MU into FR2 absolute power tolerance to solve the difference.

# 2.	Discussion

It was agreed to apply the same MU as minimum output power for FR2 absolute power tolerance [1]. However, combined MU derived from UL and DL MU is used for FR1 absolute power tolerance [2]. We propose to use combined MU in FR2 as well. The derivation formula and concrete values are shown below.

The System Uncertainty = SQRT(UL MU2 + DL MU2)

Table 1 MU value for FR2 absolute power tolerance (PC3, Max device size ≤ 30 cm, FR2a & FR2b)

|  | UL MU (Same as Minimum output power) | DL MU (Same as Reference sensitivity power level) | The System Uncertainty |
| --- | --- | --- | --- |
| NTC testing | +/- 6.15 dB | +/- 5.19 dB | +/- 8.05 dB |
| ETC testing | +/- 6.41 dB | +/- 5.45 dB | +/- 8.42 dB |

Proposal 1 : Use +/- 8.05 dB for NTC testing and +/- 8.42 dB for ETC testing for FR2 absolute power tolerance MU.

In addition to MU, we propose to introduce DL uncertainty into the calculation of TT.

TT for Absolute power tolerance = SQRT(TT for Minimum output power2 + TT for Reference sensitivity2)

Table 2 TT value for FR2 absolute power tolerance (PC3, Max device size ≤ 30 cm, FR2a & FR2b)

|  | Frequency sub range | CBW | TT for Minimum output power | TT for Reference sensitivity | TT for Absolute power tolerance |
| --- | --- | --- | --- | --- | --- |
| NTC testing | FR2a | 50 MHz | 4.21 dB | 2.34 dB | 4.82 dB |
|  |  | 100 MHz | 2.52 dB |  | 3.44 dB |
|  |  | 200 MHz | 0.66 dB |  | 2.43 dB |
|  |  | 400 MHz | 0 dB |  | 2.34 dB |
|  | FR2b | 50 MHz | 1.17 dB |  | 2.62 dB |
|  |  | ≥100 MHz | 0 dB |  | 2.34 dB |
| ETC testing | FR2a | 50 MHz | 4.37 dB | 2.45 dB | 5.01 dB |
|  |  | 100 MHz | 2.68 dB |  | 3.63 dB |
|  |  | 200 MHz | 0.82 dB |  | 2.58 dB |
|  |  | 400 MHz | 0 dB |  | 2.45 dB |
|  | FR2b | 50 MHz | 1.33 dB |  | 2.79 dB |
|  |  | ≥100 MHz | 0 dB |  | 2.45 dB |

Proposal 2 : Adopt the TT values in Table 2 for FR2 absolute power tolerance.

The testability at each test point has already been analyzed assuming target level +/- power tolerance is the worst case [3]. When Proposal 1 is endorsed, it is necessary to add DL MU to the worst case assumption of the possible actual level. However, target level +/- power tolerance +/- DL MU can be below than the lower limit of measurable level under many conditions. The annex shows the testability analysis for all test points. For example, in the case of FR2b, Test point 1, CBW = 50 MHz, the minimum possible level is -13.45 dBm (= target level 6.0 dBm – power tolerance 14 dB – DL MU 5.45 dB) and the lower limit of measurable level is -8.5 dBm. In order to maximize the coverage of test points, we propose to test the lower power tolerance even in these cases. A bad UE may be judged as PASS by TE noise floor, but it is better than not measuring.

Proposal 3 : Test the lower absolute power tolerance even if the possible level is below the lower limit of measurable level.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1 : Use +/- 8.05 dB for NTC testing and +/- 8.42 dB for ETC testing for FR2 absolute power tolerance MU.

Proposal 2 : Adopt the TT values in Table 2 for FR2 absolute power tolerance.

Proposal 3 : Test the lower absolute power tolerance even if the possible level is below the lower limit of measurable level.

# 4.	References

[1] R5-213807, “On FR2 relative power MU and power control test case testability”, Anritsu, RAN5#91-e

[2] R5-184484, “Proposal for remaining Maximum Test System Uncertainty for NR FR1 TRx tests”, Anritsu, Rohde & Schwarz, RAN5#80

[3] R5-215321, “Testability analysis and test point optimization for FR2 absolute power tolerance and aggregate power tolerance”, Anritsu, RAN5#92-e

# Annex: testability analysis for all test points

According to the test requirement of minimum output power test case (Table 6.3.1.5-2 in TS 38.521-2), the lower limit of measurable power is below.

Table 3 Lower limit of measurable power [dBm] for PC3

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| FR2a | -13 | -10.6 | -7.6 | -4.6 |
| FR2b | -8.5 | -5.5 | -2.5 | +0.5 |

From TS 38.521-2, target levels and power tolerance at each test point are below.

Table 6.3.4.2.5-1: Absolute power tolerance: test point 1 for power class 3

|  | Frequency range | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | FR2a | 8.0 | 8.0 | 8.1 | 8.2 |
| power | FR2b | 6.0 | 6.0 | 6.1 | 6.2 |
| Power tolerance (Note 2) |  | ± (14+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.5, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.1.5. Note 2: Do not test lower limit at CBW ≥ 400 MHz for FR2a and CBW ≥ 100 MHz for FR2b. |  |  |  |  |  |

Table 6.3.4.2.5-2: Absolute power tolerance: test point 2 for power class 3

|  | Frequency range | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | FR2a | 11.0 | 11.0 | 11.1 | 11.2 |
| power | FR2b | 9.0 | 9.0 | 9.1 | 9.2 |
| Power tolerance (Note 2) |  | ± (12+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.5, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.1.5. Note 2: Do not test lower limit at CBW ≥ 200 MHz for FR2b |  |  |  |  |  |

Table 6.3.4.2.5-3: Absolute power tolerance: test point 3 for power class 3

|  | Frequency range | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | FR2a | 21.0 | 21.0 | 21.1 | 21.2 |
| power | FR2b | 19.0 | 19.0 | 19.1 | 19.2 |
| Power tolerance |  | ± (12+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.5, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.1.5. |  |  |  |  |  |

The minimum possible level in absolute power tolerance test case is calculated by target level  – power tolerance – DL MU (5.45 dB in Table 1) are shown below. By comparing with Table 3, test points with a minimum possible level less than the lower limit of measurable level are colored red.

Table 4 Minimum possible level [dBm]: test point 1 for PC3

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| FR2a | -11.45 | -11.45 | -11.35 | -11.25 |
| FR2b | -13.45 | -13.45 | -13.35 | -13.25 |

Table 5 Minimum possible level [dBm]: test point 2 for PC3

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| FR2a | -6.45 | -6.45 | -6.35 | -6.25 |
| FR2b | -8.45 | -8.45 | -8.35 | -8.25 |

Table 6 Minimum possible level [dBm]: test point 3 for PC3

|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| FR2a | +3.55 | +3.55 | +3.65 | +3.75 |
| FR2b | +1.55 | +1.55 | +1.65 | +1.75 |