3GPP TSG-RAN5 Meeting #94-e	R5-220966

# Electronic Meeting, 21st Feb. – 04th Mar. 2022

Title:	Discussion on test cases about EN-DC with 3UL (2CC LTE, 1CC NR)

Source:	Huawei, Hisilicon

Agenda item:	5.3.37.6

Document for:	Discussion

Introduction

In RAN#94e the Rel-16 RAN5 work item for UE Conformance Test Aspects for Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) (Acronym: DC_Pcmax_3UL_CC-UEConTest) was approved [1]. The purpose of this document is to provide the discussion on workplan design about EN-DC with 3UL (2CC LTE, 1CC NR).

Discussion

RAN4 specification

Based on the 38.101-3, configured output power for inter-band EN-DC with 2 E-UTRA uplink and 1 NR uplink is defined as follows:

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for the E-UTRA CG as specified in TS 36.101 [4] clause 6.2.5A modified by PLTE as follows:

PCMAX_L_ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PLTE, PPowerClass,EN-DC }

PCMAX H _ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c , PPowerClass,E-UTRA, PLTE, PPowerClass,EN-DC}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

……

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PPowerClass refers to the maximum output power of the E-UTRA intra-band CA power class given in Table 6.2.2A-1 of TS 36.101 [4],

-	If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG, PPowerClass refers to the maximum output power of the NR intra-band CA power class given in sub clause 6.2A.1 of [2],

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in clause 6.2.3A and clause 6.2.4A of TS 36.101 [4] respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX_ E-UTRA,c is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers within the E-UTRA CG.

Observation 1: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, the configured output power for E-UTRA CG depends on the MPR, A-MPR and power class requirements what is specified in LTE CA.

Expected output of RAN5

MOP

According to Observation 1, the maximum output power for intra-band contiguous LTE CA refers to 36.101-1 clause 6.2.2A.

Table 6.2.2A-1: CA UE Power Class for intraband contiguous CA

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_3C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_5B |  |  |  |  | 23 | +2/-22 |  |  |
| CA_7C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_8B |  |  |  |  | 23 | +2/-22 |  |  |
| CA_38C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_39C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_40C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_40D |  |  |  |  | 23 | +2/-2 |  |  |
| CA_41C |  |  | 26 | +2/-22 | 23 | +2/-22 |  |  |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| CA_42C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_48C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_66B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_66C |  |  |  |  | 23 | +2/-2 |  |  |
| NOTE 1: Void NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For intra-band contiguous carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, MPR value in PCMAX_L_ E-UTRA,c is specified in 36.101 clause 6.2.3A as following:

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1 due to higher order modulation and contiguously aggregated transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3A-1 for UE power class 3 CA bandwidth classes B and C, in Table 6.2.3A-1a for UE power class 2 CA bandwidth class C, and Table 6.2.3A-2 for UE power class 3 CA bandwidth class D. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3A-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | CA bandwidth Class B and C / Smallest Component Carrier Transmission Bandwidth Configuration |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 25 RB | 50 RB | 75 RB | 100 RB |  |
| QPSK | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 25 | > 50 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 25 | > 50 | > 75 | > 100 | ≤ 3 |
| 64 QAM | ≤ 8 and allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 8 or allocation extends across two CC's | > 12 or allocation extends across two CC's | > 16 or allocation extends across two CC's | > 18 or allocation extends across two CC's | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  | ≤ 5 |

For MOP testing, the selection of UL configuration shall ensure the 0 dB MPR. For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, the E-UTRA UL configuration shall ensure 0 dB MPR for E-UTRA intra-band CA as per the MPR requirements in Table 6.2.3A-1 in TS 36.101, which is not the same as E-UTRA single carrier in 2CC test case. For this reason, test points for MOP need to be re-designed to ensure 0 dB MPR for E-UTRA CA. In addition, test requirements also need to be re-calculated. Therefore, it is proposed to add a new test case for MOP testing for inter-band EN-DC with intra-band UL CA on the E-UTRA CG.

Proposal 1: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, new test case is required for MOP testing.

In the lastest TS 38.521-3 6.2B.1.3 MOP for inter-band EN-DC, both test configuration and test requirements are targeted to inter-band EN-DC with 1 E-UTRA and 1 NR uplink, but the test case title and test applicability do not specify such a limitation to applicable EN-DC combinations. In order to differentiate the applicability between the existing 2CC MOP case and the new 3CC MOP case to be added, it is proposed to update the existing MOP test case 6.2B.1.3 for limiting test applicability to inter-band EN-DC with 1 E-UTRA and 1 NR uplink.

- 6.2B.1.3	UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)

- 6.2B.1.3.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

- 6.2B.1.3.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC with 1 E-UTRA CC and 1 NR CC operating on FR1.

Proposal 1a: The existing MOP test case 6.2B.1.3 shall be updated as only applicable to inter-band EN-DC with 1 E-UTRA CC and 1 NR CC.

Configured Output Power

Similar to MOP, configured output power is tested only for the UL configuration where no MPR/A-MPR applies. Thus, test points for configured output power also need to be re-designed to ensure 0 dB MPR for E-UTRA intra-band CA. It is proposed to add a new test case for configured output power testing for inter-band EN-DC with intra-band UL CA on the E-UTRA CG.

Proposal 2: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, new test case is required for configured output power testing.

Similar to 6.2B.1.3 MOP for inter-band EN-DC, the applicability for 6.2B.4.1.3 configured output power for inter-band EN-DC does not limit the applicable EN-DC configurations to inter-band EN-DC with 1 E-UTRA and 1 NR uplink, either. In order to differentiate the applicability between the existing 2CC configured output power case and new 3CC configured output power case to be added, it is proposed to update the existing configured output power test case 6.2B.4.1.3 for limiting test applicability to inter-band EN-DC with 1 E-UTRA and 1 NR uplink.

- 6.2B.4.1.3	Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC and 1 NR CC)

- 6.2B.4.1.3.1	Test purpose

To verify the UE does not exceed the power bounds defined by PCMAX and PCMAX_H.

- 6.2B.4.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC with 1 E-UTRA CC and 1 NR CC within FR1.

- 6.2B.4.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.4.1.0.1.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

Proposal 2a: The existing configured output power test case 6.2B.4.1.3 shall be updated as only applicable to inter-band EN-DC with 1 E-UTRA and 1 NR uplink.

Spurious emission

According to TS 38.101-3, for inter-band EN-DC, spurious emissions requirements are verified at the specific frequencies where second and third order intermodulation products occur.

- 6.5B.3.3	Inter-band EN-DC within FR1

##### 6.5B.3.3.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both both uplink carriers active. Limits on configured maximum output power for the uplink according to clause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.1-1: (Void)

- 6.5B.3.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN-DC with uplink assigned to one LTE band and one NR band the requirements in Table 6.5B.3.3.2-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, the associated IMD2 and IMD3 products are still generated by one E-UTRA band and one NR band intermodulation, which is basically the same as 2UL spurious emissions case. For this reason, the spurious emissions testing for EN-DC with 3UL can be covered by its 2CC fallback.

Proposal 3: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, the spurious emissions testing can be covered by 2CC fallback.

Other Tx characteristics

Except for MOP, configured output power and spurious emissions, all other Tx characteristics testing for inter-band EN-DC within FR1 is applicable to LTE anchor agnostic approach due to no exception requirements. For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, no exception requirements is still applied to NR or intra-band E-UTRA carriers, so LTE anchor agnostic is still applicable with any one E-UTRA carrier from E-UTRA CG.

Take MPR 6.2B.2.3 as an example, the highlighted text “inter-band EN-DC within FR1 with 1 NR UL CC” in test applicability has covered inter-band EN-DC with 2 E-UTRA and 1 NR UL. Therefore, the existing test cases have already covered inter-band EN-DC with intra-band UL CA on the E-UTRA CG and no need to make any revision.

- 6.2B.2.3	UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC)

6.2B.2.3.1	Test purpose

Same test purpose as in clause 6.2.2.1 in TS 38.521-1 [8] for the NR carrier.

6.2B.2.3.2	Test applicability

The requirements of this test apply to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC within FR1 with 1 NR UL CC.

NOTE:	Test execution is not necessary if clause 6.5B.2.3.1 SEM or clause 6.5B.2.3.3 ACLR is executed since MPR requirement is verified in these two test cases.

6.2B.2.3.3	Minimum conformance requirements

For inter-band EN-DC between E-UTRA and FR1 NR, UE maximum output power reduction specified in TS 36.101 [5] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.3.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

Proposal 4: For Tx characteristics except for MOP, configured output power and spurious emissions, the existing test cases have already covered inter-band EN-DC with intra-band UL CA on the E-UTRA CG, so no need to make any revision.

Proposal

For WI DC_Pcmax_3UL_CC-UEConTest, it is proposed RAN5 consider below proposals.

Proposal 1: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, new test case is required for MOP testing.

Proposal 1a: The existing MOP test case 6.2B.1.3 shall be updated as only applicable to inter-band EN-DC with 1 E-UTRA CC and 1 NR CC.

Proposal 2: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, new test case is required for configured output power testing.

Proposal 2a: The existing configured output power test case 6.2B.4.1.3 shall be updated as only applicable to inter-band EN-DC with 1 E-UTRA and 1 NR uplink.

Proposal 3: For inter-band EN-DC with intra-band UL CA on the E-UTRA CG, the spurious emissions testing can be covered by 2CC fallback.

Proposal 4: For Tx characteristics except for MOP, configured output power and spurious emissions, the existing test cases have already covered inter-band EN-DC with intra-band UL CA on the E-UTRA CG, so no need to make any revision.

References

[1]	RP-213228 New WID on UE Conformance Test Aspects for Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1)