3GPP TSG-RAN5 Meeting #94-e	R5-221088

# Electronic Meeting, 21st Feb – 4th Mar 2021

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2823 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR URLLC MAC Test Case for DL Grant Prioritisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test case for NR URLLC DL Grant prioritisation needs to be added as per work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case added ##### 7.1.1.2.5 Correct HARQ process handling / DL grant prioritization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE may fail to interoperate with network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.5 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... 38.523-2CR ... 0209 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.1.1.2.4.3.3	Specific message contents

Table 7.1.1.2.4.3.3-1: SystemInformationBlockType1 (steps 2 and 4 of table 7.1.1.2.4.3.2-1)

| Derivation path: 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | 20 |  | FDD |
| l139 | 2 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.2.5	Correct HARQ process handling / DL grant prioritization

7.1.1.2.5.1	Test Purpose (TP)

(1)

with { E in RRC_CONNECTED state and is configured with two PUCCH-config each corresponds to a PHY priority}

ensure that {

when { UE receives DL MAC PDU’s with DL grant indicating different priorities }

then { UE transmit the HARQ feedback using correct PUCCH resource as per priority}

}

Editors note: Can the test coverage need to be provided for simultaneous 2 HARQ transmission or can be tested with 2 transmission in different time slots? The first requires additional mechanism to make sure HARQ feedback is with right priority by enforcing different hARQ feedbacks for 2 PDSCH simultaneous transmissions. The test sequence currently is for second option.

7.1.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.213 clause 9, 9.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.213, clause 9]

A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from phy-PriorityIndex, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in Clause 9.2.4. For a PUSCH transmission with semi-persistent CSI report, a UE determines a priority index from a priority indicator field, if provided, in a DCI format that activates the semi-persistent CSI report. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

[TS 38.213, clause 9.1]

If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE can be indicated by pdsch-HARQ-ACK-CodebookList to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks

-	a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1

[TS 38.213, clause 9.2.4]

A UE can be configured by SchedulingRequestResourceConfig a set of configurations for SR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. A UE can be configured by schedulingRequestID-BFR-SCell a configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. The UE can be provided, by phy-PriorityIndex in SchedulingRequestResourceConfig, a priority index 0 or a priority index 1 for the SR. If the UE is not provided a priority index for SR, the priority index is 0.

7.1.1.2.5.3	Test description

7.1.1.2.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink and parameters as in Table 7.1.1.2.5.3.1-1.

Table 7.1.1.2.5.3.1-1: MAC Parameters

| Parameter | Value | Comment |
| --- | --- | --- |
| pdsch-HARQ-ACK-Codebook | Not Present | It is assumed this will force the UE to use pdsch-HARQ-ACK-CodebookList-r16 |
| pdsch-HARQ-ACK-CodebookList-r16 | dynamic, semiStatic | 2 entries |

7.1.1.2.5.3.2	Test procedure sequence

Table 7.1.1.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE with priority indiacator =0 | <-- | (PDCCH (C-RNTI) priority Ind =0) (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU with poll bit not set. | <-- | MAC PDU | - | - |
| 3 | Check: Does the UE transmit an HARQ ACK on PUCCH associated with priority indicator 0? | --> | HARQ ACK | 1 | P |
| 4 | SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE with priority indiacator =1 | <-- | (PDCCH (C-RNTI) priority Ind =1) (PDCCH (C-RNTI)) | - | - |
| 5 | SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU with poll bit not set. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit an HARQ ACK on PUCCH associated with priority indicator 1? | --> | HARQ ACK | 1 | P |

7.1.1.2.5.3.3	Specific message contents

None