3GPP TSG-RAN5 Meeting #94-e	R5-221099

# Electronic Meeting, 21st Feb – 4th Mar 2021

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2829 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR V2X Sidelink radio link failure / Transmission side test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test case for NR V2X PC5 Sidelink radio link failure / Transmission side needs to be added as per work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case added 12.1.6.3 PC5-only operation / Sidelink failure / Sidelink radio link failure / Transmission side |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE may fail to interoperate with network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.6.3 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... 38.523-2 CR ... 0212 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 12.1.6

#### 12.1.6.3	PC5-only operation / Sidelink failure / Sidelink radio link failure / Transmission side

12.1.6.3.1	Test Purpose (TP)

(1)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink and has sent an RRCReconfigurationSidelink message to peer UE }

ensure that {

when { UE doesn't receive RRCReconfigurationCompleteSidelink or RRCReconfigurationFailure before T400 expires}

then { UE releases PC5-RRC connection and indicates the release to upper layer}

}

(2)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink and has established a AM SL DRB}

ensure that {

when { Retransmission number of the AM SL DRB reaches the maximum number of retransmissions}

then { UE releases PC5-RRC connection and indicates the release to upper layer.}

}

(3)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { MAC detects that maximum number of consecutive HARQ DTX has been reached }

then { UE releases PC5-RRC connection and indicates the release to upper layer }

}

12.1.6.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.9.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.9.3]

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to sub-clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to sub-clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

12.1.6.3.3	Test description

12.1.6.3.3.1	Pre-test conditions

System Simulator:

-	SS-UE

-	Operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	GNSS simulator

UE:

- UE is authorised to perform NR sidelink communication.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On).

12.1.6.3.3.2	Test procedure sequence

[Editor’s Note: For the below test procedure sequence it is assumed that the initiating UE side (= UE under test) as well as the peer UE (= SS-UE) are both out-of-coverage. Also how step 5 can be achieved is TBD]

Table 12.1.6.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is configured by upper layer to release SL DRB to NR-SS-UE1. Note: This step is triggered by MMI or AT command. | -- |  |  |  |
| 2 | UE send an RRCReconfigurationSidelink message to NR-SS-UE1 to indicate SL DRB release? | --> | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 3 | SS responds nothing and waits for the expiration of t_400 (1 second). | - | - | - | - |
| 4 | Check UE has released PC5 RRC connection as per generic procedure TBD. | - | - | 1 | P |
| 5 | The UE is brought to state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On), Test Mode = On. | - | - | - | - |
| 7 | UE is configured by upper layers to send two RLC PDU’s on AM DRB to NR-SS-UE1. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| 8 | The UE transmits one AMD PDU#1 to NR-SS-UE1 on AM DRB | --> | AMD PDU#1 (SN=0) | - | - |
| - | EXCEPTION: Steps 9-10 are repeated maxRetxThreshold times | - | - | - | - |
| 9 | The NR-SS-UE1 transmits an RLC STATUS PDU. ACK_SN =1 and NACK_SN =0. | <-- | STATUS PDU | - | - |
| 10 | The UE transmits one AMD PDU#1 to NR-SS-UE1 | --> | AMD PDU#1 (SN=0) | - | - |
| 11 | The NR-SS-UE1 transmits an RLC STATUS PDU. ACK_SN =1 and NACK_SN =0. | <-- | STATUS PDU | - | - |
| 12 | Check UE has released PC5 RRC connection as per generic procedure TBD. | - | - | 2 | P |
| 13 | The UE is brought to state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On), Test Mode = On. | - | - | - | - |
| - | UE is configured by upper layers to send a MAC PDU to NR-SS-UE1. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| - | The NR-SS-UE1 MAC is configured to not send HARQ feedback |  |  |  |  |
| - | Exception: step 14 is repeated sl-MaxNumConsecutiveDTX-r16 times |  |  |  |  |
| 14 | The UE transmits one MAC PDU to NR-SS-UE1 | --> | MAC PDU | - | - |
| 15 | Check UE has released PC5 RRC connection as per generic procedure TBD. | - | - | 3 | P |

12.1.6.3.3.3	Specific message contents

### TBD