3GPP TSG-RAN5 Meeting #94-e	R5-221352

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0714 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 Enhanced Beam Correspondence test updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Beam Correspondence test case is still pending in RAN5 test specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of Beam Correspondence tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompete test requirements for Beam Correspondence |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections Skipped>

6.6.2.4.2	Test procedure

r16 is supported

FFS

Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceSSB-RS-based-r16 is supported

- For Min peak EIRP:

- Same test description as in clause 6.2.1.1.4 (skipping test step 7) with the following exceptions: Measurement shall be carried out using side conditions defined in Table 6.6.1.3.3.1.1-1

- For Spherical coverage:

- Same test description as in clause 6.2.1.2.4 with the following exceptions: Measurement shall be carried out using side conditions defined in Table 6.6.1.3.3.1.1-1.

Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceCSI-RS-based-r16 is supported

FFS

Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, uplink beam management and beamCorrespondenceSSB-based-r16 is supported

FFS

Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, uplink beam management and beamCorrespondenceCSI-RS-based-r16 is supported

FFS

Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, uplink beam management, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 is supported

FFS

Test procedure if beamCorrespondenceWithoutUL-BeamSweeping, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 is supported

FFS

6.6.2.4.3	Message contents

Editor’s note : Requires additional updates for Rel.16 related SSB and CSI-RS Beam Correspondence IEs.

Same as the message contents in 6.6.1.4.3 with additional IEs for Rel.16 enhanced Beam Correspondence as listed in the tables below.

FFS.

6.6.2.5	Test requirements

The defined %-tile EIRP in measurement distribution derived in step 2.6 shall exceed the values specified in Table 6.2.1.2.5-3 in clause 6.2.1.2.5. The defined %-tile ΔEIRPBC in measurement distribution derived in step 2.7 shall not exceed the values specified in Table 6.6.1.5-1.

Table 6.6.1.5-1: UE beam correspondence tolerance for power class 3

| Operating band | Max ∆EIRPBC at 85th %-tile ∆EIRPBC CDF (dB) |
| --- | --- |
| n257 | 3.0 +TT |
| n258 | 3.0 +TT |
| n260 | 3.2 +TT |
| n261 | 3.0 +TT |
| NOTE: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1 |  |

<Unchanged Sections Skipped>

# A.3	DL reference measurement channels

## A.3.1	General

Unless otherwise stated, Tables A.3.3.2-1 and A.3.3.2-2 are applicable for measurements of the Receiver Characteristics (clause 7).

Unless otherwise stated, Tables A.3.3.2-1 and A.3.3.2-2 also apply for the modulated interferer used in Clauses 7.5 and 7.6 with test specific bandwidths.

CSI-RS configuration parameter defined in Table A.3.1-2 and Table A.3.1-3 are used for verifying the beam correspondence requirement, 2 slots of CSI-RS shall be provided at each test grid point. The DL channel shall be configured for zero power on all tones except those used by CSI-RS in slots containing CSI-RS for beam refinement, and the DL and UL channel sizes shall be the same during verification.

Table A.3.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| CORESET frequency domain allocation |  |  | Full BW |
| CORESET time domain allocation |  |  | 2 OFDM symbols at the begin of each slot |
| PDSCH mapping type |  |  | Type A |
| PDSCH start symbol index (S) |  |  | 2 |
| Number of consecutive PDSCH symbols (L) |  |  | 12 |
| PDSCH PRB bundling |  | PRBs | 2 |
| Dynamic PRB bundling |  |  | false |
| MCS table for TBS determination |  |  | 64QAM |
| Overhead value for TBS determination |  |  | 0 |
| First DMRS position for Type A PDSCH mapping |  |  | 2 |
| DMRS type |  |  | Type 1 |
| Number of additional DMRS |  |  | 2 |
| FDM between DMRS and PDSCH |  |  | Disable |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2 |
|  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 12 for CSI-RS resource 2 |
|  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity | Slots | 60 kHz SCS: 80 for CSI-RS resources 1 and 2 120 kHz SCS: 160 for CSI-RS resources 1 and 2 |
|  | CSI-RS offset | Slots | 60 kHz SCS: 40 for CSI-RS resources 1 and 2 120kHz SCS: 80 for CSI-RS resources 1 and 2 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  | TCI state #0 |
| PTRS configuration |  |  | PTRS is not configured |

Table A.3.1-2: CSI-RS parameters for beam correspondence based on SSB and CSI-RS

| Resource Type | aperiodic |
| --- | --- |
| Resource Set Config |  |
| repetition | on |
| aperiodicTriggeringOffset | Depending on UE capability |
| Resource Config |  |
| nzp-CSI-RS-ResourceId | 30 for resource #0 |
|  | 31 for resource #1 |
|  | 32 for resource #2 |
|  | 33 for resource #3 |
|  | 34 for resource #4 |
|  | 35 for resource #5 |
|  | 36 for resource #6 |
|  | 37 for resource #7 |
| powerControlOffset | 0 |
| powerControlOffsetSS | db0 |
| nrofPorts | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 |
|  | 7 for resource #1 |
|  | 8 for resource #2 |
|  | 9 for resource #3 |
|  | 10 for resource #4 |
|  | 11 for resource #5 |
|  | 12 for resource #6 |
|  | 13 for resource #7 |
| cdm-Type | noCDM |
| density | 3 |
| nrofRBs | 48 for channel bandwidth≥100MHz 32 for channel bandwidth=50MHz |
| qcl-info | Type D to SSB |

The CSI-RS configuration parameter defined in Table A.3.1-3 is used for verifying the beam correspondence requirement. CSI-RS shall be provided once every 10msec.

Table A.3.1-3: CSI-RS parameters for CSI-RS based beam correspondence

| Resource Type | aperiodic |
| --- | --- |
| Resource Set Config |  |
| repetition | on |
| aperiodicTriggeringOffset | Depending on UE capability |
| Resource Config |  |
| nzp-CSI-RS-ResourceId | 30 for resource #0 |
|  | 31 for resource #1 |
|  | 32 for resource #2 |
|  | 33 for resource #3 |
|  | … |
|  | … |
|  | … |
|  | 29+N for resource #(N-1), where N is maxNumberRxBeam in UE capability IE of MIMO-ParametersPerBand |
| powerControlOffset | 0 |
| powerControlOffsetSS | db0 |
| nrofPorts | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 |
|  | 7 for resource #1 |
|  | 8 for resource #2 |
|  | 9 for resource #3 |
|  | … |
|  | … |
|  | … |
|  | 5+N for resource #(N-1), where N=maxNumberRxBeam-1 in UE capability IE of MIMO-ParametersPerBand |
| cdm-Type | noCDM |
| density | 3 |
| nrofRBs | 48 for channel bandwidth≥100MHz 32 for channel bandwidth=50MHz |
| qcl-info | Type D to SSB |

<End of changes>