3GPP TSG-RAN5 Meeting #94-e	R5-221492

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2169 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of inter-band cell environment for Protocol testing with NR CA 3CC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no inter-band cell environment for Protocol testing with NR CA 3CC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new simulated NR cell in multi cell network scenario and the related definition of test frequency and parameters of the new simulated NR cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is still no inter-band cell environment for Protocol testing with NR CA 3CC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1, 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-220049r1. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.4.1.1.3	Standalone NR single mode multi cell network scenarios

For standalone NR FDD or TDD intra-frequency multi cell environment, NR Cell 1, NR Cell 2 and NR Cell 4 are used.

For standalone NR FDD or TDD inter-frequency multi cell environment, NR Cell 1, NR Cell 3 and NR Cell 6 are used.

For standalone NR FDD or TDD inter-band cell environment, NR Cell 1, NR Cell 10 and NR Cell 500 are used.

For standalone NR FDD or TDD multi tracking area intra-frequency multi cell environment, NR Cell 1 and NR Cell 11 are used.

For standalone NR FDD or TDD multi tracking area inter-frequency multi cell environment, NR Cell 1 and NR Cell 23 are used.

For standalone NR FDD or TDD multi PLMN inter-frequency multi cell environment, NR Cell 1, NR Cell 12, NR Cell 13 and NR Cell 14 are used.

< Unchanged sections omitted >

Table 4.4.2-1: Definition of test frequencies and simulated NR cells

| Test frequency | RAT | Operating band | Range | Simulated NR cells |
| --- | --- | --- | --- | --- |
| NRf1 | NR | Operating band under test | Mid (Note 1, Note 3 | NR Cell 1, NR Cell 2, NR Cell 4, NR Cell 11 (Note 2), NR Cell 489 |
| NRf2 | NR | Operating band under test | High (Note 1, Note 3) | NR Cell 3, NR Cell 12, NR Cell 23 |
| NRf3 | NR | Operating band under test | Low (Note 1, Note 3) | NR Cell 6, NR Cell 13 |
| NRf4 | NR | Operating band under test | (Note 1) | NR Cell 14 |
| NRf5 | NR | Operating band for inter-band cells | Mid (Note 1) | NR Cell 10, NR Cell 30, NR Cell 31 |
| NRf6 | NR | Operating band for inter-band cells | High (Note 1) | NR Cell 28, NR Cell 29 |
| NRf7 | NR | Operating band for inter-band cells | Low (Note 1) |  |
| NRf8 | NR | Operating band for SDL cell | Mid (note 1) | NR Cell 32 |
| NRf9 | NR | Operating band for SUL cell | Mid (note 1) | NR Cell 33 |
| NRf10 | NR | Operating band for inter-band cells | Mid (Note bb) | NR Cell 500 |
| Note 1: For signalling test, see clause 6.2.3. Note 2: For signalling test, simultaneous co-existence of NR Cell 2 with NR Cell 11 is not allowed due to the same timing is used for Cell 2 and Cell 11 in 38.523-3 [23]. Note 3: For RRM test with NR intra-band non-contiguous CA, the test frequencies for the set of non-contiguous component carriers are specified in clauses 4.3.1.1.4 for FR1 and in clause 4.3.1.2.4 for FR2 without any regard to range. Thus “Low”, “Mid” and “High” information in this table does not apply. Unless otherwise stated, test point with maximum Wgap is chosen. Note 4: For signalling test with NR inter-band CA three bands only, the test band is different from the Operating band for NRf1 and the Operating band for NRf5. |  |  |  |  |

Table 4.4.2-2: Default NR parameters for simulated NR cells

| cell ID |  | NR Cell Identifier |  |  | Physical layer cell identity |  | PRACH-rootSequenceIndex FDD |  | PRACH-rootSequenceIndex TDD | SSB-Index2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | gNB Identifier |  | Cell Identity |  |  | Note 1 |  | Note 1 |  |  |
| NR Cell 1 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0000'B | 0 |  | 0 |  | 0 |  | 1 |
| NR Cell 2 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0010'B | 2 |  | 32 |  | 32 |  | 0 |
| NR Cell 3 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 0011'B | 3 |  | 0 |  | 0 |  | 1 |
| NR Cell 4 |  | '00 0000 0000 0000 0000 0000 0011'B |  | '00 0000 0100'B | 4 |  | 64 |  | 64 |  | 1 |
| NR Cell 6 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 0110'B | 6 |  | 0 |  | 0 |  | 1 |
| NR Cell 10 |  | '00 0000 0000 0000 0000 0000 0101'B |  | '00 0000 1010'B | 10 |  | 0 |  | 0 |  | 1 |
| NR Cell 11 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0000 1011'B | 11 |  | 96 |  | 96 |  | 0 |
| NR Cell 12 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 1100'B | 12 |  | 32 |  | 32 |  | 0 |
| NR Cell 13 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 1101'B | 13 |  | 32 |  | 32 |  | 0 |
| NR Cell 14 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0000 1110'B | 14 |  | 0 |  | 0 |  | 1 |
| NR Cell 23 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 0111'B | 23 |  | 64 |  | 64 |  | 1 |
| NR Cell 28 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0001 1100'B | 28 |  | 0 |  | 0 |  | 1 |
| NR Cell 29 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0001 1101'B | 29 |  | 32 |  | 32 |  | 0 |
| NR Cell 30 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0001 1110'B | 30 |  | 32 |  | 32 |  | 0 |
| NR Cell 31 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 1111'B | 31 |  | 64 |  | 64 |  | 1 |
| NR Cell 32 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0000'B | 32 |  | - |  | - |  | 1 |
| NR Cell 33 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0001'B | 33 |  | - |  | - |  | - |
| NR Cell 489 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '01 1110 1001'B | 489 |  | 128 |  | 128 |  | 0 |
| NR Cell 500 |  | '00 0000 0000 0000 0000 0000 1000'B |  | '00 0000 0111'B | 500 |  | 0 |  | 0 |  | 1 |
| Note 1: To avoid collision of the preambles between intra-frequency cells, with the default zeroCorrelationZoneConfig value set to 15, the PRACH-rootSequenceIndex values have been separated by 32 root sequences per intra-frequency cell. Note 2: This SSB-Index does not apply for RRM test cases in TS 38.533 [18]. RRM test cases shall use the SSB index defined in A.3 of TS 38.533 [18]. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.4.2-3: Default NAS parameters for simulated NR cells

| cell ID | Tracking Area |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC | MNC |  |  | AMF region ID | AMF Set ID | AMF Pointer |  |
| NR Cell 1 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 [26] clause 2.10 |
| NR Cell 2 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 3 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 4 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 6 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 10 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 11 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 23 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 12, NR Cell 28 | TAI-3 | 002 | 11 | 1 | TAI-3 | 253 | 1 | 1 |  |
| NR Cell 13, NR Cell 29 | TAI-4 | 003 | 21 | 1 | TAI-4 | 252 | 1 | 1 |  |
| NR Cell 14, NR Cell 30 | TAI-5 | 004 | 31 | 1 | TAI-5 | 251 | 1 | 1 |  |
| NR Cell 31 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 489 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 500 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (REGISTRATION ACCEPT ) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates 5G-GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |  |

Table 4.4.2-4: Default parameters for simulated SNPN cells

| cell ID | Network Identifier (NID) |  |
| --- | --- | --- |
|  | Assignment mode (Note 1) | NID value |
| NR Cell 1 | 1 | 1 |
| NR Cell 2 | 1 | 2 |
| NR Cell 3 | 1 | 3 |
| NR Cell 4 | 1 | 4 |
| NR Cell 6 | 1 | 6 |
| NR Cell 10 | 1 | 10 |
| NR Cell 11 | 1 | 11 |
| NR Cell 12 | 1 | 12 |
| NR Cell 13 | 1 | 13 |
| NR Cell 14 | 1 | 14 |
| NR Cell 23 | 1 | 23 |
| NR Cell 28 | 1 | 28 |
| NR Cell 29 | 1 | 29 |
| NR Cell 30 | 1 | 30 |
| NR Cell 31 | 1 | 32 |
| NR Cell 32 | 1 | 32 |
| NR Cell 33 | 1 | 33 |
| NR Cell 489 | 1 | 489 |
| NR Cell 500 | 1 | 500 |
| Note 1: As per TS 23.003 Cl 12.7.1, Network Identifier comprises of Assignment mode (one hexadecimal digit) and NID value (10 hexadecimal digits). Self-assignment NIDs are chosen by setting the assignment mode to value 1. |  |  |

<<< END OF CHANGES >>>