3GPP TSG-RAN5 Meeting #94-e	R5-221514

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2227 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to V2X message DIRECT LINK SECURITY MODE COMMAND |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, According to 24.587 7.3.13.1.1 LSBs fo KNRP-sess ID is not needed when null intefity protection algorithm is selected. 2, According to 38.508-1 4.9.22 and 4.9.23 Authentication procedure is skipped, KNRP is not derived so MSBs of KNRP ID is not needed. 3, Core spec only request that Sequence number need increase but does not request that Sequence number must increase by 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Set LSBs fo KNRP-sess ID to Not present in Rx condition. 2, Set MSBs of KNRP ID to Not present in Rx condition. 3, Change Sequence number to “Not Checked” in Tx condition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail in the preamble procedure of V2X unicast TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-220588r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	DIRECT LINK SECURITY MODE COMMAND

Table 4.7.4-18: DIRECT LINK SECURITY MODE COMMAND

| Derivation Path: TS 24.587 [54] Table 7.3.13.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK SECURITY MODE COMMAND message identity |  | '0000 1110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Selected security algorithms |  | '0000 0000'B |  |  |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| UE PC5 unicast signalling security policy |  |  |  |  |
| UE PC5 unicast signalling security policy IEI |  | '59'H |  |  |
| Signalling integrity protection policy |  | '000'B | Signalling integrity protection not needed |  |
| Signalling ciphering policy |  | '000'B | Signalling ciphering not needed. |  |
| Nonce_2 |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| LSBs of KNRP-sess ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| Key establishment information container |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| MSBs of KNRP ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

<End of modified section 1>